The Las Cruces Regional ITS Architecture Update

State RFP 06-24

Final Documentation

November 20, 2007

Submitted By:

Consensus Systems Technologies Corporation

17 Miller Avenue, PO Box 517

Shenorock, NY 10587-0517

914-248-8466

rsj@consystec.com

www.consystec.com

Table of Contents

Table of Contents	ii

Table of Figures	iv

List of Tables	iv

Revision History	v

1.	Introduction	1

1.1.	Purpose	1

1.2.	Document Overview	2

1.3.	Scope of the Architecture	2

2.	Las Cruces Regional ITS Architecture Development Process	5

2.1.	Process to Create the Architecture	5

2.1.1.	Create Initial Architecture Update	6

2.1.2.	Stakeholder Outreach Meeting	6

2.1.3.	Draft Architecture Update-1	7

2.1.4.	Architecture Review Meeting	7

2.1.5.	Draft Architecture Update-2	7

2.1.6.	Architecture Documentation	8

2.1.7.	Deliver Final Architecture Products	8

2.2.	Requirements of the Final FHWA Rule and FTA Policy on Architecture	8

2.2.1.	Specific Requirements of the Final FHWA Rule or FTA Policy	8

2.2.2.	How the Final Rule and FTA Policy Requirements are Met	8

3.	ITS Architecture Concepts	10

4.	Identification of Stakeholders	14

4.1.	Champion	14

4.2.	Regional Stakeholders	14

4.3.	Operational Concepts	18

5.	Systems Inventory	33

5.1.	Systems by Stakeholder	33

5.2.	Systems by Architecture Entity	47

6.	Needs and Services	48

6.1.	Needs Identification	48

6.2.	Services	51

6.3.	Comparison of Needs and Services	53

7.	Interfaces and Information Exchanges	57

7.1.	Top Level Regional System Interconnect Diagram	57

7.2.	Customized Market Packages	96

7.3.	Regional Architecture Information Flows	97

8.	Functional Requirements	101

9.	Standards	104

9.1.	Discussion of Key Standards for the Region	104

9.2.	Reference to the Detailed Standards information on the Web Site	108

10.	Project Sequencing	110

10.1.	Process For Selecting Projects	110

10.2.	Gap Analysis	118

10.3.	How to Use the Recommended Project Sequencing	121

11.	Agreements	123

11.1.	Types of Agreements	123

11.2.	Existing Agreements	124

11.3.	Potential Agreements	125

12.	Using the Las Cruces Regional ITS Architecture	132

12.1.	Using the Las Cruces Regional ITS Architecture in the Planning Process	132

12.1.1.	Operational Strategies	135

12.1.2.	Strategy Evaluation and Prioritization	136

12.1.3.	Long Range Transportation Plan	138

12.1.4.	Issues/Challenges	139

12.1.5.	Recommendations	139

12.2.	Using the Las Cruces Regional ITS Architecture for Programming	140

12.2.1.	Architecture Use in Programming Federal Funds	140

12.2.2.	Architecture Use in Organization Capital Planning	141

12.2.3.	Architecture Use to Identify and Define Projects	142

12.2.4.	Challenges	142

12.2.5.	Recommendations	143

12.3.	Using the Las Cruces Regional ITS Architecture in Project Definition	143

12.3.1.	Issues/Challenges	147

12.3.2.	Recommendations	148

13.	Maintaining the Architecture	149

13.1.	Roles and Responsibilities for Maintenance	150

13.1.1.	Definitions	150

13.1.2.	Stakeholders	151

13.1.3.	Maintenance Working Group	151

13.1.4.	Responsible Agency	152

13.1.5.	Maintenance Manager	152

13.2.	Timetable for Maintenance	152

13.2.1.	Periodic Updates	153

13.2.2.	Event-Driven Updates	154

13.3.	Architecture Baseline	154

13.4.	Change Management Process	155

13.4.1.	Identify Change	156

13.4.2.	Evaluate and Review the Change Request	157

13.4.3.	Update Baseline	158

13.4.4.	Update of the Turbo Architecture Database	159

13.4.5.	Notify Stakeholders	159

Table of Figures

Figure 1.  Las Cruces MPO Area Boundary	3

Figure 2.  Las Cruces Regional ITS Architecture Development Process	5

Figure 3.  Information flows	11

Figure 4.  Example of a National ITS Architecture Market Package	12

Figure 5.  Example of Las Cruces Regional ITS Architecture Customized Market Package	13

Figure 6.  Las Cruces Regional ITS Architecture System Interconnect Diagram	93

Figure 7.  Example Customized Market Package	97

Figure 8.  Example of Element Detail With Interfaces	98

Figure 9.  Example of Architecture Flows Between Elements	100

Figure 10.  Example of Interface	108

Figure 11.  Example of Standards Mapping Page	109

Figure 12.  Example Market Package: APTS2 - Transit Fixed Route Operations for RoadRUNNER Transit	126

Figure 13.  ITS Architecture and the Transportation Planning Process	134

Figure 14.  Supporting the Transportation Planning Processes	135

Figure 15.  Project Implementation Process	144

Figure 16.  Change Management Process	155

List of Tables

Table 1.  Mapping of Requirements to Architecture Outputs	9

Table 2.  Stakeholders	15

Table 3.  Stakeholder Roles and Responsibilities	19

Table 4.  Inventory Sorted by Stakeholder	34

Table 5.  Summary of Transportation Needs/Priorities	48

Table 6.  Summary of High Priority Transportation Needs	50

Table 7.  Selected Regional Services (Market Packages)	51

Table 8.  Mapping of Needs to Services	53

Table 9.  Applicable ITS Standards	104

Table 10.  List of Sources Used for Identifying Projects	111

Table 11.  Las Cruces ITS Projects	114

Table 12.  Projects Suggested by Gap Analysis	119

Table 13.  Types of Institutional Agreements	123

Table 14.  Existing Institutional Agreements	124

Table 15.  Potential Institutional Agreements	127

Table 16.  Regional Project Development Process Relation to FHWA System Engineering Process	145

Table 17.  System Engineering Requirements supported by Las Cruces Regional ITS Architecture	147

Table 18.  Functions (Equipment Packages) Assigned to Architecture Elements	170

Table 19.  Equipment Package Descriptions	178

Revision History

| Filename | Version | Date | Author | Comment |
| --- | --- | --- | --- | --- |
| LasCrucesITSArchitecture-draftv1.doc | 0.01 | 09/20/07 | P. Chan/ B. Eisenhart | First draft |
| LasCrucesITSArchitecture-v1.doc | 1.00 | 11/20/07 | P. Chan/ B Eisenhart | Final Deliverable |

# Introduction

The Las Cruces Regional Intelligent Transportation Systems (ITS) Architecture Update is a roadmap for transportation systems integration in the Las Cruces Metropolitan Planning Organization area over the next 20 years.  The architecture has been developed through a cooperative effort by the regional transportation agencies, covering all modes and all roads.  The architecture represents a shared vision of how each agency’s systems will work together in the future, sharing information and resources to provide a safer, more efficient, and more effective transportation system for travelers in the region.

The architecture is an important tool that will be used by:

- Operating Agencies to recognize and plan for transportation integration opportunities in the MPO region;

- Planning Agencies to better reflect integration opportunities and operational needs into the transportation planning process; and

- Other organizations and individuals that use the transportation system in the Las Cruces MPO region.

The architecture provides an overarching framework that spans all of these organizations and individual transportation projects.  Using the architecture, each transportation project can be viewed as an element of the overall transportation system, providing visibility into the relationship between individual transportation projects and ways to cost-effectively build an integrated transportation system over time.  The architecture also is coordinated with the New Mexico Statewide ITS Architecture to ensure consistency at those jurisdictional and operational points where regionally-based operations coincide and/or integrate with statewide planning and operations.

## Purpose

The Las Cruces Regional ITS Architecture Update represents a consensus blueprint for ITS investments in the Las Cruces MPO region.  This Las Cruces Regional ITS Architecture Update is an update of the existing Las Cruces Regional ITS Architecture that was developed in 2003.  The development of this update began by reviewing the existing 2003 architecture and updated transportation planning documents that were available.  With the participation of ITS agencies (stakeholders) in the region at two stakeholder workshops, the architecture was interactively updated.  The architecture goes on to define possible integration opportunities between agencies within the region and adjoining regions and identifies how cooperation between the agencies in the deployment of ITS systems can be used to satisfy transportation needs.

This architecture can be used to efficiently structure implementations of ITS technologies.  By creating a long range plan for the implementation of these systems and technologies, agencies can:

- Prepare for future expansion

- Leverage funding

- Identify standard interfaces

In addition to structuring implementations of ITS technologies, the Las Cruces Regional ITS Architecture allows the region to comply with the FHWA Rule/FTA Policy on Architecture and Standards.  The FHWA Final Rule 23 CFR 940, (and corresponding FTA policy) to implement Section 5206(e) of the TEA-21 requires that ITS projects funded through the Highway Trust Fund conform to the National ITS Architecture and applicable standards.  The Rule/Policy requires that the National ITS Architecture be used to develop a local implementation of the National ITS Architecture, which is defined as a regional ITS architecture.  This update of the Las Cruces Regional ITS Architecture will bring the regional architecture into full compliance with this Rule/Policy, which will facilitate the approval of federal funds to support ITS projects in the region.

## Document Overview

This document is organized into thirteen main sections and four Appendices.  Section 1 provides introductory information on the project, this document, and discusses the scope of the architecture.  Section 2 describes the process used to develop the Las Cruces Regional ITS Architecture Update.  Section 3 gives a brief introduction and overview of the National ITS Architecture, and how it relates to this regional ITS architecture.  The stakeholders are identified in Section 4, while their systems are inventoried in Section 5.  The needs addressed by ITS and the services used to address those needs are covered in Section 6.  The interfaces and information exchanges are described in Section 7.  The definition of functional requirements for the elements of the architecture is described in Section 8.  A discussion of ITS standards applicable to the region is provided in Section 9.  A listing of planned ITS projects in the region, along with sequencing of these projects, are described in Section 10.  A discussion of agreements is provided in Section 11.  Finally, Section 12 provides guidance on using the regional ITS architecture and Section 13 presents the architecture maintenance plan.  The Appendices include a list of Acronyms (Appendix A), a summary of comments received and how the comments were addressed (Appendix B), a detailed listing of the functions assigned to each element (Appendix C), and the Maintenance Change Request Form (Appendix D).

## Scope of the Architecture

The geographic scope of the architecture is the entire Las Cruces MPO boundary, which is shown graphically in Figure 1.  The Las Cruces MPO region covers Dora Ana County, the City of Las Cruces, and the Town of Mesilla.

There is one other ITS architecture that borders on the Las Cruces Regional ITS Architecture: the New Mexico Statewide ITS Architecture.  The New Mexico Statewide ITS architecture provides a general expression of systems in the state, while the Las Cruces regional ITS architecture provides specific examples of the systems within the Las Cruces MPO region.  For example, the Statewide ITS Architecture describes interfaces to the element MPO/RPO Traffic Database, while the Las Cruces architecture describes a specific example of this for the region, the Las Cruces MPO Data Warehouse.  The Statewide ITS Architecture was developed just prior to this update of the Las Cruces architecture and there was significant review of the Statewide ITS Architecture to verify that the contents of both architectures are consistent between the two architectures.

Figure 1.  Las Cruces MPO Area Boundary

As mentioned in the introduction, the timeframe considered for the Las Cruces Regional ITS Architecture is a 20-year outlook for ITS activities in the region, with an emphasis on those ITS activities likely to be implemented in the next 5 years.  This means that the architecture addresses existing ITS systems as well as those planned for development over the next 20 years.  The regional ITS architecture represents a snapshot of the currently anticipated ITS and other projects based on information gathered from stakeholders, and research from agency websites or published agency documents.

The architecture covers services across a broad range of ITS, including traffic management, maintenance and construction operations, incident management, emergency services, transit management, traveler information, and archived data management.

# Las Cruces Regional ITS Architecture Development Process

## Process to Create the Architecture

The development of the Las Cruces Regional ITS Architecture Update relied heavily on stakeholder input to ensure that the ITS architecture reflected local and regional needs and plans.  The development process used is illustrated in Figure 2.

Create Initial

Architecture Update

Review Architecture   in one-day meeting

Update Architecture

Definition-1

Architecture Review

Meeting

Update Architecture

Definition-2

Architecture

Documentation

Deliver Final

Architecture Products

Figure 2.  Las Cruces Regional ITS Architecture Development Process

The following process steps were used to develop the ITS architecture:

- Review the 2003 Las Cruces Regional ITS Architecture and create an initial architecture update focusing on ITS elements and a draft set of customized ITS Services to be provided.

- Conduct stakeholder outreach through a one-day workshop on May 9, 2007.

- Create a complete updated architecture definition for review (web-based).

- Allow stakeholder review of the draft regional ITS architecture and conduct additional stakeholder review through individual stakeholder interactions and through a one-day architecture review workshop in July 19, 2007.

- Finalize the ITS architecture based on review comments.

- Develop and deliver a draft Architecture document.

- Following comment resolution deliver the final architecture products.

### Create Initial Architecture Update

An updated set of ITS elements, services and interconnections were created beginning with the original Las Cruces regional ITS architecture.  The original Las Cruces regional ITS architecture was first documented into the current version of Turbo Architecture (3.1), which is based on Version 5.1 of the National ITS Architecture.  The Las Cruces MPO ITS Strategic Deployment Plan, published in 2003, was used to establish an initial list of services that the elements of the architecture would provide.

For each existing or future ITS service operating or expected in the region, the market package diagram (the collection of ITS elements, equipment packages, and functions that work together to perform a specific ITS service – see Section 3 for details on the National ITS Architecture) for that service from the National ITS Architecture was edited so that each National ITS Architecture subsystem or terminator was associated with the local stakeholder element name.  In some cases, multiple instances of the market package were developed where the service had more than one instance in the region.  This would be the case if there were multiple agencies performing the same service within the region.  This set of customized market packages using the draft elements created previously, was created in preparation for stakeholder outreach so that each could be reviewed and further customized based on actual operating procedures (or theories) for each agency.

### Stakeholder Outreach Meeting

A one-day stakeholder outreach workshop was held at the Branigan Cultural Center on May 9, 2007.  A key overall objective of the meeting was to develop a regional ITS architecture update that is a consensus architecture, that is, each of the participants understands and agrees to the ITS elements and specific information exchanges between the ITS elements identified in the architecture that they participated in defining.  This is not to say that the resulting ITS Architecture has credible funding identified that would lead to full deployment.  The ITS architecture only identifies the current set of ITS elements and interfaces that the stakeholders agree to.  Existing funding processes will continue to be used to decide how to allocate limited resources for ITS elements and interfaces for deployment.

The meeting also incorporated an overview or training in the National ITS Architecture and regional ITS architectures, and a discussion of the scope of the architecture, so that stakeholders would understand and more fully participate in the ITS architecture development process.  The first part of the workshop focused on understanding ITS, what an ITS Architecture is, and the development approach for the regional ITS architecture update.  After some basic training on ITS architectures, the focus shifted to developing the Las Cruces Regional ITS Architecture update.  Stakeholders were asked to participate in identifying and defining their ITS needs while reviewing and revising their draft inventory and the inventory of fellow stakeholders.  The second part of the workshop was spent reviewing the customized market package diagrams, adding or deleting diagrams, elements, and interconnections when necessary.

### Draft Architecture Update-1

Following the stakeholder workshop, the customized market packages were revised and a draft architecture was created.  Using the customized market package diagrams (as modified during and post the workshop), the Turbo Architecture database was updated, “built”, and utilized to create a draft ITS architecture.  This involved the following activities:

- Updating the ITS inventory (and stakeholders when necessary)

- Revising the customized market packages

- Development of a draft Operational Concept

- Creating a Turbo Architecture database that represents the sum of all of the customized market packages.

In addition to creating the Turbo Architecture database and the customized set of market package diagrams, a high level Architecture Interconnect Diagram (AID, also known as a Sausage Diagram) of all the elements in the ITS architecture was created.  A draft of this architecture document was created.  There was also a hypertext version of the complete Turbo Architecture database that was created and placed on a generally accessible website (www.consystec.com).  This website describes each element of the ITS architecture and all of their interconnections with other elements.  The website was developed using additional software tools that go beyond the basic Turbo Architecture software.

Stakeholders were notified by email (collected at the kick-off meeting, the stakeholder meetings and through independent research on the internet) that a review period for the Las Cruces Regional ITS Architecture Update had commenced, and feedback was solicited.  Stakeholders were encouraged to review the regional ITS architecture on the website, and were encouraged to provide feedback electronically from the website.  Comments received during the course of this project are summarized and maintained in a database.  These stakeholders’ comments, as well as comments generated during the quality control process, and their corresponding dispositions are summarized in Appendix B.

### Architecture Review Meeting

In addition to the stakeholder website review, a review of architecture material was held on July 19, 2007 at the Branigan Cultural Center.  During both meetings, stakeholders reviewed comments generated during the review period, and reviewed the changes to any of the customized market package diagrams that were a direct result of the comments.  In addition, the meeting covered discussions of market package prioritization and the identification of regional projects.

### Draft Architecture Update-2

Following the stakeholder meeting, the architecture parts were again updated, and additional architecture elements, such as project list, list of agreements, and functional requirements were developed.  These updates resulted from comments received during the meeting, or from outstanding comments from the Las Cruces MPO, liaisons or from e-mails from the project website.

### Architecture Documentation

A draft Architecture document was created.  This document, along with an updated web site, forms a complete description of the Las Cruces Regional ITS Architecture that is compliant with the architecture and standards Rule.

### Deliver Final Architecture Products

Following a comment period on the document and revised website, a final architecture document and final updated website will be developed as the final deliverables on the architecture.

## Requirements of the Final FHWA Rule and FTA Policy on Architecture

### Specific Requirements of the Final FHWA Rule or FTA Policy

The FHWA Final Rule (23CFR 940) and FTA Policy on Intelligent Transportation System Architecture and Standards, which took effect on April 8, 2001, defines a set of requirements that regional ITS architectures should meet.  The following is a list of specific requirements from the FHWA Rule/FTA Policy:

- A description of the region (scope)

- Identification of participating agencies and their systems (inventory)

- Operations concepts

- Agreements required for implementation

- System functional requirements

- Interface requirements

- Identification of ITS Standards

- Sequence of projects required for implementation

- Develop a Process for maintaining your regional ITS Architecture

### How the Final Rule and FTA Policy Requirements are Met

Table 1 shows how the requirements of the rule are met by the outputs developed for the Las Cruces Regional ITS Architecture:

Table 1.  Mapping of Requirements to Architecture Outputs

| Regional ITS Architecture Requirements | Where Requirements Documented |
| --- | --- |
| Description of region | Geographic definition, identification of services and a timeframe are given in Section 1 of this document. |
| Identification of participating agencies and other stakeholders | Listing of stakeholders and their definitions is given in Section 4 of this document. An inventory of the elements operated by the stakeholders is contained in Section 5 of this document. The same information is also available in the hyperlinked web site and in the Turbo Architecture database. |
| An operational concept that identifies the roles and responsibilities of participating agencies and stakeholders | The operational concept is defined in Section 4 of this document. |
| A list of any agreements (existing or new) required for operations | The discussion of existing and needed new agreements is given in Section 11 of this document. |
| System functional requirements | The functional requirements of the ITS systems are described in an overview in Section 8 of this document, are presented in detail as Appendix C of this document, They are also provided in detail in the hyperlinked web site and in the Turbo Architecture database. |
| Interface requirements and information exchanges with planned and existing systems and subsystems | The Interfaces and information flows are described in a general way in Section 7 of this document, and are described in detail in the hyperlinked web site and in the Turbo Architecture database. |
| Identification of ITS standards supporting regional and national interoperability | The general identification of standards for ITS in the Las Cruces region is contained in Section 9 of this document. The detailed descriptions of standards for each interface is described in the hyperlinked web site and in the Turbo Architecture database. |
| The sequence of projects required for implementation | Projects, and their sequencing, are covered in Section 10 of this document. |
| Develop and implement procedures and responsibilities for maintaining the architecture as needs evolve within the region. | The Maintenance Plan is contained in Section 13 of this document. |

# ITS Architecture Concepts

The Las Cruces Regional ITS Architecture is an example of a Regional ITS Architecture, which has been defined by FHWA Rule 940 as a “regional framework for ensuring institutional agreement and technical integration for implementation of ITS projects”.  Regional ITS architectures, including the Las Cruces Regional ITS Architecture, are developed in order to provide a guide for the integration of transportation systems.  The architecture is based upon the US National ITS Architecture Version 5.1.  A complete description of this architecture can be found at http://www.iteris.com/itsarch.  The Las Cruces Regional ITS Architecture Update uses a set of common concepts or terms drawn from the National ITS Architecture to describe the parts of the Las Cruces region.  This section will provide a description of the most common concepts or terms as an aid to the understanding the remainder of the document.

What are some of the main parts of an ITS architecture?  They are made of the following:

- Organizations

- Systems operated

- Services provided

- Functions performed

- Information exchanged

The organizations that operate systems in the region covered by the architecture are referred to as stakeholders.  These are public agencies, private organizations or the traveling public with a vested interest, or a "stake" in one or more transportation elements within a Regional ITS Architecture.

The systems operated by the stakeholders are referred to as elements.  In the Las Cruces Regional ITS Architecture the elements represent actual systems, such as City of Las Cruces Transportation Center.  An element may also represent field devices, for example the element City of Las Cruces Traffic Signals.  A more thorough discussion of the architecture elements is contained in Section 5.  As mentioned above, the Las Cruces Regional ITS Architecture Update is based upon the National ITS Architecture which contains general terms for these systems (elements).  Since these National ITS Architecture terms show up repeatedly in later discussion they will be defined here.

The National ITS Architecture uses two terms to describe the systems that make up an architecture.  They are:

- Subsystems, which represent the primary systems described by the architectures.  For example the TMC element mentioned above represents a regional ITS architecture example of the Traffic Management Subsystem defined in the National ITS Architectures.  The National ITS Architecture has 22 subsystems defined.

- Terminators, which represent systems that are on the boundary of the architecture.  In general only interfaces to the terminators are described in the national architectures.  An example of a terminator from the National ITS Architecture is the Weather Service.  The National ITS Architecture has 73 terminators defined.

As a part of developing a regional ITS architecture, each element of the region is mapped to the subsystems and/or terminators that most closely define the functions of the element.  This mapping allows the regional version to use the details associated with the subsystems and terminators in the National ITS Architecture.  As an example, the element in the Las Cruces Regional ITS Architecture Update called National Weather Service is mapped to the National ITS Architecture terminator Weather Service.

The information exchanged between elements (in the Las Cruces Regional ITS Architecture Update) or between subsystems and terminators in the National ITS Architecture is described by information flows or architecture flows.  There are hundreds of these flows defined in the National ITS Architecture, and it is this information that is used to create the interface definitions in the Las Cruces Regional ITS Architecture.  For example, in Figure 3 the top two boxes show an interface between two subsystems, with its information flows defining the exchange of information.  A corresponding interface in the Las Cruces Regional ITS Architecture is shown in the bottom two boxes.

Centre

Operations TOC

NMDOT

District 1 Traffic

Operations Center

Emergency

Preparedness

NM DPS District

Dispatch Center

incident

information

Subsystem

Traffic Management

Subsystem

Subsystem

Emergency

Management

Subsystem

National ITS Architecture representation of an interface

Las Cruces Regional ITS Architecture representation of a similar interface

incident

information

Figure 3.  Information flows

By mapping the Las Cruces Regional ITS Architecture elements (e.g. NMDOT District 1 Traffic Operations Center) to National ITS Architecture subsystems (or terminators) (e.g. Traffic Management Subsystem), the interfaces defined in the National ITS architecture can be used as the basis for defining the interfaces in the Las Cruces Regional ITS Architecture.

The next key concept used by the architectures is that of market packages.  These represent slices of an architecture that provide a transportation service.  In the National ITS architecture, these market packages are combinations of subsystems and information flows that are used to provide the service.  An example of a National ITS Architecture market package is shown in Figure 4.  This shows the subsystems and information flows (some of which go to terminators) that perform the collection and distribution of traffic flow and traffic images used to monitor a road network.  In the development of the Las Cruces Regional ITS Architecture, a set of customized market packages were created that define the elements and interfaces used to provide the transportation services in the Las Cruces Region.

Figure 4.  Example of a National ITS Architecture Market Package

Figure 5 shows one of the customized market packages within the Las Cruces Regional ITS Architecture (in this case for the NMDOT District 1 TOC).  This diagram shows how NMDOT might implement this service.  There are two types of interfaces shown in the customized market package:

- Traffic Management Center to Roadside Equipment and

- Traffic Management Center to Information Service Provider

Notice the name of the service has been changed to Network Monitoring (from Network Surveillance in the original National ITS Architecture example).  This customization of the service was in recognition of the sensitivity that stakeholders expressed regarding the term “surveillance”, and represents just another way that the generic ITS services of the National ITS Architecture are customized for the Las Cruces Regional ITS Architecture.

Figure 5.  Example of Las Cruces Regional ITS Architecture Customized Market Package

Notice that the customized market package includes only some of the interfaces that were in the National ITS Architecture market package.  It does not include interfaces to personnel or a map update provider element.  Elements mapping to these are not included in the Las Cruces Regional ITS Architecture.

One final concept to mention relates to the functions performed by the elements in the architecture.  The National ITS Architecture has the concept of an equipment package, which defines a piece of a subsystem (within the market package) that performs a specific function.  For example, in Figure 4, Collect Traffic Surveillance (identified by the white box within the Traffic Management Subsystem) is a function (or equipment package) that is performed by the Traffic Management Subsystem when performing the Network Monitoring Transportation Service.  In the Las Cruces Regional ITS Architecture functions have been identified for the key elements from a mapping of equipment packages to elements using a mapping of equipment packages to each element.  For example, the NMDOT District 1 Traffic Operations Center (shown in Figure 3) will implement the Collect Traffic Surveillance equipment package (shown in Figure 4 as functionality in the Traffic Management Subsystem).  Further information regarding how functions are defined for each element is found in Section 8 on Functional Requirements.

# Identification of Stakeholders

## Champion

In order to successfully develop a Regional ITS Architecture for the Las Cruces Region, it is necessary to have a “champion” who can lead the effort from the agency’s viewpoint.  This individual, or group of individuals, should have the following skills/capabilities:

- They must have a vision for interconnectivity, partnership and regional integration.

- They must have knowledge of the local and regional ITS systems and projects.

- They must understand what a regional ITS architecture is and how to use it most effectively in the planning process.

- They must be a consensus builder or facilitator, and

- They must have executive level access to resources in order to gain the support of various regional or statewide agencies.

The Champion for the development of the Las Cruces Regional ITS Architecture is the Las Cruces Metropolitan Planning Organization (MPO).  The champion is supported by the Las Cruces ITS Steering Committee comprised of representatives of key stakeholders within the region.  A designated Las Cruces MPO planning staff person will continue to champion the use and maintenance of the Las Cruces Regional ITS Architecture (along with input and consensus from the ITS Steering Committee) beyond the timeframe of this development effort.

## Regional Stakeholders

Stakeholder coordination and involvement is one of the key elements to the development of a regional ITS architecture.  Because ITS often transcends traditional transportation infrastructure, it is important to consider a range of stakeholders beyond the traditional traffic, transit, and maintenance areas.  In addition, it is important to consider stakeholders at a regional level in adjoining regions of the state.

The Las Cruces Regional ITS Architecture includes a wide range of stakeholders.  Many of these stakeholders, along with various other agencies, were present at one of the stakeholder meetings described in Section 2 above.  The following is a list of agencies/participants who were either present during the architecture development workshops and provided their input during those workshops, or who were contacted specifically by ConSysTec Corporation for their inputs on the Las Cruces Regional ITS Architecture:

- City of Las Cruces – Traffic Operations

- Dona Ana County - Engineering

- Elephant Butte Irrigation District

- FHWA

- Las Cruces Metropolitan Planning Organization

- Las Cruces Public School District

- Mesilla Valley Regional Dispatch Authority

- New Mexico State University

- NMDOT – District 1

- NMDOT - ITS Bureau

- RoadRUNNER Transit

The Las Cruces Regional ITS Architecture is defined by a set of elements (or systems), each of which is owned (or operated or maintained) by a stakeholder.  The above listing includes all the agencies that attended stakeholder meetings.  Most, but not all of them own, operate, or maintain elements in the architecture.  (An example of an agency that does not is FHWA, whose role in the Las Cruces Regional ITS Architecture is approval of the ITS Architecture and approval of projects for funding by the federal government that are represented therein.)  Table 2 provides a listing of the full range of stakeholders who own or operate elements in the Las Cruces Regional ITS Architecture.  The table provides a name and description of the agency, department, or organization represented by the stakeholder.

The list of stakeholders can also be found on the Las Cruces Regional Architecture web page by selecting the “Stakeholders” button on the left side menu bar.

Table 2.  Stakeholders

| Stakeholder Name | Stakeholder Description |
| --- | --- |
| Archive Data Users | Users (and their systems) of general archive data. |
| City of Las Cruces | Municipal government agencies in the City of Las Cruces. Responsible for the City's website, department of commerce, convention and visitor's bureau, the City's Office of Emergency Management, etc. |
| City of Las Cruces Department of Public Works | City of Las Cruces department responsible for traffic operations as well as maintenance and construction activities (including roadway maintenance and signal maintenance) throughout the City. The department is also responsible for the installation and maintenance of traffic signals, street lights, flashing beacons, signs, markings, and the permit parking program for the City. |
| City of Las Cruces Fire Department | City of Las Cruces department responsible for fire and EMS dispatch and response. |
| City of Las Cruces Police Department | City of Las Cruces department responsible for police dispatch and response. |
| Contract Transport Companies | Emergency medical service providers that offer ambulance services and medical transports. |
| Dona Ana County | County government agencies for Dona Ana County. Responsible for the County's website, tourism, convention and visitor's bureau, the County's Office of Emergency Management, etc. |
| Dona Ana County Fire Department | The fire and emergency response agency for Dona Ana County. Includes the Dona Ana County fire and EMS. |
| Dona Ana County Flood Commission | The flood commission responsible for the dams, Rio Grande River and irrigation systems in and around the Las Cruces region. |
| Dona Ana County Public Works Division | The Dona Ana County department responsible for the maintenance, construction, and traffic operations of county roads. |
| Dona Ana County Sheriffs Office | The Dona Ana County office that dispatches the Dona Ana County Sheriff vehicles to incidents on the County's roads. |
| Elephant Butte Irrigation District | The Elephant Butte Irrigation District measures river water levels through its Water Resource Information System (WRIS) and in the future may have additional water flow measuring devices that would help warn of flooding at other places in the regional transportation network. |
| Financial Institution | Financial and banking institutions that play a role in the transfer of funds for fare collection as well as for other fee based transportation services. |
| Independent Public School Districts | Gadsden Independent Schools or Hatch Valley Municipal School District. The districts have been included for their role in providing "fixed route" transit services and in supporting emergency evacuations with their fleet of school buses. |
| Las Cruces MPO | The Las Cruces Metropolitan Planning Organization (MPO) for the Las Cruces area. The MPO is responsible for carrying out the metropolitan transportation planning process which includes developing transportation plans and programs including the LRTP and TIP. |
| Las Cruces Public School District | The public school district for the City of Las Cruces. The district has been included for their role in providing "fixed route" transit services and in supporting emergency evacuations with their fleet of school buses. |
| Local Media | Represents both print (newspaper) and broadcast (TV, radio) news media. |
| Local Transit Agencies | Represents local transit agencies that operate in the Region that are not specifically called out by this architecture. |
| Mesilla Valley Regional Dispatch Authority | The designated Public Safety Answering Point (PSAP) for Dona Ana County. Intended to be able to route calls to local emergency, medical, fire and law enforcement. |
| Municipal Public Safety | Represents local or municipal public safety (fire/EMS districts and municipal police) that are not explicitly represented in the architecture. This includes dispatch functions, PSAPs (public safety answering points), etc. operated at the municipal level (in the future these may be regional in nature). |
| Municipalities | Represents the municipalities or villages located within the Region that are not specifically called out by the ITS architecture. |
| New Mexico Department of Public Safety - DPS | New Mexico Department of Public Safety includes the State Police and the Motor Transport Division. |
| New Mexico General Services Department | State department providing general support services for the state government of New Mexico. Divisions include the Information Systems Division (ISD), which is responsible for providing data processing, application development, telecommunications and radio communications resources and services to the State of New Mexico (operation of the Radio Communications Bureau). |
| New Mexico National Guard | The National Guard for the State of New Mexico. |
| New Mexico State University | The New Mexico State University. Includes all departments of the University, including the NMSU events department and the NMSU Police Department. |
| NMDOT - New Mexico Department of Transportation | The Statewide stakeholder for the New Mexico Department of Transportation. |
| NMDOT District 1 | Represents the District 1 offices of the New Mexico Department of Transportation, located in Las Cruces, New Mexico. |
| NOAA | National Oceanic and Atmospheric Administration (includes the National Weather Service). |
| Other Counties | Represents the government agencies in adjacent counties to the Las Cruces MPO geographic region. |
| Private HAZMAT Providers | Private responders to HAZMAT incidents. They are secondary responders to incidents. |
| Private Sector Traveler Information Service Providers | Local, regional and national information service providers that provide travel information to the traveling public (both subscription service and general broadcast information). Includes internet sites, service bureaus, etc |
| Private Tow/Wrecker Providers | Private companies that provide tow or wrecker services for the Region. |
| Private Weather Information Provider | Weather information providers that provide regional weather information to regional agencies. |
| Public/Private Utilities | Public or private utility companies that operate within the Region. |
| Rail Operators | Operators of private rail systems (e.g. BNSF, Amtrak, etc.), including passenger and freight. |
| Regional Airports | Represents the regional airports that are within the Region. |
| Regional Event Coordinators | This entity supports the information exchange/coordination with various special event activities. It also includes groups that promote events, such as a Chamber of Commerce or Visitor's Bureau. |
| Regional Medical Centers | Agencies that own hospitals and trauma centers. Includes the personnel and equipment to operate Life Flight systems. |
| Regional Public Safety Agencies | This stakeholder is a stakeholder group representing all public safety agencies within the Region. |
| Regional Transit Providers | This stakeholder is a stakeholder group representing all the Region's transit agencies and transit operators. |
| RoadRUNNER Transit | The public transportation system of the City of Las Cruces. Includes fixed route operations and paratransit/demand-response operations (Dial-A-Ride). |
| South Central Regional Transit District | The rural transit district that has the responsibility for transit system coordination in the RPO in surrounding counties and NMDOT Districts. |
| Southwest Regional Transit District | The rural transit district that has the responsibility for transit system coordination in the RPO around Las Cruces and NMDOT District 1. |
| Texas Department of Public Safety | The department of Texas Government responsible for the Texas Highway Patrol. |
| Travelers | Traveling public accessing various modes of transportation, including surface street, air, rail/transit, and non-motorized. |
| TxDOT | Texas Department of Transportation |
| U.S. Department of Defense | The Department of Defense operates bases (both army and air force) within the state. |
| US Customs and Border Protection | This agency of the Department of Homeland Security is charged with providing security at the US border. |

The stakeholders listed in Table 2 represent a mix of specific agencies or organizations and generic names used to represent a variety of stakeholders.  An example of a specific agency or organization from the architecture is City of Las Cruces Department of Public Works, which represents the City of Las Cruces department responsible for traffic operations and maintenance throughout the City.  An example of a generic stakeholder name would be Municipalities, which represents all of the departments or agencies (police, fire, EMS, traffic, maintenance and construction, etc.) at the municipal level.  Another example of a general stakeholder in the architecture definition is Local Transit Agencies, which would represent the any transit agencies not specifically called out in the architecture but operates within the MPO region.

## Operational Concepts

An operational concept documents each stakeholder’s current and future roles and responsibilities in the operation of the regional ITS system.  The operational concept documents these roles and responsibilities across a range of transportation services.  The services covered by the Las Cruces Regional ITS Architecture include:

- Traffic Signal Control.  The development of traffic signal systems that react to changing traffic conditions and provide coordinated intersection timing over a corridor, an area, or multiple jurisdictions.

- Highway Management.  The development of systems to monitor freeway traffic flow and roadway conditions, and provide strategies such as ramp metering or lane access control to improve the flow of traffic on the freeway.  Includes systems to provide information to travelers on the roadway.

- Incident Management.  The development of systems to provide rapid and effective response to incidents.  Includes systems to detect and verify incidents, along with coordinated agency response to the incidents.

- Transit Management.  The development of systems to more efficiently manage fleets of transit vehicles or transit rail.  Includes systems to provide transit traveler information both pre-trip and during the trip as well as electronic fare payment systems used on transit vehicles.

- Traveler Information.  The development of systems to provide static and real time transportation information to travelers.

- Emergency Management.  The development of systems to provide emergency call taking, public safety dispatch, and emergency operations center operations.

- Maintenance and Construction Management.  The development of systems to manage the maintenance of roadways in the region, including winter snow and ice clearance.  Includes the managing of construction operations.

- Archive Data Management.  The development of systems to collect transportation data for use in non-operational purposes (e.g.  planning and research).

identifies the roles and responsibilities of key stakeholders for the specified range of transportation services.

Table 3.  Stakeholder Roles and Responsibilities

| Transportation Service | Stakeholder | Roles/ Responsibilities |  |
| --- | --- | --- | --- |
| Traffic Signal Control | City of Las Cruces Department of Public Works |  | Operate network monitoring equipment (CCTV Cameras, field sensors, etc.) on City streets. |
|  |  |  | Operate traffic signal systems on City owned streets, including traffic signals, sensor systems (CCTV), and right of way requests. |
|  |  |  | Provide traffic information reports to regional information service providers, private information service providers, and the NMDOT Advanced Traveler Information System. |
|  |  |  | Provide traffic information to regional agencies including transit, emergency management, maintenance and construction, and the media. Provide traffic information to travelers through City's DMS. |
|  |  |  | Coordinate traffic information and control with other municipal and county TOCs, and NMDOT TMCs. |
|  |  |  | Coordinate HRI signal adjustments with private Rail Operators and NMDOT. |
|  |  |  | Provide security monitoring of critical infrastructure for the City. |
|  |  |  | Provide transit signal priority for regional fixed route transit vehicles. |
|  |  |  | Provide emergency signal preemption for the region's Fire and EMS vehicles. |
|  |  |  | Operate network monitoring equipment (CCTV Cameras, field sensors, etc.) on City streets. |
|  | Dona Ana County Public Works Division |  | Operate traffic signal systems on county owned streets, including traffic signals, sensor systems, and right of way requests. |
|  |  |  | Provide traffic information reports to regional information service providers, private information service providers, and the New Mexico Advanced Traveler Information System. |
|  |  |  | Provide traffic information to regional agencies including transit, emergency management, maintenance and construction, and the media. Provide traffic information to travelers through County DMS. |
|  |  |  | Coordinate traffic information and control with municipal TOCs and NMDOT TMCs. |
|  |  |  | Coordinate HRI signal adjustments with private Rail Operators. |
|  |  |  | Operate reversible lane signals and controls on County streets and bridges. |
|  |  |  | Provide emergency signal preemption for regional Fire and EMS vehicles. |
|  |  |  | Operate network monitoring equipment (CCTV Cameras, field sensors, etc.) on State owned arterials. |
|  | New Mexico State University |  | Operate network monitoring equipment (CCTV Cameras, field sensors, etc.) on NMSU campus. |
|  |  |  | Provide traffic information reports to regional information service providers, private information service providers, and the university’s website. |
|  |  |  | Coordinate traffic information and control with municipal and county TOCs and with NMDOT TOC. |
|  |  |  | Provide traffic information to regional agencies including transit, emergency management, maintenance and construction, and the media. Provide traffic information to travelers through university-owned DMS. |
|  | NMDOT |  | Operate traffic signal systems on State owned arterials, including traffic signals, sensor systems, and right of way requests. |
|  |  |  | Provide traffic information reports to regional information service providers, private information service providers, and the New Mexico Advanced Traveler Information System. |
|  |  |  | Provide traffic information to regional agencies including transit, emergency management, maintenance and construction, and the media. Provide traffic information to travelers through State owned DMS. |
|  |  |  | Coordinate traffic information and control with municipal and county TOCs and with other NMDOT TOCs/TMC. |
|  |  |  | Coordinate HRI signal adjustments with private Rail Operators and NMDOT. |
|  |  |  | Operate reversible lane signals and controls on NMDOT arterials, highways, and bridges. |
|  |  |  | Provide security monitoring of critical infrastructure for the State. |
|  |  |  | Operate network monitoring equipment (CCTV Cameras, field sensors, etc.) on State owned highways. |
| Highway Management | NMDOT |  | Operate lane control equipment on State owned highways. |
|  |  |  | Operate HOV lanes, or HOV lane controls, on State owned highways or near river crossings. |
|  |  |  | Provide traffic information reports to regional information service providers, private information service providers, and the New Mexico Advanced Traveler Information System. |
|  |  |  | Provide traffic information to regional transportation agencies and the general public through traffic information devices (DMS, Highway Advisory Radio, HCRS, NMDOT Advanced Public Information System, etc.). |
|  |  |  | Coordinate rail schedules and adjustments for high speed rail intersections with private rail operators and NMDOT/Las Cruces MPO. |
|  |  |  | Operate reversible lane signals and controls on NMDOT arterials, highways, and bridges. |
|  |  |  | Provide security monitoring of critical infrastructure for the State. |
|  |  |  | Coordinate traffic information and traffic control with other regional TMCs. |
|  |  |  | Coordinate traffic information other State's regional TMCs or Statewide TMC. |
|  |  |  | Perform network monitoring for detection and verification of incidents on state owned streets. |
| Incident Management (Traffic) | City of Las Cruces Department of Public Works |  | Provide incident information to the City's emergency responders through the Mesilla Valley Public Safety Communications Center and directly to other regional emergency responders. |
|  |  |  | Coordinate maintenance resources for incident response. |
|  |  |  | Perform network monitoring for detection and verification of incidents on City-owned streets. |
|  | Dona Ana County Public Works Division |  | Provide incident information to the County's emergency responders through Mesilla Valley Public Safety Communications Center and directly to other regional emergency responders. |
|  |  |  | Coordinate maintenance resources for incident response with the County's Public Works Department. |
|  |  |  | Perform network monitoring for detection and verification of incidents on State owned highways and arterials. |
|  | NMDOT |  | Provide incident information to travelers via traffic information devices on highways (e.g. DMS, NMDOT Advanced Traveler Information Service, etc). |
|  |  |  | Provide incident information to the City's emergency responders through the Mesilla Valley Public Safety Communications Center and directly to regional emergency responders, including the State Police, County Sheriff, County Fire and EMS, and City Police, Fire and EMS. |
|  |  |  | Coordinate maintenance resources for incident response with the NMDOT’s Maintenance and Construction Division. |
|  |  |  | Coordinate with State Police vehicles for incidents on the region's highways. |
| Incident Management (Emergency) | New Mexico Department of Public Safety - DPS |  | Coordinate incident response with other public safety agencies (police, fire, EMS, sheriff, etc.) as well as with NMDOT. |
|  |  |  | Perform incident detection and verification for the highways within the region, and provide this information to traffic and other public safety agencies. |
|  |  |  | Coordinate maintenance resources in response to incidents on state highways with regional maintenance providers. |
|  |  |  | Coordinate an incident response with regional rail operations for incidents involving rail. |
|  |  |  | Receive cellular emergency calls for incidents within the county. |
|  | Mesilla Valley Regional Dispatch Authority |  | Coordinate public safety resources for incident response with the City/ County/Regional TOCs. |
|  |  |  | Coordinate incident response with all City/County public safety agencies (police, fire and EMS). |
|  |  |  | Dispatch and track the location of municipal police vehicles, county sheriff vehicles, fire vehicles and EMS vehicles as well as coordinate with all other public safety agencies within the region. |
|  |  |  | Perform incident detection and verification for the streets within the region, and provide this information to regional TOCs. |
|  |  |  | Coordinate an incident response with regional rail operations for incidents involving rail. |
|  |  |  | Coordinate maintenance resources in response to incidents within the region. |
|  |  |  | Coordinate with ambulance services in response to incidents within the region. |
|  |  |  | Receive emergency calls for incidents within the region. |
|  | City of Las Cruces Public Safety Department |  | Coordinate public safety resources for incident response with the City's TOC. |
|  |  |  | Coordinate incident response with all City public safety agencies (police, fire and EMS). |
|  |  |  | Dispatch the City's police vehicles (and track their location), fire vehicles and EMS vehicles as well as coordinate with all other public safety agencies within the region. |
|  |  |  | Perform incident detection and verification for the streets within the City, and provide this information to the City's TOC. |
|  |  |  | Coordinate an incident response with regional rail operations for incidents involving rail. |
|  |  |  | Coordinate maintenance resources in response to incident with the City. |
|  |  |  | Coordinate with ambulance services in response to incidents within the City. |
|  | New Mexico State University Police Department |  | Coordinate public safety resources for incident response with the university campus. |
|  |  |  | Dispatch the university's police and fire vehicles and coordinate with all other public safety agencies within the region for incidents. |
|  |  |  | Perform incident detection and verification on the university campus, and provide this information to the university’s TOC. |
|  |  |  | Coordinate maintenance resources in response to incident on campus. |
|  |  |  | Coordinate public safety resources for incident response with the university’s TOC. |
|  | Dona Ana County Public Safety |  | Dispatch the County's sheriff vehicles (and track their location) and fire and EMS vehicles as well as coordinate with all other public safety agencies within the region. |
|  |  |  | Perform incident detection and verification for the streets within the county, and provide this information to the regional TMCs. |
|  |  |  | Coordinate an incident response with regional rail operations for incidents involving rail. |
|  |  |  | Coordinate maintenance resources in response to incident with the County. |
|  | NMDOT |  | Perform incident detection and verification on state highways, and provide this information to the regional TMCs. |
|  |  |  | Coordinate incident response with the state police and regional public safety providers. |
| Transit Management | RoadRUNNER Transit |  | Provide transit schedule and fare information to the agency's transit information center, the New Mexico Advanced Traveler Information System, regional traveler information providers, and private sector traveler information service providers. |
|  |  |  | Provide fixed route bus service and a fixed route light rail service for the City and outlying areas (on agreement). |
|  |  |  | Provide demand response (paratransit) bus service for the agency defined service area, with the ability to provide a demand response transit plan from the agency's transit information center. |
|  |  |  | Provide transit passenger electronic fare payment on all agency fixed route, light rail, and paratransit vehicles. |
|  |  |  | Provide transit security on all agency transit vehicles and at transit transfer centers through silent alarms, sensors, and monitoring systems. |
|  |  |  | Provide automated transit maintenance scheduling through automated vehicle condition reports on all agency fixed route, light rail and paratransit vehicles. |
|  |  |  | Obtain traffic signal priority for designated fixed route transit vehicles from the city's within the agency's service area through the city's field equipment. |
|  |  |  | Coordinate transit service with other regional transit providers, as well as regional intermodal terminals and regional airports. |
|  |  |  | Provide transit traveler information to the agency website, private sector traveler information services, and the New Mexico Advanced Traveler Information System, as well as making it available on all transit information kiosks. |
|  |  |  | Coordinate emergency plans with Municipal, County, and Statewide EOCs and provide emergency transit services for evacuations, fires, and disasters (including re-entry). |
|  |  |  | Track vehicle location and evaluate schedule performance on all agency transit vehicles. |
|  | South Central Regional Transit District |  | Provide transit schedule and fare information to the RTD transit website, the New Mexico Advanced Traveler Information System, and private sector traveler information service providers. |
|  |  |  | Provide fixed route transit service for the specified transit district. |
|  |  |  | Provide demand response (paratransit) bus service for the agency defined service area, with the ability to provide a demand response transit plan from the agency's transit information center. |
|  |  |  | Provide transit passenger electronic fare payment on all agency transit vehicles. |
|  |  |  | Provide transit security on all agency transit vehicles through silent alarms and monitoring systems. |
|  |  |  | Provide automated transit maintenance scheduling through automated vehicle conditions reports on all RTD transit vehicles. |
|  |  |  | Coordinate transit service with other regional transit providers, as well as regional intermodal terminals and regional airports. |
|  |  |  | Provide transit traveler information to the agency website, private sector traveler information services, and the New Mexico Advanced Traveler Information System, as well as making it available on all transit information kiosks. |
|  |  |  | Coordinate emergency plans with Municipal, County, and Statewide EOCs and provide emergency transit services for evacuations, fires, and disasters (including re-entry). |
|  |  |  | Track vehicle location and evaluate schedule performance on all regional transit vehicles. |
|  | Local Transit |  | Provide transit schedule and fare information to the agency's website, the New Mexico Advanced Traveler Information System, and private sector traveler information service providers. |
|  |  |  | Track vehicle location and evaluate schedule performance on all agency transit vehicles. |
|  |  |  | Provide demand response (paratransit) bus service for the agency defined service area, with the ability to provide a demand response transit plan from the agency's transit information center. |
|  |  |  | Provide transit passenger electronic fare payment on all transit vehicles. |
|  |  |  | Provide transit security on all agency transit vehicles through silent alarms and monitoring systems. |
|  |  |  | Provide automated transit maintenance scheduling through automated vehicle condition reports on all agency transit vehicles. |
|  |  |  | Coordinate transit service with other regional transit providers, as well as regional intermodal terminals and regional airports. |
|  |  |  | Provide transit traveler information to the agency website, private sector traveler information services, and the New Mexico Advanced Traveler Information System, as well as making it available on all transit information kiosks. |
|  |  |  | Coordinate emergency plans with Municipal, County, and Statewide EOCs and provide emergency transit services for evacuations, fires, and disasters (including re-entry). |
|  |  |  | Track vehicle location and evaluate schedule performance on all agency transit buses. |
|  | NMDOT |  | Provide transit schedule and fare information to the District 1 website, the New Mexico Advanced Traveler Information System, and private sector traveler information service providers. |
|  |  |  | Provide fixed route shuttle bus service for the agency defined service area. |
|  |  |  | Provide transit security on all agency transit vehicles and at parking facilities through silent alarms, sensors, and monitoring systems. |
|  |  |  | Provide automated transit maintenance scheduling through automated vehicle condition reports on all agency shuttles and rail vehicles. |
|  |  |  | Coordinate transit service with other regional transit providers, as well as regional intermodal terminals and regional airports. |
|  |  |  | Provide transit traveler information to the agency website, private sector traveler information services, and the New Mexico Advanced Traveler Information System, as well as making it available on all transit information kiosks. |
|  |  |  | Coordinate emergency plans with Municipal, County, and Statewide EOCs and provide emergency transit services for evacuations, fires, and disasters (including re-entry). |
|  | Independent School Districts |  | Provide school bus schedule and route information to the school district website. |
|  |  |  | Provide fixed route school bus service for the cities, municipalities, and counties within the region. |
|  |  |  | Provide transit security on all transit vehicles through silent alarms and monitoring systems. |
|  |  |  | Provide automated transit maintenance scheduling through automated vehicle conditions reports on all Independent School District buses. |
|  |  |  | Coordinate emergency plans with Municipal, County, and Statewide EOCs and provide emergency transit services for evacuations, fires, and disasters (including re-entry). |
|  |  |  | Track vehicle location and evaluate schedule performance on all school district buses. |
|  | New Mexico State University |  | Provide transit schedule and fare information to the NMSU website and to private sector traveler information service providers. |
|  |  |  | Provide fixed route bus service for the university and the surrounding area. |
|  |  |  | Provide transit passenger electronic fare payment on all courtesy shuttles. |
|  |  |  | Provide transit security on all agency transit vehicles through silent alarms and monitoring systems. |
|  |  |  | Provide automated transit maintenance scheduling through automated vehicle condition reports on all agency courtesy shuttles. |
|  |  |  | Coordinate transit service with other regional transit providers, as well as regional intermodal terminals and regional airports. |
|  |  |  | Coordinate emergency plans with Municipal, County, and Statewide EOCs and provide emergency transit services for evacuations, fires, and disasters (including re-entry). |
|  |  |  | Track vehicle location and evaluate schedule performance on all NMSU transit and courtesy vehicles. |
| Traveler Information | NMDOT |  | Collect traffic, maintenance and construction information, and transit schedule and fare information from regional providers. |
|  |  |  | Operate the NMDOT Advanced Traveler Information System. |
|  |  |  | Provide traffic, maintenance and construction and transit information to the media, private travelers, and various traveler information services (including the agency website). |
|  |  |  | Provide broadcast information to travelers. |
|  |  |  | Coordinate and share traveler information with all other traveler information providers within the region. |
|  |  |  | Provide traveler information to private travelers (in vehicle, personal computing device, or kiosk) upon request. |
|  |  |  | Provide traveler information to the media. |
|  | Private Traveler Information Service Providers |  | Collect traffic, incident, transit schedule, road maintenance and weather information and provide it to the media and private travelers. |
|  |  |  | Coordinate and share traveler information with all other traveler information providers within the region. |
|  | City of Las Cruces |  | Collect traffic, maintenance and construction information, and transit schedule and fare information from regional providers. |
|  |  |  | Provide traffic, maintenance and construction and transit information to the media, private travelers, and various traveler information services (including the agency website). |
|  | Dona Ana County |  | Collect traffic, maintenance and construction information, and transit schedule and fare information from regional providers. |
|  |  |  | Provide traffic, maintenance and construction and transit information to the media, private travelers, and various traveler information services (including the agency website). |
| Emergency Management | New Mexico Department of Public Safety - DPS |  | Dispatch State Police vehicles (and track their location) as well as coordinate with all other public safety agencies within the region. |
|  |  |  | Participate in the incident response, coordination, and reporting of the Regional Communications Network in a coordination effort only (no dispatch function). |
|  |  |  | Coordinate with regional hospitals for care facility status. |
|  |  |  | Receive and respond to threat information from the state agency, City or municipality TOC regarding critical infrastructure. |
|  |  |  | Generate Amber Alerts and distribute them to regional emergency management agencies, transit agencies, traffic agencies, and the media. |
|  |  |  | Generate and coordinate wide area alerts and distribute them to regional emergency management agencies, transit agencies, traffic agencies, regional information service providers, and the media. |
|  |  |  | Plan and coordinate region wide emergency plans, evacuation and reentry plans, and disaster management plans dealing with HAZMAT incidents. |
|  |  |  | Provide regional traffic, transit, emergency management, and maintenance operations with disaster information to disseminate to the traveling public. |
|  |  |  | Aid in the coordination of region wide emergency plans, evacuation and reentry plans, and disaster management plans. |
|  |  |  | Receive early warning information and threat information from the statewide EOC and/or the department of homeland security. |
|  |  |  | Operate the New Mexico Statewide EOC. |
|  | Mesilla Valley Regional Dispatch Authority |  | Dispatch the regional (city/ county) police, fire and EMS vehicles (and track their location) and coordinate with all other public safety agencies and communications centers within the region. |
|  |  |  | Coordinate with regional hospitals for care facility status as well as patient care and status. |
|  |  |  | Track location of regional public safety vehicles. |
|  |  |  | Coordinate signal preemption with City of Las Cruces for EMS vehicles. |
|  |  |  | Receive and respond to threat information from the City's TOC regarding critical infrastructure. |
|  |  |  | Operate the regional EOC. |
|  |  |  | Receive Amber Alert and other Wide Area Alert information from state police. |
|  |  |  | Receive Wide Area Alert information from the Statewide EOC and the regional EOCs. |
|  |  |  | Receive early warning information and threat information from the statewide EOC and/or the department of homeland security. |
|  |  |  | Responding to transit emergencies/alarms on-board transit vehicles or at regional transit facilities. |
|  |  |  | Aid in the coordination of region wide emergency plans, evacuation and reentry plans, and disaster management plans. |
|  |  |  | Provide evacuation, incident, and transportation system status information to regional public information systems. |
|  | City of Las Cruces Public Safety Department |  | Participates in the incident response, coordination, and reporting of the Regional Communications Network in a coordination effort only (no dispatch function). |
|  |  |  | Dispatch the City's police, fire and EMS vehicles (and track their location) and coordinate with all other public safety agencies and communications centers within the region. |
|  |  |  | Track location of city public safety vehicles. |
|  |  |  | Coordinate with regional hospitals for care facility status as well as patient care and status. |
|  |  |  | City Fire and EMS receive local signal preemption at the City of Las Cruces traffic signals. |
|  |  |  | Receive Amber Alert and other Wide Area Alert information from state police. |
|  |  |  | Receive Wide Area Alert information from the Statewide EOC and regional EOCs. |
|  |  |  | Receive early warning information and threat information from the statewide EOC and/or the department of homeland security. |
|  |  |  | Responding to transit emergencies/alarms on-board transit vehicles or at regional transit facilities. |
|  |  |  | Aid in the coordination of region wide emergency plans, evacuation and reentry plans, and disaster management plans. |
|  |  |  | Provide evacuation, incident, and transportation system status information to regional public information systems. |
|  | Dona Ana County Public Safety |  | Dispatch the County's public safety vehicles (and track their location) as well as coordinate with all other public safety agencies within the region. |
|  |  |  | Coordinate with regional hospitals for patient care and status. |
|  |  |  | Receive local signal preemption from County signals. |
|  |  |  | Perform incident detection and verification for streets within the counties of the region. |
|  |  |  | Receive Amber Alert and other Wide Area Alert information from state police. |
|  |  |  | Receive Wide Area Alert information from the Statewide EOC and the regional EOCs. |
|  |  |  | Receive early warning information and threat information from the statewide EOC and/or the department of homeland security. |
|  |  |  | Aid in the coordination of region wide emergency plans, evacuation and reentry plans, and disaster management plans. |
|  |  |  | Provide evacuation, incident, and transportation system status information to regional public information systems. |
|  | NMDOT |  | Dispatch the NMDOT D1 HELP Courtesy Patrol vehicles (and track their location) and coordinate with all other public safety agencies and communications centers within the region. |
|  |  |  | Dispatch the NMDOT Motor Transport Division response vehicles (and track their location). |
|  |  |  | Provide local signal preemption to County/Municipal public safety vehicles. |
|  |  |  | Receive Amber Alert and other Wide Area Alert information from state police and post alert information on NMDOT DMS. |
|  |  |  | Aid in the coordination of region wide emergency plans, evacuation and reentry plans, and disaster management plans. |
|  |  |  | Provide evacuation, incident, and transportation system status information to regional public information systems, the media, and on the NMDOT website. |
|  | New Mexico General Services Department |  | Dispatch the forest service and other public department vehicles (and track their location) and coordinate with all other public safety agencies and communications centers within the region. |
|  |  |  | Aid in the coordination of region wide emergency plans, evacuation and reentry plans, and disaster management plans. |
|  | New Mexico National Guard |  | Dispatch the New Mexico National Guard vehicles (and track their location) and coordinate with all other public safety agencies and communications centers within the region. |
|  |  |  | Receive Amber Alert and other Wide Area Alert information from state police. |
|  |  |  | Receive Wide Area Alert information the Statewide EOC. |
|  |  |  | Receive early warning information and threat information from the statewide EOC and/or the department of homeland security. |
|  |  |  | Aid in the coordination of region wide emergency plans, evacuation and reentry plans, and disaster management plans. |
|  | Contract Transport Providers (Ambulance Services) |  | Dispatch the private ambulances and other EMS vehicles (and track their location) and coordinate with all other public safety agencies and communications centers within the region. |
|  |  |  | Coordinates with regional medical centers regarding the status of the care facility as well as the patient status en route to the medical center. |
|  | New Mexico State University |  | Dispatch the University Campus Police vehicles (and track their location) and coordinate with all other public safety agencies and communications centers within the region. |
|  |  |  | Receive Amber Alert and other Wide Area Alert information from state police. |
|  |  |  | Receive Wide Area Alert information from the Statewide EOC and the regional EOCs. |
|  |  |  | Operate the University's EOC. |
|  |  |  | Receive early warning information and threat information from the statewide EOC and/or the department of homeland security. |
|  |  |  | Aid in the coordination of region wide emergency plans, evacuation and reentry plans, and disaster management plans. |
|  |  |  | Provide evacuation, incident, and transportation system status information to regional public information systems. |
| Maintenance and Construction Management | City of Las Cruces Department of Public Works |  | Provide maintenance resources to assist regional emergency management agencies in incident response. |
|  |  |  | Coordinate maintenance resources for incidents with other regional maintenance providers. |
|  |  |  | Receive vehicle location information from agency maintenance and construction vehicles. |
|  |  |  | Receive vehicle maintenance conditions from agency maintenance and construction vehicles and coordinate fleet maintenance with agency equipment repair facility/garage. |
|  |  |  | Collect environmental sensor data with agency field equipment and distribute road weather information to regional traffic, maintenance and transit agencies as well as the national weather service and the media. |
|  |  |  | Provide road weather information to regional emergency management providers, regional traffic agencies, regional transit agencies, and regional public information services. |
|  |  |  | Provide winter road maintenance for all City streets and provide roadway maintenance status information to regional public safety agencies, regional transit agencies, and the agency's traffic center. |
|  |  |  | Provide maintenance of streets within the city, including pavement maintenance and all construction activities. |
|  |  |  | Dispatch City maintenance vehicles. |
|  |  |  | Provide maintenance to all field equipment owned and operated by the City. |
|  |  |  | Manage work zones on all City maintenance and construction activities, as well as monitors work zone safety with agency field devices and vehicles. |
|  |  |  | Coordinate maintenance and construction activities with other regional maintenance and construction agencies. |
|  |  |  | Distributes maintenance and construction plans and work zone information to regional information service providers, regional traffic operations, emergency operations, rail operations, and the media. |
|  | New Mexico State University |  | Receive vehicle location information from the university’s maintenance and construction vehicles. |
|  |  |  | Receive vehicle maintenance conditions from university maintenance and construction vehicles and coordinate fleet maintenance with agency equipment repair facility/garage. |
|  |  |  | Provide winter road maintenance for all campus streets and provide roadway maintenance status information to the university campus police and fire departments, Aggie Shuttle, and the university's traffic center. |
|  |  |  | Provide maintenance of streets within campus, including pavement maintenance and all construction activities. |
|  |  |  | Provide maintenance status to the Aggie Shuttle, campus safety agencies, and to the university website. |
|  |  |  | Dispatch the university’s maintenance and construction vehicles. |
|  |  |  | Provide maintenance to all field equipment owned and operated by the university. |
|  |  |  | Manage work zones on campus streets. |
|  |  |  | Distribute maintenance and construction plans and work zone information to regional information service providers, regional traffic operations, emergency operations, rail operations, and the media. |
|  | Dona Ana County Public Works Division |  | Provide maintenance resources to assist regional emergency management agencies in incident response. |
|  |  |  | Coordinate maintenance resources for incidents with other regional maintenance providers. |
|  |  |  | Receive vehicle location information from agency maintenance and construction vehicles. |
|  |  |  | Receive vehicle maintenance conditions from agency maintenance and construction vehicles and coordinate fleet maintenance with agency equipment repair facility/garage. |
|  |  |  | Collect road weather information with agency field equipment and distribute it to regional traffic, maintenance and transit agencies as well as the national weather service and the media. |
|  |  |  | Provide road weather information to regional emergency management providers, regional traffic agencies, regional transit agencies, and regional public information services. |
|  |  |  | Provide winter road maintenance for all County roads and provide roadway maintenance status information to regional public safety agencies, regional transit agencies, and the agency's traffic center. |
|  |  |  | Provide maintenance of streets within the counties, including pavement maintenance and all construction activities. |
|  |  |  | Provide maintenance status to regional transit agencies, municipal public safety agencies, and to travelers (through ISPs). |
|  |  |  | Dispatch County maintenance vehicles. |
|  |  |  | Provide maintenance to all field equipment owned and operated by the County. |
|  |  |  | Manage work zones on all County maintenance and construction activities, as well as monitors work zone safety with agency field devices and vehicles. |
|  |  |  | Coordinate maintenance and construction activities with other regional maintenance and construction agencies. |
|  |  |  | Distribute maintenance and construction plans and work zone information to regional information service providers, regional traffic operations, emergency operations, rail operations, and the media. |
|  | NMDOT District 1 |  | Provide maintenance resources to assist regional emergency management agencies in incident response. |
|  |  |  | Coordinate maintenance resources for incidents with other regional maintenance providers. |
|  |  |  | Receive vehicle location information from agency maintenance and construction vehicles. |
|  |  |  | Receive vehicle maintenance conditions from agency maintenance and construction vehicles and coordinate fleet maintenance with agency equipment repair facility/garage. |
|  |  |  | Collect road weather information with agency field equipment and distribute it to regional traffic, maintenance and transit agencies as well as the national weather service and the media. |
|  |  |  | Provide road weather information to regional emergency management providers, regional traffic agencies, regional transit agencies, and regional public information services. |
|  |  |  | Provide automated roadway treatment on agency owned roads or bridges. |
|  |  |  | Provide winter road maintenance for all agency arterials and highways and provide roadway maintenance status information to regional public safety agencies, regional transit agencies, and the agency's traffic center. |
|  |  |  | Provide maintenance of state highways within the region, including pavement maintenance and all construction activities. |
|  |  |  | Dispatch agency maintenance vehicles. |
|  |  |  | Provide maintenance to all field equipment owned and operated by the agency. |
|  |  |  | Manage work zones on all agency maintenance and construction activities, as well as monitors work zone safety with agency field devices and vehicles. |
|  |  |  | Coordinate maintenance and construction activities with other regional maintenance and construction agencies. |
|  |  |  | Distribute maintenance and construction plans and work zone information to regional information service providers, regional traffic operations, emergency operations, rail operations, and the media. |
| Archived Data Management | City of Las Cruces Department of Public Works |  | Collect and archive traffic information from City traffic devices and from the City's TOC. |
|  |  |  | Coordinate with Las Cruces MPO and Dona Ana County traffic database and NMDOT's historical database. |
|  | Dona Ana County Public Works Division |  | Collect and archive traffic information from County traffic devices and from the County's TOC. |
|  |  |  | Coordinate with Las Cruces MPO's traffic database and NMDOT's historical database. |
|  | NMDOT District 1 |  | Coordinate with NMDOTs Statewide Crash Information System for regional and statewide crash records. |
|  |  |  | Collect and archive emergency safety and crash information from regional emergency management providers. |
|  |  |  | Collect and archive transit information about transit services within the region. |
|  |  |  | Coordinate with other regional traffic databases. |
|  |  |  | Collect and archive maintenance and construction information from the agency's maintenance garage and regional maintenance departments. |
|  | Las Cruces MPO |  | Collect and archive traffic information from regional traffic management providers for planning purposes. |
|  |  |  | Collect and archive traffic count information from the Las Cruces MPO's own field equipment. |
|  |  |  | Coordinate with other regional traffic databases. |
|  |  |  | Function as a data warehouse for emergency archive information, maintenance and construction information, and other traffic information. |

# Systems Inventory

The Las Cruces Regional ITS Architecture inventory is a list of “elements” that represent all existing and planned ITS systems in the region as well as non-ITS systems that provide information to or get information from the ITS systems.  These elements are owned, operated, or maintained by stakeholder agencies, companies, or groups.  The focus of the inventory is on those systems that support, or may support, interfaces that cross stakeholder boundaries (e.g.  public to private interfaces).

The vast majority of the inventory represents ITS systems within the Las Cruces Region, but the inventory does contain some elements that represent systems in adjoining regions or those that are statewide in nature.  An example of an element that is statewide in nature would be “Other NMDOT District TOCs”, which represents the existing or planned traffic operations centers in NMDOT Districts.  The significance of having “Other NMDOT District TOCs” in the Las Cruces Regional ITS Architecture is because it interfaces with the “NMDOT District 1 Traffic Operations Center”, which is located in Las Cruces, New Mexico.

Each element in the inventory is described by a name, the stakeholder, a description, general status (e.g.  existing or planned), and the associated subsystems or terminators from the National ITS Architecture that the elements is mapped to for modeling purposes.

## Systems by Stakeholder

Table 4 sorts the inventory by stakeholder so that each stakeholder can easily identify all the elements that are defined in the architecture.  For each element in the inventory, the table provides an element description and an indication of whether the element exists or is planned.  The list of elements sorted by stakeholder can also be found on the Las Cruces Regional Architecture web page by selecting the “Inventory by Stakeholder” button on the menu bar.

The majority of elements in the inventory represent a specific existing or planned system.  Examples of specific systems are the “City of Las Cruces Traffic Operations Center” or the “MVRDA Communications Center.”  Some of the elements represent sets of devices, rather than a single specific system or device.  An example of this type of element is the element “NMDOT CCTV”, and represents all of the CCTV that are, or will be, operated by NMDOT in the region.  The element describes the type of field device, not the specific number of devices.

A third type of element in the inventory is a “generic” element that represents all of the systems of a certain type in the region.  An example of this type of element is “Local Transit Dispatch” which not only represents all the transit dispatch centers not specifically called out in the architecture.  These generic elements have been created for two primary reasons.  First, they represent elements with similar types of interfaces.  So, from a standardization standpoint, describing how one of the major elements in the region (e.g.  the City of Las Cruces Traffic Operations Center) interfaces with various public transit dispatch functions would be the same.  Second, describing many systems with a single element helps keep the architecture from growing too large.

Table 4.  Inventory Sorted by Stakeholder

| Stakeholder Name | Element Name | Element Description | Status |
| --- | --- | --- | --- |
| Archive Data Users | Archive Data Users | Any user of archive data products from any Regional archive management system. This may include individual users, computer applications, or modeling systems utilizing the archived data. | Existing |
| City of Las Cruces | City of Las Cruces Animal Control Vehicles | Emergency vehicles dispatched for incidents involving live animals. | Existing |
| City of Las Cruces | City of Las Cruces Website | Transportation information website for the City of Las Cruces. In the future will include real-time construction, work zone, special event, incident, and traffic information. | Existing |
| City of Las Cruces | City of Las Cruces Public Information System | The office that provides the official interface between the City of Las Cruces traffic and maintenance departments and interests outside the departments such as the media. | Existing |
| City of Las Cruces | City of Las Cruces Convention and Visitors Bureau | The City of Las Cruces Department of Tourism responsible for attracting various travelers, conventions, etc. to the City of Las Cruces. | Existing |
| City of Las Cruces | City of Las Cruces Transportation Center | The multimodal transportation center in downtown Las Cruces. Connections for the RoadRUNNER bus service, RPO transit services and other transit services are available. | Existing |
| City of Las Cruces Department of Public Works | City of Las Cruces Public Works Dispatch | The maintenance and construction division for the City of Las Cruces. Operates as the dispatch function for the City of Las Cruces public works (maintenance, construction, snow plows, etc.) vehicles. | Existing |
| City of Las Cruces Department of Public Works | City of Las Cruces Traffic Archive | The archive system for the City of Las Cruces that collects traffic and traveler information as well as accident data for planning purposes. | Existing |
| City of Las Cruces Department of Public Works | City of Las Cruces Traffic Operations Center | The traffic operations center located in the City of Las Cruces that controls the traffic signal systems and other field equipment owned by the City of Las Cruces. | Existing |
| City of Las Cruces Department of Public Works | City of Las Cruces Traffic Signal Shop | Traffic signal shop that repairs and replaces traffic signals and other ITS field equipment for the City of Las Cruces. | Existing |
| City of Las Cruces Department of Public Works | City of Las Cruces RWIS | Road Weather Information System (RWIS) operated and maintained by the City of Las Cruces. | Planned |
| City of Las Cruces Department of Public Works | City of Las Cruces Traffic Signals | Traffic signal systems owned and operated by the City of Las Cruces. May include preemption capabilities, video detection, pedestrian signals, etc. | Existing |
| City of Las Cruces Department of Public Works | City of Las Cruces Maintenance and Construction Vehicles | Maintenance and construction vehicles owned and operated by the City of Las Cruces. Includes maintenance, construction, snow plows, etc. ITS technologies onboard vehicles may include AVL system, MDTs, etc. | Existing |
| City of Las Cruces Department of Public Works | City of Las Cruces ITS Field Equipment | Field equipment owned and operated by the City of Las Cruces that is not expressly identified in this architecture (e.g. VIVDS, Flood Gates, security monitoring field equipment, etc.). | Planned |
| City of Las Cruces Department of Public Works | City of Las Cruces Fleet Management Facility | The equipment repair facility for the City of Las Cruces. Handles maintenance and construction vehicles and equipment. | Existing |
| City of Las Cruces Department of Public Works | City of Las Cruces Field Sensors | Traffic and vehicle sensors owned and operated by the City of Las Cruces. | Existing |
| City of Las Cruces Department of Public Works | City of Las Cruces DMS | Dynamic message signs owned and operated by the City of Las Cruces. May be shared resources with the NMDOT and Homeland Security. | Existing |
| City of Las Cruces Department of Public Works | City of Las Cruces CCTV | Closed circuit television (CCTV) cameras owned and operated by the City of Las Cruces. | Existing |
| City of Las Cruces Department of Public Works | City of Las Cruces Asset Management System | The asset management system for the City of Las Cruces. | Planned |
| City of Las Cruces Department of Public Works | City of Las Cruces Storage Facility | The storage facility for maintenance and construction operations (e.g. gravel, dirt, extra vehicles, etc,) for the City of Las Cruces. | Existing |
| City of Las Cruces Fire Department | City of Las Cruces Fire and EMS Vehicles | Fire and EMS vehicles owned and operated by the City of Las Cruces ITS technologies on-board fire and EMS vehicles may include mobile data terminals (MDTs), AVL, signal preemption devices, etc. Also includes HAZMAT vehicles dispatched within the City limits. | Existing |
| City of Las Cruces Police Department | City of Las Cruces Police Vehicles | Police vehicles owned and operated by the City of Las Cruces. ITS technologies on-board cruisers may include mobile data terminals (MDTs), AVL, etc. | Existing |
| Contract Transport Companies | Private Ambulance Provider Vehicles | The private vehicles owned and operated by private ambulance providers within the Region. | Existing |
| Contract Transport Companies | Private Ambulance Provider Dispatch | This covers the private ambulance dispatch systems that serve the Region. | Existing |
| Dona Ana County | Dona Ana County/City of Las Cruces Emergency Operations Center (EOC) | The emergency operations center (EOC) for Dona Ana County including the City of Las Cruces. | Existing |
| Dona Ana County | Dona Ana County Asset Management System | The asset management system for the County of Dona Ana. | Planned |
| Dona Ana County | Dona Ana County Public Information System | The office that provides the official interface between Dona Ana County traffic and public works departments and interests outside the departments such as the media. | Existing |
| Dona Ana County | Dona Ana County Traffic Planning System | The archive system for Dona Ana County that uses traffic and traveler information as well as accident data for planning purposes. | Existing |
| Dona Ana County | Dona Ana County Website | Transportation information website for Dona Ana County. In the future will include real-time construction, work zone, special event, incident, and traffic information. | Existing |
| Dona Ana County | Dona Ana County Animal Control Vehicles | Emergency vehicles dispatched for incidents involving live animals. | Existing |
| Dona Ana County Fire Department | Dona Ana County Fire and EMS Vehicles | Fire and EMS vehicles owned and operated by Dona Ana County. Onboard ITS systems may include mobile data terminals (MDTs), AVL, cameras, etc. | Existing |
| Dona Ana County Flood Commission | Dona Ana County Flood Commission Water Level Monitoring Systems | This element represents the current Water Resource Information System (WRIS) and future water level monitoring systems that might be used to identify flooding that would affect the transportation system. | Existing |
| Dona Ana County Public Works Division | Dona Ana County Field Sensors | Traffic and vehicle sensors owned and operated by Dona Ana County. | Existing |
| Dona Ana County Public Works Division | Dona Ana County Fleet Management Facility | The equipment repair facility for Dona Ana County. Handles maintenance and construction vehicles and equipment. | Existing |
| Dona Ana County Public Works Division | Dona Ana County ITS Field Equipment | Field equipment owned and operated by Dona Ana County. Includes all other ITS field equipment that isn't specifically called out in the architecture (e.g. RWIS, Flood Gates, etc.). | Existing |
| Dona Ana County Public Works Division | Dona Ana County Traffic Signals | Traffic signal systems owned and operated by Dona Ana County. May include preemption capabilities, video detection, pedestrian signals, etc. | Existing |
| Dona Ana County Public Works Division | Dona Ana County DMS | Dynamic message signs owned and operated by Dona Ana County. Includes portable message signs for maintenance and construction activities or emergency conditions/warnings. | Existing |
| Dona Ana County Public Works Division | Dona Ana County Traffic Operations Center | The traffic operations center located for Dona Ana County that controls the traffic signal systems and other field equipment owned by Dona Ana County. | Existing |
| Dona Ana County Public Works Division | Dona Ana County CCTV | Closed circuit television (CCTV) cameras owned and operated by Dona Ana County. | Existing |
| Dona Ana County Public Works Division | Dona Ana County Public Works Dispatch | The maintenance and construction division for Dona Ana County. Operates as the dispatch function for Dona Ana County's maintenance and construction vehicles as well as snow plows. | Existing |
| Dona Ana County Public Works Division | Dona Ana County Storage Facility | The storage facility for maintenance and construction operations (e.g. gravel, dirt, extra vehicles, etc,) for Dona Ana County. | Existing |
| Dona Ana County Public Works Division | Dona Ana County Public Works Vehicles | Maintenance and construction vehicles owned and operated by Dona Ana County (includes snow plows). May include AVL and MDTs onboard vehicles. | Existing |
| Dona Ana County Sheriffs Office | Dona Ana County Sheriff Vehicles | Sheriff vehicles owned and operated by the Dona Ana County Sheriff's Department. Onboard ITS systems may include mobile data terminals (MDTs), AVL, cameras, etc. | Existing |
| Elephant Butte Irrigation District | EBID Water Level Monitoring Systems | This element represents the current Water Resource Information System (WRIS) and future water level monitoring systems that might be used to identify flooding that would affect the transportation system. | Existing |
| Financial Institution | Financial Institution | Represents the financial institutions the regional transit agencies will use as part of electronic fare payment systems. | Planned |
| Independent Public School Districts | Gadsden Ind/Hatch Valley Municipal School District Dispatch | Dispatch function for Gadsden Independent/Hatch Valley Municipal School District Public Schools. Includes radio communication with school buses. In the future it will include an automated dispatch function, CAD, etc. | Existing |
| Independent Public School Districts | Gadsden Ind/Hatch Valley Municipal School District Buses | The buses owned and operated by the Gadsden Independent/Hatch Valley Municipal Public Schools. ITS equipment installed on district buses may include AVL, MDTs and security monitoring equipment (CCTV, silent alarms, etc.) | Existing |
| Las Cruces MPO | Las Cruces MPO Data Warehouse | The data warehouse for the Las Cruces MPO. Allows virtual access points into other archives through one portal. | Existing |
| Las Cruces MPO | Las Cruces MPO Traffic Monitoring Program | The traffic count archive for the Las Cruces MPO. Collects traffic count information from its constituent cities as well as its own traffic count equipment. | Existing |
| Las Cruces Public School District | Las Cruces Public School District Dispatch | Dispatch function for Las Cruces Public Schools. Includes radio communication with school buses. In the future it will include an automated dispatch function, CAD, etc. | Existing |
| Las Cruces Public School District | Las Cruces Public School District Buses | The buses owned and operated by the Las Cruces Public Schools. ITS equipment installed on district buses may include AVL, MDTs and security monitoring equipment (CCTV, silent alarms, etc.) | Existing |
| Local Media | Local Print and Broadcast Media | Includes the local newspapers and the local TV and radio stations | Existing |
| Local Transit Agencies | Local Transit Website | The transit website for the local transit agencies operating within the Region. Websites may include vanpooling forms, information in finding a vanpool, and information regarding the board of directors, demand response transit plans, etc. | Existing |
| Local Transit Agencies | Regional Smart Card | Smart Card used for transit, parking, and other electronic systems. | Planned |
| Local Transit Agencies | Local Transit Dispatch | The dispatch function for local transit agencies are not specifically called out in the Architecture. May have an automated dispatch or CAD system in the future. | Existing |
| Local Transit Agencies | Local Transit Vehicles | Fixed route and demand response transit vehicles owned and operated by local transit agencies. The ITS applications on local transit vehicles may Include AVL, MDTs and security monitoring equipment (CCTV, silent alarms, etc.). | Existing |
| Mesilla Valley Regional Dispatch Authority | MVRDA Communications Center | The joint emergency dispatch system for the City of Las Cruces, Town of Mesilla and Dona Ana County. Includes dispatch functions for police, fire, and EMS. Also includes the public safety answering point for the City of Las Cruces. | Existing |
| Mesilla Valley Regional Dispatch Authority | MVRDA Reverse 911 System | Automated callback system to disseminate public information to a specified geographic area. | Existing |
| Municipal Public Safety | Municipal Fire and EMS Vehicles | Fire and EMS vehicles owned and operated by local fire districts that are not explicitly called out in the architecture. | Existing |
| Municipal Public Safety | Municipal Police Vehicles | Police vehicles that are owned and operated by the municipal police departments that are not explicitly called out in the architecture. | Existing |
| Municipalities | Municipal Emergency Operations Centers (EOC) | The emergency operations centers for the municipalities within the Region. Primary responsibility is to monitor local capabilities and request state support when needed. | Planned |
| New Mexico Department of Public Safety - DPS | New Mexico Statewide Emergency Operations Center (EOC) | The statewide emergency operations center located in Albuquerque. | Existing |
| New Mexico Department of Public Safety - DPS | NM DPS Alert Coordinator | The origination point for Amber Alerts and other wide area alerts throughout the State. | Existing |
| New Mexico Department of Public Safety - DPS | NM DPS Vehicles | Emergency vehicles that are owned and operated by the NM DPS (State Police). ITS applications installed on the police vehicles may include AVL, MDTs, maintenance scheduling equipment, etc. | Existing |
| New Mexico Department of Public Safety - DPS | NM DPS District 4 Dispatch Center | The district dispatch center for the NM DPS (State Police). May have an integrated CAD system. There are 12 districts, all of which have their own dispatch center (except D12). | Existing |
| New Mexico General Services Department | New Mexico State Radio Communications Bureau (RCB) | State communications dispatch for DPS, Fire/ Rescue statewide, NMDOT, Forest Service, EMS, and hospitals. Also known as Santa Fe Control. Own communications infrastructure in state | Existing |
| New Mexico National Guard | New Mexico National Guard Command System | The National Guard Command and Deployment system for the State of New Mexico. They would assist in emergency management, particularly as it relates to disasters or evacuations. | Existing |
| New Mexico State University | NMSU Events Office | The event office at the New Mexico State University that coordinates with regional traffic systems for large events (football games, etc.). | Existing |
| New Mexico State University | NMSU Emergency Operations Center (EOC) | The emergency operations center (EOC) for the New Mexico State University. | Existing |
| New Mexico State University | NMSU Police Vehicles | Police vehicles owned and operated by the New Mexico State University. Includes the ITS equipment installed on the cruisers (AVL, MDTs, etc.). | Existing |
| New Mexico State University | NMSU Traffic Operations Center | Traffic operations center for New Mexico State University. Controls all ITS field equipment owned and operated by NMSU, including CCTV cameras, traffic detection devices, and dynamic message signs. | Existing |
| New Mexico State University | NMSU Fire Vehicles | Fire vehicles owned and operated by New Mexico State University. Onboard ITS systems may include mobile data terminals (MDTs), AVL, cameras, etc. | Existing |
| New Mexico State University | NMSU ITS Field Equipment | This element represents the ITS field equipment (roadside) owned and operated by New Mexico State University. | Existing |
| New Mexico State University | NMSU Emergency Dispatch Center | 911 dispatch for the New Mexico State University Police Department, fire EMS, and public utilities. | Existing |
| New Mexico State University | Aggie Shuttle Vehicles | Fixed route transit vehicles owned and operated by the Aggie Shuttle. ITS technologies on vehicles may include AVL, MDTs and security monitoring equipment (CCTV, silent alarms, etc.). | Existing |
| New Mexico State University | NMSU Parking System | Parking system owned and operated by New Mexico State University. In the future, systems may include gates, barriers, parking fare equipment, security equipment, etc. | Existing |
| New Mexico State University | NMSU Equipment Repair Facility | The equipment repair facility for New Mexico State University. Handles maintenance and construction vehicles and equipment. | Existing |
| New Mexico State University | NMSU Maintenance and Construction Vehicles | Maintenance and construction vehicles owned and operated by New Mexico State University. May include AVL and MDTs onboard vehicles. | Existing |
| New Mexico State University | Aggie Shuttle Dispatch | The dispatch function for the Aggie Shuttle. May include a CAD system. | Existing |
| New Mexico State University | NMSU Office of Facilities Operation and Utilities | Responsible for maintaining the buildings and infrastructure for New Mexico State University. | Existing |
| New Mexico State University | NMSU Website | The website for the New Mexico State University. The website may include traveler information such as transit schedules (Aggie Shuttle), parking information, special events, and traffic information on campus. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Automatic Traffic Recording Devices | Traffic recording devices that are owned and operated by NMDOT. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Field Sensors | Traffic and vehicle sensors owned and operated by NMDOT. Includes vehicle detection systems (video, microwave, acoustic, etc) and traffic counting systems. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT DMS | Dynamic message signs owned and operated by NMDOT. | Existing |
| NMDOT - New Mexico Department of Transportation | NM Motor Transport Division (MTD) District Offices | The MTD district offices which dispatch commercial vehicle enforcement personnel | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT CCTV | Closed Circuit Television Cameras (CCTV) owned and operated by NMDOT. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Advanced Traveler Information System | The office that provides the official interface between the NMDOT traffic and maintenance departments and interests outside the departments such as the media. Currently this is performed by CARS- 511, but it may change in the future. | Existing |
| NMDOT - New Mexico Department of Transportation | NM Motor Transport Division (MTD) Response Vehicles | The MTD response vehicles within the NM DPS that handle commercial vehicle incidents, enforcements, etc. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Traffic Safety Division Data System | Regional database that collects traffic safety and incident information. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Consolidated Highway Database (CHDB) | An automated database that is owned and operated by NMDOT that contains traffic information for state highways. To be replaced by the TIMS system in the future. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT RWIS | Road weather information systems that are owned and operated by NMDOT. | Planned |
| NMDOT - New Mexico Department of Transportation | NMDOT Park and Ride Facilities | Park and ride facilities that are owned and operated by NMDOT. Systems at the park and ride facility may include gates, barriers, parking fare equipment, etc. Also includes security equipment to monitor the parking facilities and the shelters or kiosks at park and ride locations. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Park and Ride Dispatch | The dispatch function for the NMDOT Park and Ride shuttles. May have a CAD system in the future. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Patrol Yard | Storage facility/yard owned, operated, and used by the New Mexico Department of Transportation. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Rest Area/ Visitor Center/ Truck Stop/ Service Plaza Kiosks | This represents the rest areas and visitor centers with varied amenities that are existing or planned by NMDOT. Tourist information, emergency evacuation information, and general traffic information could be provided in the future. | Planned |
| NMDOT - New Mexico Department of Transportation | NMDOT Other District TOCs | Traffic Management Centers in other NMDOT Districts. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Lane Controls | Lane control devices owned and operated by NMDOT. These include lane control signals on bridges. | Planned |
| NMDOT - New Mexico Department of Transportation | NMDOT Road Closure Gates | This element represents automated road closure gates owned and operated by NMDOT used for the remote closure of roads. Intended to be for areas where ice or snow or other adverse weather conditions exist on a frequent basis and that cause hazardous conditions for motorists (e.g. canyon roads, etc.). | Planned |
| NMDOT - New Mexico Department of Transportation | NMDOT Historical Traffic Database | The archive data management system for traffic operations and traffic information for the NMDOT Districts. | Planned |
| NMDOT - New Mexico Department of Transportation | NMDOT Work Zone Equipment | Work zone monitoring and alerting equipment owned by NMDOT. | Planned |
| NMDOT - New Mexico Department of Transportation | NMDOT Roadway Treatment Devices | Roadway treatment devices (anti-icing equipment, sensors, etc.) owned and operated by NMDOT to treat roads (usually ice, snow, etc.). | Planned |
| NMDOT - New Mexico Department of Transportation | Other NMDOT District Maintenance Dispatch | NMDOT Maintenance dispatch function in neighboring districts. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Signal Lab | The NMDOT Signal Lab manages traffic signal systems statewide that are owned and operated by NMDOT. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Statewide Crash Information System | Statewide database of vehicle crash records. Input to system provided by elements within the architecture. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Statewide Pavement Management System | Statewide database for pavement management. Input into system provided by elements in the Region. Aids in planning for the life of pavement and funding requirements for resurfacing (based on historical evidence). | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Statewide TMC | NMDOT Transportation Management Center located in Albuquerque. Currently controls the ITS field equipment for District 3. In the future may have the ability to control other District field equipment (but primarily serves as a backup system or operates when the District TOCs are offline - after hours). | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Highway Advisory Radio System (HAR) | Highway Advisory Radio owned and operated by NMDOT. Provides traffic and incident information to the traveling public. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Traffic Data Collection System | The traffic data collection system (sensors, etc.) that are owned and operated by NMDOT. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Traffic Signals | Traffic signal systems owned and operated by NMDOT. | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Park and Ride Shuttle Buses | The shuttles that are owned and operated by the NMDOT Park and Ride services. In the future these shuttles may have CAD systems, MDTs and security monitoring equipment (CCTV, silent alarms, etc.) | Existing |
| NMDOT - New Mexico Department of Transportation | NMDOT Highway Maintenance Management System (HMMS) | The maintenance archive management system for NMDOT. Specifically, this system aids NMDOT in determining appropriate maintenance activities as well as archiving various maintenance activities (for date, location, type of activity, labor, equipment used, materials used, etc.). | Existing |
| NMDOT District 1 | NMDOT District 1 TOC | The traffic operations center for the NMDOT Las Cruces District 1. In the future they will control all NMDOT ITS field equipment and coordinates with all other agencies. | Planned |
| NMDOT District 1 | NMDOT District 1 Security Monitoring Field Equipment | Security equipment owned and operated by NMDOT. Generally used for monitoring bridges, roads, etc. for homeland security measures. | Planned |
| NMDOT District 1 | NMDOT District 1 Public Transportation Management System (PTMS) | The public transportation management archive for NMDOT District 1. | Existing |
| NMDOT District 1 | NMDOT District 1 Equipment Repair Facility | The maintenance shop for all roadside equipment owned and operated by NMDOT. Also provides maintenance to NMDOT vehicles. | Existing |
| NMDOT District 1 | NMDOT District 1 Public Information System | The office that provides the official interface between NMDOT District 1 traffic and maintenance departments and interests outside the departments such as the media. | Existing |
| NMDOT District 1 | NMDOT District 1 Pavement Management System | District Pavement Management System to assist in storing, retrieving, analyzing and reporting information to help with pavement-related decision-making processes. | Existing |
| NMDOT District 1 | NMDOT District 1 Maintenance Units Dispatch | NMDOT maintenance sections for the Las Cruces Region. Dispatches maintenance vehicles and equipment for maintaining road and ITS equipment owned by NMDOT. | Existing |
| NMDOT District 1 | NMDOT District 1 Maintenance Office | Office in charge of maintenance and construction operations for the NMDOT Las Cruces District. | Existing |
| NMDOT District 1 | NMDOT District 1 Maintenance and Construction Vehicles | Maintenance and construction vehicles for the NMDOT Las Cruces District. | Existing |
| NMDOT District 1 | NMDOT District 1 ITS Field Equipment | This element represents any or all ITS field equipment (roadside) that is not specifically called out in the architecture. | Existing |
| NMDOT District 1 | NMDOT District 1 HELP Courtesy Patrol Vehicles | Roadway service patrol vehicles operated by NMDOT. May include mobile data terminals, AVL, etc. The trucks exist currently, and some are equipped with GPS. | Existing |
| NMDOT District 1 | NMDOT District 1 HELP Courtesy Patrol Dispatch | The dispatch function for the NMDOT HELP vehicles that provide courtesy patrols on the NMDOT owned and operated roadways. | Existing |
| NMDOT District 1 | NMDOT District 1 Website | Transportation information website for the NMDOT. Currently has real-time construction, work zone, special event, incident, and traffic information. | Existing |
| NOAA | National Weather Service | Service for national, regional, and local weather information. | Existing |
| Other Counties | Other Municipal/County Public Safety Communications Centers | This element represents the connection to other Municipal or County emergency systems, including County (Sheriff and Fire), and Local (municipal police and fire departments) law enforcement, fire, and EMS call taker and dispatch centers. | Existing |
| Other Counties | Other County TOC | Traffic operations centers in other counties that may coordinate with county traffic operations centers called out in the architecture. | Planned |
| Private HAZMAT Providers | Private HAZMAT Vehicles | Private HAZMAT vehicles that operate as second responders to incidents. | Existing |
| Private HAZMAT Providers | Private HAZMAT Response Providers | Second responders to incidents, as determined by the first responder to an incident. Responsible for regional HAZMAT mitigation and clean-up activities. | Existing |
| Private Sector Traveler Information Service Providers | Private Sector Traveler Information Services | Private traveler information providers serving the region. This element could, in the future, provide support to the National Traveler Information 511 number since it collects information from a broad array of operating centers. Could also include a website. | Existing |
| Private Tow/Wrecker Providers | Private Tow/Wrecker Vehicles | This element represents possible ITS equipment inside tow or wrecker vehicles. | Existing |
| Private Tow/Wrecker Providers | Private Tow/Wrecker Dispatch | Dispatch function for privately owned tow or wrecker service. Based on a rotation list. | Existing |
| Private Weather Information Provider | Private Weather Information Provider | Private weather services that provide regional and local weather information. | Existing |
| Public/Private Utilities | Public/Private Utilities | Public or private utility companies that operate within the Region. | Existing |
| Rail Operators | Rail Operators Wayside Equipment | The rail operated equipment at highway rail intersections. Interconnect with the region's traffic control departments. | Existing |
| Rail Operators | Rail Operations Centers | The dispatch centers for major railroads in the region (e.g. BNSF, Amtrak, etc.). | Existing |
| Regional Airports | Regional Airports | Regional Airports that are owned and operated by the local municipalities where they reside. | Existing |
| Regional Event Coordinators | Special Event Coordinators | Promoters and sponsors of special events. They coordinate with traffic and emergency providers. | Existing |
| Regional Medical Centers | Regional Medical Centers | Medical centers that are located within the region (hospitals). | Existing |
| Regional Medical Centers | Southwest MedEvac | Represents the private air evacuation/rescue service that provides transport from remote locations to area hospitals. | Existing |
| Regional Public Safety Agencies | Regional Emergency Communications Network | Regional public safety communications network. | Planned |
| Regional Transit Providers | Regional Transit Kiosks | Kiosks located throughout the region (at visitor centers, etc) that dispense regional fare cards and provide transit fare and schedule information to transit users. | Planned |
| RoadRUNNER Transit | RoadRUNNER Fixed Route Dispatch System | Computerized dispatch and system for RoadRUNNER's fixed route transit service, which services the Las Cruces Region. | Existing |
| RoadRUNNER Transit | RoadRUNNER Paratransit Vehicles | Paratransit or demand response vehicles owned and operated by RoadRUNNER. ITS systems onboard the vehicles may include AVL, MDTs (for real-time dispatching) and security monitoring equipment (CCTV, silent alarms, etc..) | Existing |
| RoadRUNNER Transit | RoadRUNNER Paratransit Dispatch Center | The dispatch function for all RoadRUNNER paratransit or demand response transit vehicles. May include a CAD system. | Existing |
| RoadRUNNER Transit | RoadRUNNER Transit Website | The transit website for RoadRUNNER. Website has schedule and fare information, bus pass purchase locations, trip planning, etc. | Existing |
| RoadRUNNER Transit | RoadRUNNER Transit Payment Card | The payment card used specifically for RoadRUNNER transit rides. May be obtained at the RoadRUNNER transit kiosks or transit centers. | Planned |
| RoadRUNNER Transit | RoadRUNNER Transit Kiosks | Transit kiosks owned and operated by RoadRUNNER. May include an opportunity to purchase a transit card, request trip information, or display notices on LED displays. | Planned |
| RoadRUNNER Transit | RoadRUNNER Fixed Route Transit Vehicles | Fixed route transit vehicles owned and operated by RoadRUNNER. ITS technologies on vehicles may include AVL, MDTs and security monitoring equipment (CCTV, silent alarms, etc..). | Existing |
| RoadRUNNER Transit | RoadRUNNER Transit Information Center | This element represents transit information centers or kiosks located around the City of Las Cruces, but provided by RoadRUNNER. | Planned |
| South Central Regional Transit District | South Central RTD Transit Website | The transit website for the South Central Regional Transit District. Information may include vanpooling forms, information in finding a vanpool, and information regarding the board of directors, demand response transit plans, etc. | Planned |
| South Central Regional Transit District | South Central RTD Transit Vehicles | Demand response transit vehicles owned and operated by the South Central RTD. They serve the rural areas in South Central New Mexico. | Planned |
| South Central Regional Transit District | South Central RTD Operations Center | This represents transit operations for those systems that will be a part of the South Central RTD. | Existing |
| Southwest Regional Transit District | Southwest RTD Operations Center | This represents transit operations for those systems that will be a part of the Southwest RTD. | Existing |
| Texas Department of Public Safety | Texas DPS Communications Service | This element represents the Texas Highway Patrol dispatch functions. They dispatch vehicles using two-way radio communication. They are also responsible for regional evacuation. | Existing |
| Travelers | Private Travelers Personal Computing Devices | Includes personal and office computers, pagers, and handheld devices used by travelers to receive ITS information. | Existing |
| Travelers | Private Vehicles | Vehicles owned by travelers. | Existing |
| TxDOT | TransVista | Traffic Management Center for the El Paso Region in El Paso, Texas. Systems include DMS, CCTV, Lane Controls, and Courtesy Patrol dispatch). Manages freeways and in the future will control arterials on state routes. | Existing |
| U.S. Department of Defense | Military Installations Operations Office | This element represents the operations offices for military installations throughout the state (e.g. Kirtland Air Force Base). | Planned |
| U.S. Department of Defense | White Sands Missile Range Facility | This element represents the operations (both traffic and emergency management) function at the White Sands Missile Range Facility. | Existing |
| US Customs and Border Protection | US Border Patrol Stations | Border patrol stations at border crossings. Border patrol stations may have ITS capabilities such as computer aided dispatch and video monitoring. | Existing |

## Systems by Architecture Entity

Each element in the Las Cruces Regional ITS Architecture inventory is mapped to one or more entities from the National ITS Architecture.  In version 5.1 of the National ITS Architecture (on which this architecture is based) there are 95 entities defined.  These 22 subsystems and 73 terminators describe a wide array of systems that provide ITS services, or interface with systems that provide ITS services.  The mapping of Las Cruces Regional ITS Architecture elements to National ITS Architecture entities has two primary benefits.  First, it allows the full set of information flows contained in the National ITS Architecture to be used in the description of Las Cruces Regional ITS Architecture interfaces.  Secondly, it allows the elements of the Las Cruces Regional ITS Architecture to be grouped by like entity.

A list of the elements in the Las Cruces Regional ITS Architecture sorted by entity can be found on the Las Cruces Regional Architecture web page by selecting the “Inventory by Entity” button on the left side menu bar.  This list on the web page allows the users of the architecture to immediately identify all the elements that have functions relating to transit management, or traffic management, or any other subsystem or terminator defined by the National ITS Architecture.

The Las Cruces Regional ITS Architecture inventory contains the following number of elements mapped to different types of entities:

- Archived Data Management: 11

- Emergency Management: 31 (including some transit elements that are mapped to emergency management because of the transit security service described in a later section)

- Information Service Providers: 15

- Maintenance and Construction Management: 11

- Traffic Management: 9

- Transit Management: 9

# Needs and Services

## Needs Identification

Transportation needs identify the transportation problems that can be solved by ITS services.  They also represent a link to transportation planning efforts that define the strategies used to address transportation problems.  These strategies involve capital improvements as well as operational improvements.  ITS solutions usually involve services that improve the efficiency, scope, or safety of operations.

The initial set of user needs described below was developed from information collected as part of the New Mexico Statewide ITS Architecture development effort and customized based upon the Las Cruces stakeholder meetings.  Table 5 below, shows an “X” where it was determined the regional priority was for the user need.  One user need prioritization shows a specific reference to public services (“fire/EMS and police).  This is done to show the prioritization difference between different public safety agencies.  .

Table 5.  Summary of Transportation Needs/Priorities

| Need Area | Specific ITS Need | High | Medium | Low | Not a Need |
| --- | --- | --- | --- | --- | --- |
| Incident Management | Need improved incident detection, management and coordination | X |  |  |  |
|  | Need to reduce delays due to accidents or construction | X |  |  |  |
|  | Need to identify alternate routes for the traveling public |  |  |  |  |
|  | Improve HAZARDOUS cargo tracking and routing |  |  |  |  |
| Traffic Management | Need to improve traffic congestion mitigation. | X |  |  |  |
|  | Need to improve traffic mitigation on the east-west corridors | X |  |  |  |
|  | Need to provide early warning of poor visibility conditions (dust, thunderstorms, etc.) | X | X |  |  |
|  | Need to improve traffic signal interconnect and coordination to improve mobility | X |  |  |  |
|  | Need traffic signal preemption for emergency vehicles (needs to be expanded) | Fire/EMS |  |  | Police |
|  | Need advanced warning of flash flood areas |  | X |  |  |
|  | Need to enhance communications and information sharing between regional agencies |  | X |  |  |
|  | Need to interconnect with neighboring regions (such as El Paso) |  | X |  |  |
|  | Need to know travel times on major routes |  |  | X |  |
|  | Need to know delays on major routes |  | X |  |  |
|  | Need to implement parking management systems (NMSU or downtown) |  |  | X |  |
|  | Need to alert drivers of speeding (automated alert systems) |  |  | X |  |
|  | Need for remote monitoring for infrastructure and at intersections | X |  |  |  |
| Emergency Management | Need to improve emergency notification/dispatch and response times | X |  |  |  |
|  | Need to improve traffic safety | X |  |  |  |
|  | Need to expand remote traveler support services (information plus roadside assistance) |  | X |  |  |
|  | Need improved tracking of emergency vehicles | X |  |  |  |
|  | Need to identify alternate routes for emergency vehicles | X |  |  |  |
|  | Need to improve evacuation planning | X |  |  |  |
| Public Transportation | Need to improve/enhance rural traveler service |  | X |  |  |
|  | Need to improve urban traveler service | X |  |  |  |
|  | Need to improve transit coordination among city/county/tribal governments | X |  |  |  |
|  | Need better communication with transit customers | X |  |  |  |
|  | Need to encourage major employers to implement transit use incentives. |  | X | X |  |
|  | Need automated maintenance system for transit fleets |  | X |  |  |
|  | Need to improve efficiency of demand-responsive transit (enhanced information) |  | X |  |  |
|  | Need to improve schedule operations for fixed-route transit vehicles | X |  |  |  |
|  | Need interactive ITS services (transit-related) |  | X |  |  |
|  | Need to deploy AVL on regional transit vehicles |  | X |  |  |
| Maintenance Operations | Need to improve vehicle routing and detours/information |  | X |  |  |
|  | Need advanced and up-to-date road closure and construction zone information |  | X |  |  |
|  | Need to reduce delays due to accidents or construction | X |  |  |  |
|  | Need to know location of maintenance vehicles |  |  | X |  |
|  | Need to improve work zone safety (alert drivers of wrong-way movements) | X |  |  |  |
|  | Need improved wind/dust monitoring |  | X |  |  |
|  | Need improved flood monitoring | X |  |  |  |
| Traveler Information | Need real-time roadway and traffic conditions information | X |  |  |  |
|  | Need real-time information about weather conditions/location | X |  |  |  |
|  | Need easier access to traveler services information (locations, types of services, etc.) |  |  | X |  |
|  | Need special event traffic information |  | X |  |  |
|  | Need to have traveler information accessible at employment sites |  |  | X |  |
| Information Management | Need to collect transportation information for use by planners |  | X |  |  |
|  | Need park-and-ride information |  |  | X |  |

For simplicity, the needs summary below identifies high priority needs and maps these to stakeholder groups.  The seven groups whose needs were considered are:

- NMDOT District 1

- Public Safety (Police, Fire, EMS, Emergency Management)

- Municipal (primarily traffic and public works) - City of Las Cruces

- County (primarily traffic and public works) – Dora Ana County

- Transit - RoadRUNNER

- Planning (MPO/RPO) - Las Cruces MPO

Table 6 provides a summary of the high priority transportation needs along with the group(s) for which these needs are high priority.

Table 6.  Summary of High Priority Transportation Needs

| Need Area | Specific ITS Need | Group |
| --- | --- | --- |
| Incident Management | Need improved incident detection, management and coordination | NMDOT, Public Safety, Municipal, County |
|  | Need to reduce delays due to accidents or construction | NMDOT, Public Safety, Municipal, County |
| Traffic Management | Need to improve traffic congestion mitigation | NMDOT, Municipal, County |
|  | Need to improve traffic mitigation on the east-west corridors | NMDOT, Municipal, County |
|  | Need to provide early warning of poor visibility conditions (dust, thunderstorms, etc.) | NMDOT, Public Safety, Municipal, County |
|  | Need to improve traffic signal interconnect and coordination to improve mobility | NMDOT, Municipal |
|  | Need traffic signal preemption for emergency vehicles | Fire/EMS |
| Emergency Management | Need to improve emergency notification/dispatch and response times | Public Safety |
|  | Need to improve traffic safety | NMDOT, Public Safety, Municipal, County |
|  | Need improved tracking of emergency vehicles | Public Safety |
|  | Need to identify alternate routes for emergency vehicles | Public Safety |
|  | Need to improve evacuation planning | Public Safety |
| Public Transportation | Need to improve urban traveler service | Transit |
|  | Need to improve transit coordination among city/county/tribal governments | Transit |
|  | Need better communication with transit customers | Transit |
|  | Need to improve schedule operations for fixed-route transit vehicles | Transit |
| Maintenance Operations | Need to reduce delays due to accidents or construction | NMDOT, Municipal, County |
|  | Need to improve work zone safety (alert drivers of wrong-way movements) | NMDOT, Municipal, County |
|  | Need improved flood monitoring | NMDOT, County |
| Traveler Information | Need real-time roadway and traffic conditions information | NMDOT, Municipal, County |
|  | Need real-time information about weather conditions/location | NMDOT, Municipal, County |

## Services

The ITS systems in the region currently provide a wide array of transportation services and that list will grow as more systems are developed or upgraded.  The current and planned services can be described by the set of market packages that are shown in Table 7.  This set of services is a subset of the services contained in the National ITS Architecture, and represent all of the selected services (market packages) based on information gathered at stakeholder meetings, needs assessments, and review of planning documents.  Each of the market packages is currently implemented, or planned for implementation by one or more stakeholders in the region.  For those services where one or more stakeholders have already implemented it, the status is set to existing.

Table 7.  Selected Regional Services (Market Packages)

| Market Package | Market Package Name | Status |
| --- | --- | --- |
| AD1 | ITS Data Mart | Planned |
| AD2 | ITS Data Warehouse | Planned |
| APTS1 | Transit Vehicle Tracking | Planned |
| APTS2 | Transit Fixed-Route Operations | Planned |
| APTS3 | Demand Response Transit Operations | Planned |
| APTS4 | Transit Passenger and Fare Management | Planned |
| APTS5 | Transit Security | Planned |
| APTS6 | Transit Maintenance | Planned |
| APTS7 | Multi-modal Coordination | Planned |
| APTS8 | Transit Traveler Information | Planned |
| ATIS1 | Broadcast Traveler Information | Planned |
| ATIS2 | Interactive Traveler Information | Planned |
| ATMS01 | Network Surveillance (Monitoring) | Existing |
| ATMS02 | Probe Surveillance | Planned |
| ATMS03 | Surface Street Control | Existing |
| ATMS04 | Freeway Control | Planned |
| ATMS05 | HOV Lane Management | Planned |
| ATMS06 | Traffic Information Dissemination | Existing |
| ATMS07 | Regional Traffic Control | Planned |
| ATMS08 | Traffic Incident Management System | Planned |
| ATMS13 | Standard Railroad Grade Crossing | Planned |
| ATMS15 | Railroad Operations Coordination | Planned |
| ATMS16 | Parking Facility Management | Planned |
| ATMS18 | Reversible Lane Management | Planned |
| EM01 | Emergency Call-Taking and Dispatch | Planned |
| EM02 | Emergency Routing | Existing |
| EM04 | Roadway Service Patrols | Planned |
| EM05 | Transportation Infrastructure Protection | Planned |
| EM06 | Wide-Area Alert | Planned |
| EM08 | Disaster Response and Recovery | Planned |
| EM09 | Evacuation and Reentry Management | Planned |
| EM10 | Disaster Traveler Information | Planned |
| MC01 | Maintenance and Construction Vehicle and Equipment Tracking | Planned |
| MC02 | Maintenance and Construction Vehicle Maintenance | Planned |
| MC03 | Road Weather Data Collection | Existing |
| MC04 | Weather Information Processing and Distribution | Planned |
| MC05 | Roadway Automated Treatment | Planned |
| MC06 | Winter Maintenance | Planned |
| MC07 | Roadway Maintenance and Construction | Planned |
| MC08 | Work Zone Management | Planned |
| MC09 | Work Zone Safety Monitoring | Planned |
| MC10 | Maintenance and Construction Activity Coordination | Planned |

Incident Management, identified as ATMS08 in the above table, is one of the key services that is planned throughout the region.  Although it is technically called “Traffic Incident Management”, and identified numerically in the National ITS Architecture as ATMS08, a broader view of this service includes several market packages, including:

  - ATMS03 – Surface Street Control

  - ATMS04 – Freeway Control

  - ATMS06 – Information Dissemination

  - ATMS07 – Regional Traffic Control

  - ATMS08 – Traffic Incident Management

  - EM1 – Emergency Call-Taking and Dispatch

  - EM2 – Emergency Routing

As indicated by Table 7 above, all of these services are identified as existing or planned for the Las Cruces Regional ITS Architecture.

## Comparison of Needs and Services

The priority needs identified in Section 6.1 will be addressed by the ITS services described in Section 6.2.  Table 8 provides a mapping of the priority needs to ITS services identified in the previous section.

Table 8.  Mapping of Needs to Services

| Need Area | Specific ITS Need | ITS Service Addressing Need | Market Package |
| --- | --- | --- | --- |
| Incident Management | Need improved incident management and coordination | Traffic Incident Management | ATMS08 |
|  |  | Emergency Response | EM1 |
|  |  | Emergency Routing | EM2 |
|  | Need to reduce delays due to accidents or construction | Traffic Incident Management | ATMS08 |
|  |  | Roadway Maintenance and Construction | MC07 |
|  |  | Work Zone Management | MC08 |
| Traffic Management | Need to improve traffic congestion mitigation | Network Surveillance | ATMS01 |
|  |  | Surface Street Control | ATMS03 |
|  |  | Freeway Control | ATMS04 |
|  |  | Traffic Information Dissemination | ATMS06 |
|  |  | Regional Traffic Control | ATMS07 |
|  |  | Traffic Incident Management | ATMS08 |
|  | Need to improve traffic mitigation on the east-west corridors | Network Surveillance | ATMS01 |
|  |  | Surface Street Control | ATMS03 |
|  |  | Traffic Information Dissemination | ATMS06 |
|  |  | Regional Traffic Control | ATMS07 |
|  | Need to provide early warning of poor visibility conditions (dust, snow, etc.) | Traffic Information Dissemination | ATMS06 |
|  |  | Weather Information Processing and Distribution | MC04 |
|  | Need traffic signal interconnect and coordination to improve mobility | Surface Street Control | ATMS03 |
|  |  | Regional Traffic Control | ATMS07 |
|  | Need traffic signal preemption for emergency vehicles. | Emergency Routing | EM02 |
|  | Need advanced warning of flash flood areas | Traffic Information Dissemination | ATMS06 |
|  |  | Early Warning System | EM07 |
|  | Need to enhance communications and information sharing between regional agencies | Traffic Information Dissemination | ATMS06 |
|  |  | Traffic Incident Management | ATMS08 |
|  |  | Emergency Response | EM01 |
|  |  | Emergency Routing | EM02 |
|  | Need to interconnect with neighboring regions (such as El Paso) | Regional Traffic Control | ATMS07 |
|  | Need to know delays on major routes | Network Surveillance | ATMS01 |
|  | Need to know travel times on major routes | Network Surveillance | ATMS01 |
|  | Need to implement parking management systems | Parking Facility Management | ATMS16 |
|  | Need to alert drivers of speeding (automated alert systems) | Speed Monitoring | ATMS19 |
| Emergency Management | Need to improve emergency notification/dispatch and response times | Traffic Incident Management | ATMS08 |
|  |  | Emergency Response | EM01 |
|  |  | Emergency Routing | EM02 |
|  | Need to improve traffic safety | Network Surveillance | ATMS01 |
|  |  | Surface Street Control | ATMS03 |
|  |  | Freeway Control | ATMS04 |
|  |  | Traffic Information Dissemination | ATMS06 |
|  |  | Traffic Incident Management | ATMS08 |
|  | Need to expand remote traveler support services (information plus roadside assistance) | Roadway Service Patrols | EM04 |
|  |  | Interactive Traveler Information | ATIS02 |
|  | Need improved tracking of emergency vehicles | Emergency Routing | EM02 |
|  | Need to identify alternate routes for emergency vehicles | Emergency Routing | EM02 |
|  | Need to improve evacuation planning | Evacuation and Reentry Management | EM09 |
| Public Transportation | Need to improve/enhance rural traveler service (inter-city) | Transit Fixed-Route Operations | APTS02 |
|  |  | Demand Response Transit Operations | APTS03 |
|  | Need to improve urban traveler service | Transit Fixed-Route Operations | APTS02 |
|  |  | Demand Response Transit Operations | APTS03 |
|  | Need to improve transit coordination among city/county/tribal governments | Multi-modal Coordination | APTS07 |
|  | Need better communication with transit customers | Transit Passenger and Fare Management | APTS04 |
|  |  | Transit Traveler Information | APTS08 |
|  | Need to encourage major employers to implement transit use incentives | N/A | N/A |
|  | Need automated maintenance system for transit fleets | Transit Maintenance | APTS06 |
|  | Need to improve efficiency of demand-responsive transit (enhanced information) | Demand Response Transit Operations | APTS03 |
|  |  | Transit Traveler Information | APTS08 |
|  | Need to improve schedule operations for fixed-route transit vehicles | Transit Fixed-Route Operations | APTS02 |
|  | Need interactive ITS services (transit-related) | Transit Passenger and Fare Management | APTS04 |
|  |  | Transit Traveler Information | APTS08 |
|  | Need to deploy AVL on regional transit vehicles | Transit Vehicle Tracking | APTS01 |
| Maintenance Operations | Need to improve vehicle routing and detours/information | Surface Street Control | ATMS03 |
|  |  | Traffic Information Dissemination | ATMS06 |
|  |  | Roadway Maintenance and Construction | MC07 |
|  |  | Work Zone Management | MC08 |
|  | Need advanced and up-to-date road closure and construction zone information | Traffic Information Dissemination | ATMS06 |
|  |  | Roadway Closure Management | ATMS21 |
|  |  | Winter Maintenance | MC06 |
|  |  | Roadway Maintenance and Construction | MC07 |
|  | Need to reduce delays due to accidents or construction | Traffic Incident Management | ATMS08 |
|  |  | Roadway Maintenance and Construction | MC07 |
|  |  | Work Zone Management | MC08 |
|  | Need to improve work zone safety (alert drivers of wrong-way movements) | Work Zone Safety Monitoring | MC09 |
|  | Need improved wind monitoring | Road Weather Data Collection | MC03 |
|  | Need improved dust monitoring | Road Weather Data Collection | MC03 |
|  | Need to know location of maintenance vehicles | Maintenance and Construction Vehicle and Equipment Tracking | MC01 |
| Traveler Information | Need real-time roadway and traffic conditions information | Network Surveillance | ATMS01 |
|  |  | Traffic Information Dissemination | ATMS06 |
|  |  | Broadcast Traveler Information | ATIS1 |
|  | Need real-time information about weather conditions/location | Traffic Information Dissemination | ATMS06 |
|  |  | Road Weather Data Collection | MC03 |
|  |  | Weather Information Processing and Distribution | MC04 |
|  | Need easier access to traveler services information (locations, types of services, etc.) | Broadcast Traveler Information | ATIS1 |
|  | Need special event traffic information | Network Surveillance | ATMS01 |
|  |  | Traffic Information Dissemination | ATMS06 |
|  |  | Traffic Incident Management | ATMS08 |
|  |  | Broadcast Traveler Information | ATIS1 |
|  | Need to have traveler information accessible at employment sites | Broadcast Traveler Information | ATIS1 |
| Information Management | Need to collect transportation information for use by planners | ITS Data Mart | AD1 |
|  |  | ITS Data Warehouse | AD2 |
|  | Need park-and-ride information | Parking Facility Management | ATMS16 |
|  |  | Transit Fixed-Route Operations | APTS02 |
|  |  | Transit Traveler Information | APTS08 |

# Interfaces and Information Exchanges

## Top Level Regional System Interconnect Diagram

A system interconnect diagram, or sausage diagram, shows the systems and primary types of interconnections in the region.  The National ITS Architecture interconnect diagram has been customized for the Las Cruces Region based on the information gathered from the stakeholders and system inventory.  Figure 6 on the following page summarize the existing and planned ITS elements for the region in the context of a physical interconnect.  Elements (and their primary associated National ITS Architecture entity) are called out in the boxes surrounding the main interconnect diagram.  In the center of the figure, the rectangles represent the subsystems of the National ITS Architecture.  The Las Cruces Regional ITS Architecture has elements that map to 17 of the 22 possible subsystems of the National ITS Architecture.  In addition, the Las Cruces Regional ITS Architecture has elements that map to 15 terminators of the National ITS Architecture.  These terminators are shown on the right side (in yellow) of the diagrams below.  Terminators include entities such as Care Facilities (which maps to Regional Medical Centers).

The diagram also identifies the three basic types of communications used to interconnect the elements of the architecture.  These communications types are defined as:

- Fixed point to fixed point Communications: A communications link serving stationary sources.  It may be implemented using a variety of public or private communications networks that may physically include wireless (e.g., microwave) as well as wireline infrastructure.  Both dedicated and shared communications resources may be used.

- Wide Area Wireless Communications: A communications link that provides communications via a wireless device between a user and an infrastructure-based system.  Both broadcast (one-way) and interactive (two-way) communications services are grouped into wide-area wireless communications.  These links support a range of services including real-time traveler information and various forms of fleet communications.

- Dedicated Short Range Communications: A wireless communications channel used for close-proximity communications between vehicles and the immediate infrastructure.  It supports location-specific communications for ITS capabilities such as toll collection, transit vehicle management, driver information, automated commercial vehicle operations, and pre-emption or priority.

Figure 6.  Las Cruces Regional ITS Architecture System Interconnect Diagram

## Customized Market Packages

The market packages identified in the National ITS Architecture (see Section 3) have been customized to reflect the unique systems and connections within the Las Cruces region, as well as connections just outside of the region (e.g.  other counties).  Each customized market package can be shown graphically, with the market package name, the entity from the National ITS Architecture and the specific Las Cruces elements associated with the entity.  In addition the market packages show the information flows that are exchanged (or will be exchanged) between elements.

Figure 7 is an example of an ATMS market package for Surface Street Control that has been customized for the City of Las Cruces.  This market package shows the two subsystems, Traffic Management and Roadway, and the associated elements.  Information flows (called “architecture flows” in the National ITS Architecture) between the subsystems indicate what information is being shared.  The market packages that were customized for the Las Cruces Regional ITS Architecture can be found on the Las Cruces Regional Architecture web page by selecting the “Market Packages by Functional Area” button on the left side menu bar.  Under the “Market Package by Functional Area” tab, market packages are grouped by functional areas (e.g.  Traffic Management, Maintenance and Construction, and Public Transportation).  Each set of customized market packages (based on the functional area) can be viewed by clicking on the Market Package Diagram icon under each area heading.  It is important to note that while the market package table on the web page shows all of the market packages from the National ITS Architecture, only those selected for the region are included in the diagrams.  The selected market packages on the web page also are highlighted in the web page table with bold print and are indicated as existing or planned.

Figure 7.  Example Customized Market Package

## Regional Architecture Information Flows

While it is important to identify the various systems and stakeholders as part of the Las Cruces Regional ITS Architecture, a primary purpose of the architecture is to identify the connectivity between transportation systems in the region.  The interconnect (sausage) diagrams shown previously in Figure 6 showed the high level relationships of the elements in the region.  The market packages from the National ITS Architecture represent services that can be deployed as an integrated capability, and the customized market package diagrams show the information flows between the subsystems and terminators (elements within the region) that are most important to the operation of the market packages.  How these systems interface with each other is an integral part of the overall architecture.

There are 161 different elements identified as part of the Las Cruces Regional ITS Architecture.  These elements include City, County and Regional traffic operations centers, transit centers, transit vehicles, public safety dispatch centers, media outlets, and others — essentially all of the existing and planned physical components that contribute to the region’s intelligent transportation system.  Interfaces have been defined for each element in the architecture.  For example, the City of Las Cruces Traffic Operations Center has planned interfaces with 45 other elements in the region ranging from field equipment to transit centers.  Some of the interfaces are far less complex.  For example the City of Las Cruces Transportation Center has interfaces with only one other element in the architecture, the RoadRUNNER Fixed Route Dispatch System.  The Las Cruces Regional ITS Architecture defines a total of 714 interfaces from one element to another.

Elements and their interfaces are accessible via the Las Cruces Regional ITS Architecture web page by clicking on the “ITS Inventory” button.  On the web page, elements are listed alphabetically in the column on the left, and the description of each element appears on the right.  By clicking on (selecting) an element, the element detail page comes up where the user can view the element definition, who the stakeholder is, the current status of the element, and the other elements with which the selected element interfaces with.  Figure 8 below is an example of an element detail page for the City of Las Cruces Traffic Operations Center.

Figure 8.  Example of Element Detail With Interfaces

By clicking on (selecting) an element that interfaces with the City of Las Cruces Traffic Operations Center, more detailed information about the particular interface is pulled up (e.g.  architecture flows).  Architecture flows between elements define specific information that is exchanged by the elements.  Each architecture flow has a direction, name and definition.  Most of the architecture flows match ones from the National ITS Architecture (the mapping of elements to National ITS Architecture entities allowed the developers to match the architecture flows to the appropriate interfaces).  In some cases, new “user defined” flows have been created for interfaces or connections that are not expressed in the National ITS Architecture (NOTE: User defined flows have a “_ud” at the end of the flow name to indicate they are user defined).  These architecture flows define the interface requirements between the various elements in the region.  The Las Cruces Regional ITS Architecture defines 2387 triplets (a source element, architecture flow and destination element) throughout the region.

An example of architecture flows between two elements is shown in Figure 9.  In this interface, the architecture flows that go between the MVRDA Communications Center and the City of Las Cruces Traffic Operations Center are shown.  Although both of these elements are existing, the architecture flows on this interface are shown as planned (indicated by the (P) after each flow).  This signifies that these elements do not currently share data communications (since the architecture flows are all shown as planned).

Each of the individual element interfaces can be accessed on the Las Cruces Regional ITS Architecture web page by following the instructions listed previously.  Specifically, the user can click on the “ITS Inventory” button, select the element whose interfaces they are reviewing in order to bring up the element detail page.  Once on the element detail page, scrolling down to the “Interfaces” and clicking on (selecting) an interfacing element, more detailed information about the particular interface is pulled up (e.g.  architecture flows).  Once you have selected the interfacing element, a screen should appear that is similar to the diagram shown in Figure 9 (which shows all of associated architecture flows).  Selecting any of the architecture flows will provide a definition, and any standards associated with that architecture flow are noted.

Figure 9.  Example of Architecture Flows Between Elements

# Functional Requirements

Functional requirements are a description of the functions or activities that are currently performed by the ITS elements or that are planned to be performed in the future.  For the Las Cruces Regional ITS Architecture, these functions have been developed by using the functional assignments underlying the National ITS Architecture.

In the National ITS Architecture, a Market Package is defined by subsystems, equipment packages, and architecture flows, which operate together to perform a particular transportation service.  Equipment Packages represent pieces of a subsystem that perform a single function.  (Further descriptions of these National ITS Architecture concepts are in Section 3.)  Note: there are no equipment packages defined for the Terminators of the National ITS Architecture since they represent systems on the boundary of the architecture and, therefore, do not have functional descriptions within the architecture.  Hence any element mapped only to a National ITS Architecture terminator - e.g.  National Weather Service - will not have functions assigned to it.  To consider an example of this mapping beginning with market packages, the Surface Street Control (ATMS03) market package is composed of the two Traffic Management Subsystem equipment packages, TMC Signal Control and Traffic Maintenance, and two Roadway Subsystem equipment packages, Roadway Signal Control and Roadway Equipment Coordination.  The definitions of these four equipment packages, copied from version 5.1 of the National ITS Architecture, are:

- TMC Signal Control.  This equipment package provides the capability for traffic managers to monitor and manage the traffic flow at signalized intersections.  This capability includes analyzing and reducing the collected data from traffic surveillance equipment and developing and implementing control plans for signalized intersections.  Control plans may be developed and implemented that coordinate signals at many intersections under the domain of a single traffic management subsystem.  In advanced implementations, this package collects route planning information and integrates and uses this information in predicting future traffic conditions and optimizing the traffic control strategy for these conditions.  These capabilities are achieved through real-time communication of logged routes from an Information Service Provider.  The planned control strategies can be passed back to the Information Service Provider so that the intended strategies can be reflected in future route planning.

- Traffic Maintenance.  This equipment package provides monitoring and remote diagnostics of field equipment to detect field equipment failures, issues problem reports, and tracks the repair or replacement of the failed equipment.

- Roadway Signal Controls.  This equipment package provides the capabilities to control traffic signals at major intersections and on main highways for urban areas.  This equipment package is generally constrained to a single jurisdiction.

- Roadway Equipment Coordination.  This equipment package coordinates field equipment that is distributed along the roadway by supporting direct communications between field equipment.  This includes coordination between remote sensors and field devices (e.g., Dynamic Message Signs) and coordination between the field devices themselves (e.g., coordination between traffic controllers that are controlling adjacent intersections).

The approach used in the Las Cruces Regional ITS Architecture was to begin with the mapping of equipment packages to elements as an initial definition of the functions being performed by each element.  Then this mapping is tailored, or customized, in the Turbo Architecture tool to provide a more accurate picture of the functions performed by each element.  The Turbo Architecture tool also contains a detailed mapping of functional requirements (written as “shall” statements) to each equipment package.  This mapping to functional requirements has been customized for each element, selecting only those requirements that are appropriate given the definition and connectivity of the element.  In some cases the functional requirement has been customized to better reflect the functions of the element.  This detailed mapping of functional requirements to elements is contained in the Turbo Architecture database and is provided in tabular form in a separate Excel file.

The mapping of elements to the basic functions (equipment packages) is provided in Appendix C and on the hyperlinked web site version of the architecture.  On the website, the detail page for each element (which is accessed by clicking on the hyperlinked element name within the “ITS Inventory”, “Inventory by Stakeholder” or “Inventory by Entity” tabs) has a list of the equipment packages assigned to the element.  Sometimes the user may need to scroll down to see the equipment packages.

For example, the MVRDA Communications Center element has the following equipment packages assigned to it:

- Center Secure Area Alarm Support

- Center Secure Area Sensor Management

- Center Secure Area Surveillance

- Emergency Call-Taking

- Emergency Data Collection

- Emergency Dispatch

- Emergency Early Warning System

- Emergency Environmental Monitoring

- Emergency Evacuation Support

- Emergency Response Management

- Emergency Routing

- Incident Command

- Service Patrol Management

This represents a first level of detail that can be obtained in the hyperlinked web site in connection with functionality.  The additional level of detail, or detailed functional requirements, can be accessed by clicking on any of the equipment packages associated with the element you under review.  Using the above example, viewing the MVRDA Communications Center element detail page the user can see the equipment packages listed above.  If the user were to select one of the equipment packages (all listed as hyperlinks), the equipment package detail page would appear.  This page lists the detailed functional requirements that have been customized for the Las Cruces Regional ITS Architecture as well as providing a full description of what this equipment package contains and a listing of all element that are associated with this equipment package.

# Standards

The following subsections provide a discussion of ITS standards potentially applicable in the region and how to identify the standards that might be applicable on a specific interface within the architecture.

## Discussion of Key Standards for the Region

ITS standards establish a common way in which devices connect and communicate with one another.  This allows transportation agencies to implement systems that cost-effectively exchange pertinent data and accommodate equipment replacement, system upgrades, and system expansion.  Standards benefit the traveling public by providing products that will function consistently and reliably throughout the region.  ITS standards contribute to a safer and more efficient transportation system, facilitate regional interoperability, and promote an innovative and competitive market for transportation products and services.

The use of ITS standards is very important to project development in the region.  Table 9 identifies the ITS standards that are potentially applicable to the region.  This table was created by taking the standards information available in the Turbo Architecture database (which identifies standards applicable to each architecture flow) and taking the total set of standards that result from all of the selected flows.  The table provides the status of the standards effort as of December 2005 (the most recent update of the information).  The table lists the abbreviation of Standards Development Organization (SDO) in the first column, the name of the standard in the second column and the number of the standard in the third column.  Regular updates of SDO activities will help ensure that the latest standards are utilized.  The SDOs involved in the development of ITS standards who are listed in the table include:

- American Association of State Highway and Transportation Officials (AASHTO)

- American National Standards Institute (ANSI)

- American Public Transportation Association (APTA)

- American Society for Testing and Materials (ASTM)

- Institute of Electrical and Electronics Engineers (IEEE)

- Institute of Transportation Engineers (ITE)

- National Equipment Manufacturers Association (NEMA)

- Society of Automotive Engineers (SAE)

Table 9.  Applicable ITS Standards

| SDO | Standard Title | Standard Doc ID |
| --- | --- | --- |
| AASHTO/ITE | Traffic Management Data Dictionary and Message Sets for External TMC Communication (TMDD and MS/ETMCC) | ITE TMDD 2.1 |
| AASHTO/ITE/NEMA | Octet Encoding Rules (OER) Base Protocol | NTCIP 1102 |
| AASHTO/ITE/NEMA | Transportation Management Protocols (TMP) | NTCIP 1103 |
| AASHTO/ITE/NEMA | Center-to-Center Naming Convention Specification | NTCIP 1104 |
| AASHTO/ITE/NEMA | Global Object Definitions | NTCIP 1201 |
| AASHTO/ITE/NEMA | Object Definitions for Actuated Traffic Signal Controller (ASC) Units | NTCIP 1202 |
| AASHTO/ITE/NEMA | Object Definitions for Dynamic Message Signs (DMS) | NTCIP 1203 |
| AASHTO/ITE/NEMA | Object Definitions for Environmental Sensor Stations (ESS) | NTCIP 1204 |
| AASHTO/ITE/NEMA | Object Definitions for Closed Circuit Television (CCTV) Camera Control | NTCIP 1205 |
| AASHTO/ITE/NEMA | Object Definitions for Data Collection and Monitoring (DCM) Devices | NTCIP 1206 |
| AASHTO/ITE/NEMA | Object Definitions for Ramp Meter Control (RMC) Units | NTCIP 1207 |
| AASHTO/ITE/NEMA | Object Definitions for Closed Circuit Television (CCTV) Switching | NTCIP 1208 |
| AASHTO/ITE/NEMA | Data Element Definitions for Transportation Sensor Systems (TSS) | NTCIP 1209 |
| AASHTO/ITE/NEMA | Field Management Stations (FMS) - Part 1: Object Definitions for Signal System Masters | NTCIP 1210 |
| AASHTO/ITE/NEMA | Object Definitions for Signal Control and Prioritization (SCP) | NTCIP 1211 |
| AASHTO/ITE/NEMA | Point to Multi-Point Protocol Using RS-232 Subnetwork Profile | NTCIP 2101 |
| AASHTO/ITE/NEMA | Point to Multi-Point Protocol Using FSK Modem Subnetwork Profile | NTCIP 2102 |
| AASHTO/ITE/NEMA | Point-to-Point Protocol Over RS-232 Subnetwork Profile | NTCIP 2103 |
| AASHTO/ITE/NEMA | Ethernet Subnetwork Profile | NTCIP 2104 |
| AASHTO/ITE/NEMA | Transportation Transport Profile | NTCIP 2201 |
| AASHTO/ITE/NEMA | Internet (TCP/IP and UDP/IP) Transport Profile | NTCIP 2202 |
| AASHTO/ITE/NEMA | Simple Transportation Management Framework (STMF) Application Profile | NTCIP 2301 |
| AASHTO/ITE/NEMA | Trivial File Transfer Protocol (TFTP) Application Profile | NTCIP 2302 |
| AASHTO/ITE/NEMA | File Transfer Protocol (FTP) Application Profile | NTCIP 2303 |
| AASHTO/ITE/NEMA | Application Profile for DATEX-ASN (AP-DATEX) | NTCIP 2304 |
| AASHTO/ITE/NEMA | Application Profile for XML Message Encoding and Transport in ITS Center-to-Center Communications (C2C XML) | NTCIP 2306 |
| APTA | Standard for Transit Communications Interface Profiles | APTA TCIP-S-001 3.0.0 |
| ASTM | Standard Specification for Telecommunications and Information Exchange Between Roadside and Vehicle Systems - 5 GHz Band Dedicated Short Range Communications (DSRC) Medium Access Control (MAC) and Physical Layer (PHY) Specifications | ASTM E2213-03 |
| ASTM | Standard Practice for Metadata to Support Archived Data Management Systems | ASTM E2468-05 |
| ASTM | Standard Specifications for Archiving ITS-Generated Traffic Monitoring Data | ASTM WK7604 |
| IEEE | Standard for Message Sets for Vehicle/Roadside Communications | IEEE 1455-1999 |
| IEEE | Standard for Common Incident Management Message Sets for use by Emergency Management Centers | IEEE 1512 -2006 |
| IEEE | Standard for Traffic Incident Management Message Sets for Use by Emergency Management Centers | IEEE 1512.1-2006 |
| IEEE | Standard for Public Safety Traffic Incident Management Message Sets for Use by Emergency Management Centers | IEEE 1512.2-2004 |
| IEEE | Standard for Hazardous Material Incident Management Message Sets for Use by Emergency Management Centers | IEEE 1512.3-2006 |
| IEEE | Standard for the Interface Between the Rail Subsystem and the Highway Subsystem at a Highway Rail Intersection | IEEE 1570-2002 |
| IEEE | Standard for Wireless Access in Vehicular Environments (WAVE) - Resource Manager | IEEE 1609.1-2006 |
| IEEE | Standard for Wireless Access in Vehicular Environments (WAVE) - Security Services for Applications and Management Messages | IEEE 1609.2-2006 |
| IEEE | Standard for Wireless Access in Vehicular Environments (WAVE) - Multi-Channel Operation | IEEE 1609.4-2006 |
| IEEE | Standard for Common Traffic Incident Management Message Sets for Use in Entities External to Centers | IEEE P1512.4 |
| IEEE | Standard for Wireless Access in Vehicular Environments (WAVE) - Networking Services | IEEE P1609.3 |
| IEEE | Standard for Information Technology - Telecommunications and Information Exchange Between Systems - Local and Metropolitan Area Networks - Specific Requirements - Part II: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications | IEEE P802.11p |
| SAE | Location Referencing Message Specification (LRMS) | SAE J2266 |
| SAE | Message Set for Advanced Traveler Information System (ATIS) | SAE J2354 |
| SAE | Messages for Handling Strings and Look-Up Tables in ATIS Standards | SAE J2540 |

NTCIP C2F: NTCIP Center-to-Field Standards Group

Table 10 above specifies the NTCIP Center-to-Field (NTCIP C2F) Group of Standards, which addresses the communications protocols between a center and the ITS field devices it manages.  The family includes the communications profiles that cover the interfaces between a traffic management center and dynamic message signs, ramp meters, environmental sensors, or CCTVs under its control.  These protocols are common across all Center-to-Field interfaces in the National ITS Architecture, and rather than repeat the entire list for each architecture flow, we have created this summary entry – the NTCIP C2F Group of communications standards.

The “vocabulary” (objects) is specific to the actual architecture flow in the National ITS Architecture and is therefore mapped to the corresponding Data Object standard.  (In the example above, the “Object Definitions for Dynamic Message Signs” standard would be mapped to the specific control and data flows between the Traffic Management Subsystem and the Roadway DMS equipment).

In order to satisfy a wide spectrum of system and regional communications requirements, Center-to-Field ITS deployments should each implement the combinations of the following NTCIP C2F communications protocols that best meet their needs.

This Group includes the following Standards Activities:

- NTCIP 1102: Base Standard: Octet Encoding Rules (OER)

- NTCIP 1103: Simple Transportation Management Protocol (STMP)

- NTCIP 2101: Point to Multi-Point Protocol Using RS-232 Subnetwork Profile

- NTCIP 2102: Subnet Profile for PMPP Over FSK modems

- NTCIP 2103: Subnet Profile for Point-to-Point Protocol using RS 232

- NTCIP 2104: Subnet Profile for Ethernet

- NTCIP 2201: Transportation Transport Profile

- NTCIP 2202: Internet (TCP/IP and UDP/IP) Transport Profile

- NTCIP 2301: Application Profile for Simple Transportation Management Framework (STMF)

- NTCIP 2302: Application Profile for Trivial File Transfer Protocol

- NTCIP 2303: Application Profile for File Transfer Protocol (FTP)

NTCIP C2C: NTCIP Center-to-Center Standards Group

Table 10 above specifies the NTCIP Center-to-Center (NTCIP C2C) Group of Standards, which address the communications protocols between two centers (e.g.  two traffic management centers exchanging information to facilitate regional coordination of traffic signals).  Some of the communication protocols covered by this family are CORBA, DATEX-ASN and FTP.  These protocols are common across all Center-to-Center interfaces in the National ITS Architecture, and rather than repeat the entire list for each architecture flow, we have created this summary entry – the NTCIP C2C Group of communications standards.

The standards that describe the “vocabulary” (data elements and messages) are mapped to specific architecture flows rather than the entire set of NTCIP C2C interfaces.  In the regional traffic coordination example above, the “Traffic Management Data Dictionary” and the “Message Set for External TMC Communications” standards would be mapped to the specific flows between two Traffic Management Subsystems.

In order to satisfy a wide spectrum of system and regional communications requirements, Center-to-Center ITS deployments should each implement the combinations of the following NTCIP C2C communications protocols that best meet their needs.

This Group includes the following Standards Activities:

- NTCIP 1102: Base Standard: Octet Encoding Rules (OER)

- NTCIP 1104: Center-to-Center Naming Convention Specification

- NTCIP 2104: Subnet Profile for Ethernet

- NTCIP 2202: Internet (TCP/IP and UDP/IP) Transport Profile

- NTCIP 2303: Application Profile for File Transfer Protocol (FTP)

- NTCIP 2304: Application Profile for Data Exchange ASN.1 (DATEX)

- NTCIP 2306: Application Profiles for XML Message Encoding and Transport in ITS Center to Center Communications

## Reference to the Detailed Standards information on the Web Site

The previous section provides a general discussion of the standards environment in the region.  However the architecture does contain a far more detailed standards view, one that maps applicable standards to the individual information flows that goes from one element to another.  This detailed information is contained in the hyperlinked web site and can be accessed through the links to the architecture flows shown as part of each interface.  Each element description page has a set of links that describe the information flowing to and from the element to other elements of the architecture.  Selecting any of these interface links brings the user an interface page.  For example, the interface between the City of Las Cruces Traffic Operations Center and the City of Las Cruces DMS (dynamic message signs) is shown in Figure 10.  There is one information flow going to the DMS element and two coming back from it.  Selecting one of the flows provides information regarding the flow and gives a list of ITS standards that are applicable to the flow.  An example, for the roadway information system data flow, is shown in Figure 11.

Figure 10.  Example of Interface

Figure 11.  Example of Standards Mapping Page

# Project Sequencing

The incorporation of the Las Cruces Regional ITS Architecture into the planning process will ultimately yield projects that are linked to the ITS architecture.  Through the deployment of projects produced from the planning process, the services supported in the Las Cruces Regional ITS Architecture will be implemented and made a reality in the transportation system.  Project implementation completes the evolution from: transportation needs to services, to functional descriptions in the regional ITS architecture, to project identification in the planning process and to project definition and deployment.  The overarching goal of the Las Cruces Regional ITS Architecture development process is that this evolution takes place with the maximum amount of integration knowledge possible, so as to efficiently and economically implement the ITS systems required to serve the transportation community and users.

Key to this process or evolution is to understand what dependencies or relationships exist between systems and projects so that an order can be identified for deployment.  Given the importance of integration of ITS, and the dependencies of one system on another or one project on another, it is critical to view the entire transportation system at a high functional level.  The Las Cruces Regional ITS Architecture provides this view point and aids in understanding the relationships between the ITS systems in the region.

Project sequencing defines the order in which ITS projects should be implemented.  A good sequence is based on a combination of transportation planning factors that are used to prioritize projects and the project dependencies that show how successive ITS projects can build on one another.  In most cases, the first projects in the project sequence will already be programmed and will simply be extracted from existing transportation plans.  Successive projects will then be added to the sequence based on the project dependencies and other planning factors (e.g.  stakeholder priorities).

## Process For Selecting Projects

A three step process was used to select projects for the Las Cruces Regional ITS Architecture:

- Review of the regional TIP including FY2008 - FY2013 Las Cruces MPO TIP and the statewide STIP- NMDOT FY2006 – FY2011 STIP.

- Gather stakeholder feedback through outreach workshops, offline discussions, meetings and e-mails; and

- Consider Market Package Prioritization (which is covered in Section 6.2).

The Las Cruces Regional ITS Architecture was created based on the needs for the region over the next 20 years.  This ITS architecture identifies which systems (operated by agencies in the Las Cruces MPO region or adjacent to the Las Cruces MPO region) should be interfaced to maximize integration opportunities throughout the region.  Based on the existing and future needs throughout the State and, more specifically, the Las Cruces MPO, the first step of the process was to identify existing or future deployment opportunities.  To better identify these opportunities, ConSysTec reviewed the regional planning documents supplied by the Las Cruces Regional ITS Architecture stakeholders or located on the stakeholders’ websites.  Table 10 notes the sources that were consulted in order to develop the final list of projects:

Table 10.  List of Sources Used for Identifying Projects

| No. | Title |
| --- | --- |
| 1 | NMDOT FY2006 - 2011 Statewide Transportation Improvement Program |
| 2 | 2008 - 2013 Transportation Improvement Program (TIP) for the Las Cruces Metropolitan Planning Organization (Updated April 18, 2007) |
| 3 | 2005 Transportation Plan (2005-2030), Las Cruces Metropolitan Planning Organization |
| 4 | Strategic ITS Plan for the State of New Mexico (Draft) – January 29, 2007 |
| 5 | Stakeholder Outreach Meeting - 5/09/07 |
| 6 | Architecture Review Meeting - 7/19/07 |
| 7 | Individual Stakeholder Meetings (face-to-face, telephone, electronic mail) |
| 8 | Generated by the ConSysTec Architecture Development Team |

These project documents and other inputs detailed many projects and programs scheduled to be implemented over a period of 1-20 years, with an emphasis on the next 3 years.  Although it is important for all of these documents to identify the ITS needs and priorities for specific stakeholders, ITS projects must be included in the New Mexico Statewide STIP or in a regional TIP to receive federal funding.  Both the STIP and the TIP provide a mechanism for locally elected officials, Las Cruces MPO member agencies, and staff to review the region’s capital programming.  It represents a consensus among major transportation interests in the region as to what improvements should have priority for available funds.

Some of the ITS projects identified by this process were included in the current 2008-2013 Las Cruces MPO TIP.  Therefore, they had a funding allocation (estimated cost) assigned to them.  These ITS projects were compared to the Statewide TIP (and other supporting documents) to determine if the proposed ITS project had an existing funding source.  If a TIP’s description was similar to that of the STIP, the projects were combined and the STIP information (regarding funding) was used.  In most cases, the funding source is some source of federal funds that are to be matched (by proportion – e.g.  80% federal and 20% local/state).  These projects are documented in the initial ITS project table.

There were also TIP (and STIP) projects that did not identify ITS in the project description, but were viewed as potentially having an ITS component (e.g.  signal coordination or signal preemption for an intersection improvement).  In this situation, stakeholders were asked to provide feedback regarding the likelihood of ITS being incidental to the TIP project.  Where appropriate, TIP and STIP projects have been included as having ITS components (e.g.  signals, DMS, CCTV, etc.) within their project description.  In the case where it was unclear if ITS elements were to be included in a construction project, the project was left in the table as a place holder for a potential ITS application.  Projects that were determined to have no ITS components in their project description were removed from the initial ITS project list.  Additionally, a few general projects were documented as a placeholder for all of the ITS projects that are currently ongoing or are in the planning process (for NMDOT).

The second step in the process (after identifying which projects were ITS related) was to obtain stakeholder feedback on the proposed ITS projects and their prioritization.  Input from stakeholders was used and later refined to establish which projects were allocated to the short term (0-5 years), medium term (6-10 years), and long term (over 10 years).  Obtaining stakeholder feedback was necessary for the following reasons:

- Ensure the ITS Project was consistent with stakeholder needs.

- Establish an estimated timeline or priority for ITS Project deployment and denoting a general order for project implementation.

- Understand the relationship and traceability between ITS projects and the Las Cruces Regional ITS Architecture (through customized market package prioritization).

Projects listed as part of the regional TIP process, or by the STIP, were identified as short term projects based on the implied relatively short timeframe of the related TIP.

A majority of this process was accomplished throughout stakeholder workshop and individual discussions on the project level.  During the architecture review workshop held in July 2007, a draft list of projects was presented and input from the stakeholders was incorporated into the project list.  During the workshop, comments were received from stakeholders regarding ITS project names, timeframes, funding sources, responsible agencies and programmed projects.  In addition, stakeholders were able to identify additional sources where the analysts could find ITS projects that have been planned for the region.

The results of the workshop, various stakeholder inputs after the workshop, and the project sequencing analysis are provided in Table 11.  The information included in each of the project functional areas is as follows:

- ID.  The ID number associated with this project.

- Project Name and Location.  The name and location of the proposed ITS project.

- Source.  The ITS related document or input from which the project was identified..

- Description.  The description of the project or services to be provided.

- Costs (Thousands).  The total cost (planning, engineering and construction) associated with the proposed project.

- Lead Agency.  The primary agency or stakeholder responsible for the initiation, implementation, and maintenance of the ITS project and its components

- Included in TIP.  Funding is identified through the TIP programming process and is based on federal, state, and local sources as presented by the lead agency.  Primary sources of ITS funding include, but are not limited to CMAQ, STPU, IM, state, and local amounts.

- Market Packages.  Maps the proposed ITS project to a transportation service (customized market package) identified in the Las Cruces Regional ITS Architecture, which reflects traceability from ITS project to the Las Cruces Regional ITS Architecture.  It is important to note that if there are one or more customized market packages listed in this column then it is included as part of the regional ITS architecture.

- Priority.  Indicates the estimated timeframe for an ITS project to be deployed.  It is estimated that short-term projects will be implemented in 0-5 years; medium-term projects will be implemented in 5-10 years; and long-term projects will be implemented in over 10 years.

- Integration Opportunity.  Indicates whether this service provides opportunities for cross jurisdictional integration.

- Comments.  Associated comments about the ITS project from various sources.

Table 11.  Las Cruces ITS Projects

| ID | PROJECT NAME & LOCATION | SOURCE | DESCRIPTION | COSTS | LEAD AGENCY | IN TIP? | MARKET PACKAGES | PRIORITY | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I - Traffic Management Projects |  |  |  |  |  |  |  |  |  |
| 1 | DMS Warning System | 2 | DMS for dust storm warning on border of MPO boundary | $300,000 | NMDOT | Yes | ATMS06-5 | Short Term | Project ID LCMPO-TH-06-01 |
| 2 | Traffic Signal System Upgrades | 7 | including video detection with future video monitoring. Considering red light running | - | City of Las Cruces | - | ATMS01-1 | Short Term |  |
| 3 | CCTV cameras - US 70 & Lohman | 7 |  | - | City of Las Cruces | - | ATMS03-1 | Short Term |  |
| 4 | CCTV cameras - campus | 7 | Cameras on campus (for traffic and security) and I-25 at University (for event management) | - | New Mexico State University | - | ATMS01-4 | Short Term |  |
| 5 | I-10 GRIP Project | 7 | Communications, DMS, CCTV, detectors | - | NMDOT District 1 | - | ATMS01-3, ATMS06-5 | Short Term |  |
| 6 | I-10/I-25 Interchange Reconstruction | 4 | Interchange Reconstruction/CCTV, DMS | - | NMDOT District 1 | In STIP | ATMS01-3, ATMS06-5 | Short Term |  |
| 7 | I-10/I-25/US 70 - Las Cruces | 4 | DMS Installation | - | NMDOT District 1 | In STIP | ATMS06-5 | Short Term |  |
| 8 | TMC Improvements | 4 | Provide data sharing between Las Cruces, NMDOT, El Paso, and Santa Teresa Border, TOC's, CCTV, RWIS | - | NMDOT District 1 | In STIP | ATMS07-3 |  |  |
| 9 | Signal Priority | 4 | Signal Presentation | - | Las Cruces Transit | In STIP | - | Long Term |  |
| 10 | I-10 Corridor ITS | 1 | DMS and Structures Fiber Communication Lines | - | NMDOT District 1 | - | ATMS06-5 | Short Term | Related to STIP 1319 |
| 11 | Dust Storm Warning System | 6 | Statewide – Deming and Lordsburg | - | NMDOT Research Bureau | - | MC03-3 |  |  |
| II - Transit, Transit Operations Prospects |  |  |  |  |  |  |  |  |  |
| 11 | Rolling Stock - Vans | 2 |  | $100,000 | CLC/Roadrunner Transit | Yes | APTS1-1, APTS3-1 | Short Term | Project ID LCMPO-PT-99-05 |
| 12 | Intermodal Facility | 2 | Coordinating AVL with Bus Arrival Signs | $2,475,073 | CLC/Roadrunner Transit | Yes | APTS1-1, APTS8-1 | Short Term | Project ID LCMPO-PT-04-04 |
| 13 | Support Equipment and Rolling stock | 2 | Possibly Automated Dispatch | $28,108 | CLC/Roadrunner Transit | Yes | APTS1-1, APTS2-1 | Short Term | Project ID LCMPO-PT-04-05 |
| 14 | Support Equipment and Facilities | 2 |  | $22,000 | CLC/Roadrunner Transit | Yes | APTS1-1, APTS2-1 | Short Term | Project ID LCMPO-PT-05-02 |
| 15 | Intermodal Facilities - Part 2 | 2 |  | $655,498 | CLC/Roadrunner Transit | Yes | APTS7-1 | Short Term | Project ID LCMPO-PT-06-03 |
| 16 | Rolling Stock | 2 | Possibly AVL, MDT, security features, APC | $1,192,772 | CLC/Roadrunner Transit | Yes | APTS1-1, APTS2-1, APTS4-1, APTS5-1 | Short Term | Project ID LCMPO-PT-06-04 |
| 17 | Rolling Stock and Facilities | 2 | Possibly AVL, MDT, security features, APC | $560,000 | CLC/Roadrunner Transit | Yes | APTS1-1, APTS2-1, APTS4-1, APTS5-1 | Short Term | Project ID LCMPO-PT-06-06 |
| 18 | Rolling Stock | 2 | Possibly AVL, MDT, security features, APC | $106,000 | CLC/Roadrunner Transit | Yes | APTS1-1, APTS2-1, APTS4-1, APTS5-1 | Short Term | Project ID LCMPO-PT-08-01 |
| 19 | Rolling Stock | 2 | Possibly AVL, MDT, security features, APC | $106,774 | CLC/Roadrunner Transit | Yes | APTS1-1, APTS2-1, APTS4-1, APTS5-1 | Short Term | Project ID LCMPO-PT-08-02 |
| 20 | Rolling Stock | 2 | Possibly AVL, MDT, security features, APC | $165,000 | CLC/Roadrunner Transit | Yes | APTS1-1, APTS2-1, APTS4-1, APTS5-1 | Short Term | Project ID LCMPO-PT-08-10 |
| 21 | New Maintenance / Operations Facility | 2 |  | $3,250,000 | CLC/Roadrunner Transit | Yes | APTS2-1, APTS3-1, APTS6-1 | Short Term | Project ID LCMPO-PT-08-11 |
| 22 | Intermodal Facilities - Additional Phase | 2 |  | $750,000 | CLC/Roadrunner Transit | Yes | APTS7-1 | Short Term | Project ID LCMPO-PT-08-12 |
| 23 | ITS Improvements | 2 | ?? | $165,000 | CLC/Roadrunner Transit | Yes | - | Short Term | Project ID LCMPO-PT-08-13 |
| 24 | School Bus AVL | 6 | AVL and GIS for school buses | - | Las Cruces Public Schools | - | APTS1-4 | Short Term |  |
| 25 | Electronic Fare Card System | 6 | Regional transit card | - | RoadRUNNER | - | APTS4-1 | Mid Term |  |
| III - Archive Data and Communications Projects |  |  |  |  |  |  |  |  |  |
| 26 | Citywide Wireless Network | 6 | Provide citywide wireless capability | - | City of Las Cruces | - | - | Mid Term |  |
| IV – Emergency / Public Safety Projects |  |  |  |  |  |  |  |  |  |
| 27 | Wide Area Notification- Reverse 911 | 6 | County wide | - | MVRDA | - | EM10-4 | Short term |  |
| 28 | Mobile Data terminal Gateway | 6 | Installing servers, software and connectivity. Each agency will purchase own hardware. Will include AVL | - | MVRDA | - | EM01-2 | Short term |  |
| 29 | Relocation of Communications Center | 6 | Relocate Communications Center to a more secure and larger facility. | - | MVRDA | - | - | Mid Term |  |
| 30 | Real Time Data Backup | 6 | wireless links to allow data backup by DPS D4. Back up dispatch center (using mobile command post) | - | MVRDA | - | EM01-1 |  |  |
| V – Maintenance and Construction Projects |  |  |  |  |  |  |  |  |  |
| 31 | Public Works Vehicle AVL | 6 |  | - | Dona Ana County |  | MC01-2 | Long Term |  |
| 32 | Public Works and Utilities Vehicle AVL | 6 |  | - | City of Las Cruces | - | MC01-1 | Mid Term |  |
| 33 | Upgrade of Water Level Monitoring Systems | 6 |  | - | Elephant Butte Irrigation District | - | EM07-1 | Short term |  |
| 34 | Upgrade of Water Level Monitoring Systems | 6 |  | - | Dona Ana County Flood Commission | - | EM07-2 | Mid Term |  |
| VI – Commercial Vehicle Operations Projects |  |  |  |  |  |  |  |  |  |
| 35 | Santa Teresa Port of Entry | 1 | ITS Improvements for Operational Efficiency | - | NMDOT District 1 | - | - | Short Term | An MTD project- belongs in Statewide, not LC, but is included here for information only. |

## Gap Analysis

After the initial project list was completed, and input from the Las Cruces Region stakeholders was compiled, a comparative analysis was conducted to see if there were any “gaps” in the identification of projects.  For the purposes of this section, a “gap” refers to a situation where a particular stakeholder has defined the priority of a market package to be high (see Section 4 of this document for the market package prioritization) but no ITS project exists in the previous table that covers this service for this stakeholder.

This analysis was done by first taking a look at the projects that are currently being funded or that are in some stage of the planning process (usually in the TIP or STIP).  These projects consisted of the ones defined in the project sequencing section of this document.  Then the list of projects and their descriptions were compared to information received from stakeholders as to what the region’s intelligent transportation priorities are.  Understanding the stakeholder or agency role within the region and the ITS roles of each stakeholder or agency as it relates to the specific market package, were essential in determining the type of project that is suggested in Table 11.

The main purpose of this analysis was to make sure all of the high priority services are covered by some “project” in the plan.  The indication of “high priority” suggests that these market packages/projects are to be implemented in the short term – 1 to 5 years.  Not having an ITS project defined that is related to a high priority market package could mean the window of opportunity for funding, the final goal of the planning process, may be missed.  In order to receive funding, or to at least be able to apply for funding, these suggested projects, or ones similar to these projects, as they relate specifically to “high priority” market packages, must be defined or identified at this time.

The results of the gap analysis, and suggested project definitions, are provided in Table 13.  The information included for each project includes:

- ID.  The ID number associated with this project.

- Project Name and Location.  The name and location of the proposed ITS project.

- Description.  The detailed description of the project or services to be provided.

- Lead Agency.  The primary agency or stakeholder responsible for the initiation, implementation, and maintenance of the ITS project and its components.

- Market Packages.  Maps the proposed ITS project to a transportation service identified in the Las Cruces Regional ITS Architecture and reflects traceability.

- Comments.  Associated comments about the ITS project from various sources.

Table 12.  Projects Suggested by Gap Analysis

| ID | Project Name and Location | Sample Project Description | Lead Agency | Market Packages | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NMDOT Statewide Database Connectivity | Provide for the data communication links to the NMDOT Consolidated Highway Database, Statewide Crash Information System and the Historical Traffic Database | NMDOT | AD1-4, AD1-5, AD1-6 | Could be the provided as part of the TIMS system development. |
| 4 | NMDOT District 1 PTMS | NMDOT District 1 Public Transportation Management System (PTMS) - a data warehouse that houses transit and public transportation service requirements, demand, etc. for Las Cruces. | NMDOT | AD1-7 |  |
| 5 | Las Cruces MPO ITS Data Warehouse | Las Cruces MPO ITS Data Warehouse - Collecting regional traffic, transit, and emergency information and making it available to other ISPs | Las Cruces MPO | AD2-1, AD2-2 | Las Cruces MPO Traffic Monitoring Program is ongoing - could be expanded to include these additional elements |
| 6 | Purchase transit vehicles (buses, shuttles, etc.) for fleet replacement | Purchase of fixed route or paratransit vehicles with ITS technologies (AVL, MDTs, Fare Collection, CCTV, silent alarms, etc.) already installed on them | Various | Various APTS | This suggested project could apply for all transit agencies, and could include retrofitting existing vehicles |
| 7 | Transit Security Equipment | Install transit security equipment (CCTV, silent alarms, sensors, etc.) on fleet vehicles and on transit kiosks or at transit centers | Various | APTS5 (All) | This suggested project could apply for all transit agencies |
| 8 | Transit Traveler Information | Install transit kiosks for transit traveler information at intermodal transit centers. | RoadRUNNER | APTS8-1 |  |
| 9 | NMDOT District Public Information System | A NMDOT District based public information system that collects information (traffic, emergency, transit, etc) from regional providers and broadcasts it out to the same regional providers, the media, private users and makes it available on the NMDOT District website | NMDOT | ATIS1-6, ATIS1-7 | District Level |
| 10 | NMDOT Advanced Traveler Information System | Provide for a data warehouse that is linked to all of the other data warehouses owned and operated by NMDOT, or who agreements are drawn up to include. Provides for an online interface, the systems to support real-time queries, etc. | NMDOT | ATIS2-1, ATIS2-2 | Statewide Level |
| 11 | Regional Coordination of TMCs | Provide for operational links to allow the Statewide TMC to control the District 1 TOC during "off" hours | NMDOT | ATMS7-3 |  |
| 12 | Regional Emergency Communications Network | Establish a virtual communications network (and repository) for all emergency providers for coordination of incidents and emergencies throughout the region. | Various | EM01-1 |  |
| 13 | HELP Vehicle Communications System | Provide communications infrastructure for NMDOT District 3 HELP vehicles to send incident information to regional traffic and emergency management agencies | NMDOT | EM04-1 |  |
| 14 | EOC Communications Network | Establish a communications network for coordinating disasters, and for coordinating reentry procedures following a disaster that required an evacuation | Various | EM08 and EM09 |  |
| 15 | Regional RWIS | Install RWIS, including dust monitors, at various strategic locations throughout the region | Various | MC03-1, MC03-2, MC03-3 | Includes NMDOT, City of Las Cruces, and Dona Ana County. |
| 16 | Weather Information Distribution | Establish communications infrastructure to distribute road weather information to regional transit, maintenance, traffic and emergency providers as well as post information on agency owned VMS/DMS | Various | MC04 | Includes NMDOT, City of Las Cruces, and Dona Ana County. |

## How to Use the Recommended Project Sequencing

The recommended ITS project sequencing (Table 11 above) should be used as an input for the Transportation Improvement Plan (TIP) and Long Range Transportation Plan (LRTP) for Las Cruces MPO and the Strategic/Long Range Plan for other organizations for planning purposes (as described in Section 12).  The planning process allocates ITS projects funding in coordination with other transportation projects.

As displayed in Table 11, projects are classified as short/medium/long-term timeframes.  These projects should be represented in the long range transportation plans, either the TIP, LRTP, and/or the ITS Implementation Plan.  As these sequenced projects go through the planning process, the ones identified as short-term would be transitioned in the TIP and/or local Capital Improvement Plans.  Since the table defines a short-term project as being deployed in 0-5 years and the TIP defines a project as being deployed in 1-3 years, stakeholders are required to further examine the short-term projects and determine which should be represented in the TIP.

The key question stakeholders may ask is, “Now that we have a comprehensive list of ITS projects separated by timeframes, how do I use the projects to address the needs described in Section 6.1.  To answer this question, stakeholders should focus on the following concepts:

- Why is This Important.  Stakeholders should remember the reasons for going through the process of creating sequenced ITS projects.  Ultimately they want to deploy projects that support the needs expressed in their ITS Architecture.

- Who’s in Charge.  Stakeholders should consider identifying a person or group that is responsible for managing how ITS Projects get deployed.  This person or group would be aware of the big picture by familiarizing themselves with all of the planned activities and ensure integration opportunities are maximized in project deployments.

- Systematic Process.  Stakeholders should ensure that projects are managed in a systematic manner.

- Funding Allocation.  Stakeholders should ensure funding is allocated appropriately to support projects that have dependencies or synergies to be utilized.  This is important if there are future projects that will depend on a short term or current project.  The short term or current project must be funded appropriately to support the accommodation of known future project features or interfaces, thus avoiding redesign for future project accommodation.

- Project List Management.  Stakeholders should prioritize projects within their common timeframes based on the aforementioned concepts.  It is important for short-term projects to be reviewed by stakeholders prior to being transitioned into the TIP.  A member of the ITS Subcommittee, or a group of members, should be responsible for removing projects from the regional list once implemented.  Although project lists may reflect a single project, projects are typically broken into multiple phases and are implemented in an incremental manner.  For example, many ITS projects are partially deployed as part of larger construction projects.  A project’s scope might involve interfacing with ten agencies and funding constraints may require agencies to be interconnected one at a time.  In this situation, a project might by implemented in five years, if two agencies are being interconnected per year.  If a project is partially implemented due to unforeseen circumstances (e.g.  limited funding received), then the list manager should update the project to reflect the remaining components that need to be implemented.  The key point for project list management is projects will be implemented in an incremental manner, therefore the members of the ITS Subcommittee designated as the list manager(s) should keep accurate records of the incremental process and meet with stakeholders to determine how funding should be reallocated.

- Desired Outcome.  Stakeholders should remember the desired outcome, which is to deploy projects to maximize integration opportunities throughout the Las Cruces.  Therefore, when projects are transitioned into the project development phase, stakeholders should always be aware of other project deployment activities (even if the other activities require a project to be deployed at a different time).  This mindset will require stakeholders to be flexible in developing interfaces what will allow for future expansion based on overall regional needs.

An important issue to remember is when a project it to be implemented, stakeholders should convene to determine the specific details for deploying a project (e.g. how many phases will be required for this project and which components of market packages are allocated to a particular phase?).  Table 11 should be used as a guide in which agencies/systems and interfaces consider during the discussion and design phase of project implementation.

# Agreements

There are several types of arrangements associated with the interfaces included when deploying ITS projects within the region.  The identification of institutional agreements required is crucial to the development of consensus architecture.  This section documents the agreements associated with the Las Cruces Regional ITS Architecture, whether planned or existing.

## Types of Agreements

There are several types of arrangements associated with the interfaces included when deploying ITS projects within the Las Cruces region, and throughout the state.  Data exchanges between systems require agreements on the transmission protocol and data formats to ensure compatibility.  Coordinating field device operations owned by different agencies requires defined procedures for submitting message requests and rules governing when such requests can be honored.  Such coordination can be done with informal arrangements such as a Memorandum of Understanding (MOU).  Sharing control of field devices operated by different agencies, on the other hand, involves more liability issues, which requires more formal agreements.  Coordinated incident response may also require formal agreements, but also requires group training of personnel from various agencies.  While all interfaces involve agreements for data compatibility, agreements for procedure and operation as well as training can also be critical elements to optimizing the benefits of the architecture.

Table 13 identifies types of potential agreements that could be used by agencies in the region.  .  It is recognized, however, that a specific agreement mechanism used among stakeholders may be different between them (for example the nature and limitations associated with a MOU might vary between stakeholders).  This should be taken into consideration when identifying and pursuing the proper agreement mechanism.

Table 13.  Types of Institutional Agreements

| Type of Agreement | Description |
| --- | --- |
| Handshake Agreement | Early agreement between one or more partners |
|  | Not recommended for long term operations. |
| Memorandum of Understanding | Initial agreement used to provide minimal detail and usually demonstrating a general consensus. |
|  | Used to expand a more detailed agreement like an Interagency Agreement which may be broad in scope but contains all of the standard contract clauses required by a specific agency. |
|  | May serve as a means to modify a much broader Master Funding Agreement, allowing the master agreement to cover various ITS projects throughout the region and the MOUs to specify the scope and differences between the projects. |
| Interagency Agreement | Between public agencies (e.g., transit authorities, cities, counties, etc.) for operations, services or funding |
|  | Documents responsibility, functions and liability, at a minimum. |
| Intergovernmental Agreement | Between governmental agencies (e.g., Agreements between universities and State DOT, MPOs and State DOT, etc.) |
| Operational Agreement | Between any agency involved in funding, operating, maintaining or using the right-of-way of another public or private agency. |
|  | Identifies respective responsibilities for all activities associated with shared systems being operated and/or maintained. |
| Funding Agreement | Documents the funding arrangements for ITS projects (and other projects) |
|  | Includes at a minimum standard funding clauses, detailed scope, services to be performed, detailed project budgets, etc. |
| Master Agreements | Standard contract and/or legal verbiage for a specific agency and serving as a master agreement by which all business is done. These agreements can be found in the legal department of many public agencies. |
|  | Allows states, cities, transit agencies, and other public agencies that do business with the same agencies over and over (e.g., cities and counties) to have one Master Agreement that uses smaller agreements (e.g., MOUs, Scope-of-Work and Budget Modifications, Funding Agreements, Project Agreements, etc.) to modify or expand the boundaries of the larger agreement to include more specific language. |

In addition to the agreements noted above, one element that must be considered is data ownership.  The type of agreement used to address this issue may vary depending upon agencies involved.

## Existing Agreements

The identification of institutional agreements, along with if these agreements exist or need to be formulated, is a key output of this effort and should be updated periodically as part of the overall Regional ITS Architecture.  During the stakeholder workshops held in Las Cruces, stakeholders identified a variety of agreements that currently exist in the region.  In addition, the idea of formulating additional institutional agreements was discussed in great detail.  From these meetings, and additional discussions with stakeholders who were unable to attend the workshops, a list of existing agreements throughout the Las Cruces Region was compiled.  It is represented in Table 14 below.

Table 14.  Existing Institutional Agreements

| Agencies |  | Type of Agreement | Reason |
| --- | --- | --- | --- |
| NMDOT | Dona Ana County | Operational Agreement or MOU | NMDOT provides field equipment for counties. The counties are responsible for power. |
| City of Las Cruces | Dona Ana County | Operational Agreement or MOU | The City of Las Cruces provides maintenance to ITS field equipment. Counties are responsible for power. |
| NMDOT | City of Las Cruces | Operational Agreement | Operation of NMDOT signals (based on geographic location). |
| NMDOT | Dona Ana County | Operational Agreement | Operation of NMDOT signals (based on geographic location). |
| Mesilla Valley Regional Dispatch Authority | Dona Ana County | MOU | Mutual Aid agreements for Fire/EMS. |
| Mesilla Valley Regional Dispatch Authority | City of Las Cruces | MOU | Mutual Aid agreements for Fire/EMS. |
| Mesilla Valley Regional Dispatch Authority | Town of Mesilla | MOU | Mutual Aid agreements for Fire/EMS. |
| Mesilla Valley Regional Dispatch Authority | New Mexico State University | MOU | Mutual Aid agreements for Fire/EMS. |
| Mesilla Valley Regional Dispatch Authority | City of Las Cruces | Interagency Agreement | The Mesilla Valley Regional Dispatch Authority may dispatch maintenance and construction vehicles after hours. |
| City of Rio Rancho | Dona Ana County | Interagency Agreement | The Mesilla Valley Regional Dispatch Authority may dispatch maintenance and construction vehicles after hours. |
| Las Cruces MPO | NMDOT | Intergovernmental Agreement | Data sharing agreement. Las Cruces MPO Traffic counting responsibilities |
| Las Cruces MPO | City of Las Cruces | Intergovernmental Agreement | Intersection counting responsibilities. |
| City of Las Cruces | RoadRUNNER | Interagency Agreement | Signal priority for RoadRUNNER transit vehicles throughout the City. |
| City of Las Cruces | Dona Ana County | MOU | Signal Preemption for fire and EMS vehicles. |
| NMDOT | City of Las Cruces | MOU | Signal Preemption for fire and EMS vehicles. |
| NMDOT | Dona Ana County | MOU | Signal Preemption for fire and EMS vehicles. |

## Potential Agreements

For each customized market package developed in the architecture, potential institutional agreements can be identified.  Agreements are identified on the basis of information being shared across institutional boundaries.  Instances that involve the sharing of information wholly within one institution do not require an agreement.  For example, Figure 12 illustrates transit fixed route operations for the RoadRUNNER Transit.  Taking a look at the “right” side of the diagram, the RoadRUNNER Fixed Route Dispatch System is sharing information with the RoadRUNNER Fixed Route Transit Vehicles and other high capacity transit vehicles as well as the RoadRUNNER Transit website.  Since the RoadRUNNER dispatch system is sharing information to the RoadRUNNER’s fixed route vehicles and the RoadRUNNER Transit website, no institutional agreements are necessary because the sharing of information is between elements under the same institutional entity.

Taking a look at the “left” side of the diagram (and a few interfaces on the right as well) yields a different institutional relationship.  The RoadRUNNER Fixed Route Dispatch System is collecting road network conditions about the roadways managed by the City of Las Cruces, Dona Ana County, and NMDOT.  Since the RoadRUNNER dispatch system is receiving information from other agency traffic operations centers, institutional agreements may be necessary.  Simply put, the City of Las Cruces (City of Las Cruces Traffic Operations Center), Dona Ana County (Dona Ana County Traffic Operations Center), and NMDOT (NMDOT District 1 Traffic Operations Center) agree to share road network conditions with RoadRUNNER Transit.  The same application of this theory would also apply to the maintenance and construction elements as well as the additional information service providers as identified in the figure.  Also, this particular example shows a unidirectional flow of information, but it is quite common for there to be information going in both directions that would be subject to an agreement.

Figure 12.  Example Market Package: APTS2 - Transit Fixed Route Operations for RoadRUNNER Transit

Each market package identified in the Las Cruces Regional ITS Architecture and selected as a “high priority market package” was analyzed using the same methodology as described above.  Table 15 documents the results of this analysis and identifies where institutional agreements are needed and what the purpose of the agreement are (i.e.  the information that is being shared that would require an agreement).  The table is sorted by “Priority Service”, or the transportation service where the agreement has been identified.  The “Potential Parties to Agreement” identified the institutional entities that might share information, generally starting with the “source” entity or entities.  The “Purpose” column gives a short description of the information being shared.  It is important to note that the entities listed in the following table have not been identified in this table as a source or destination for the information being shared, nor does this table contain the status of any institutional agreements.  This table is intended to identify the possible agreements between entities.  Therefore, this table should be used as a starting point when identifying or pursuing agreements between entities.

Table 15.  Potential Institutional Agreements

| Priority Service | Potential Parties to Agreement | Purpose |
| --- | --- | --- |
| Archive Data | Las Cruces MPO, NMDOT, City of Las Cruces, Dona Ana County. | Providing traffic archive data |
|  | Las Cruces MPO, NMDOT, City of Las Cruces, Dona Ana County, New Mexico State University, RoadRUNNER, South Central RTD, Southwest RTD | Archive coordination |
|  | NMDOT, Mesilla Valley Regional Dispatch Authority, NMDPS, New Mexico State University | Providing emergency archive information |
|  | NMDOT, RoadRUNNER, South Central RTD, Southwest RTD, New Mexico State University | Sharing transit archive information |
| Emergency Management | NMDOT, Mesilla Valley Regional Dispatch Authority, City of Las Cruces, Dona Ana County, Municipalities, US Border Patrol, USDOD, NMDPS, New Mexico State University | Coordinating incident response |
|  | NMDOT, Mesilla Valley Regional Dispatch Authority, City of Las Cruces, Dona Ana County, NMDPS, New Mexico State University, Municipalities, Contract Transport Companies. | Coordinate patient and care facility status, provide care facility status |
|  | City of Las Cruces and Dona Ana County Fire/ EMS | Provide signal preemption |
|  | City of Las Cruces and New Mexico State University Fire | Provide signal preemption |
|  | Dona Ana County and City of Las Cruces Fire/EMS | Provide signal preemption |
|  | Dona Ana County and New Mexico State University Fire | Provide signal preemption |
|  | NMDOT and Dona Ana County Fire/ EMS | Provide signal preemption |
|  | NMDOT and New Mexico State University Fire | Provide signal preemption |
|  | NMDOT and City of Las Cruces Fire/EMS | Provide signal preemption |
|  | NMDPS, Regional Public Safety Agencies, Regional Transit Agencies, Regional Traffic Agencies and NMDOT | Provide Amber Alert information |
|  | NMDOT, Mesilla Valley Regional Dispatch Authority, Elephant Butte Irrigation District, City of Las Cruces, Dona Ana County, NMDPS | Provide early warning information. |
|  | NMDPS, Regional Public Safety Agencies, NMDOT, New Mexico State University | Coordinating emergency plans for disasters |
|  | NMDPS, Regional Public Safety Agencies, NMDOT | Coordinating emergency plans for disasters, evacuations and reentry |
|  | Regional Public Safety Agencies, RoadRUNNER, Independent School Districts, Local Transit, New Mexico State University, South Central RTD, Southwest RTD | Coordinate transit emergency plans for evacuations and reentry |
| Roadway Service Patrols | NMDOT, Mesilla Valley Regional Dispatch Authority, NMDPS | Providing an incident report |
| Traffic Incident Management | City of Las Cruces, Mesilla Valley Regional Dispatch Authority, NMDPS, New Mexico State University | Sharing incident data and emergency resources |
|  | Dona Ana County, Mesilla Valley Regional Dispatch Authority, NMDPS | Sharing incident data and emergency resources |
|  | NMDOT, Mesilla Valley Regional Dispatch Authority, NMDPS, Contract Transport Companies, NMGSD | Sharing incident data and emergency resources |
|  | New Mexico State University, Mesilla Valley Regional Dispatch Authority, Contract Transport Companies, NMGSD | Sharing incident data and emergency resources |
|  | City of Las Cruces, Special Events Coordinators, New Mexico State University | Sharing event plans |
|  | Dona Ana County, Special Events Coordinators, New Mexico State University, City of Las Cruces | Sharing event plans |
|  | NMDOT, Special Events Coordinators, New Mexico State University, City of Las Cruces | Sharing event plans |
|  | Special Events Coordinators, New Mexico State University | Sharing event plans |
|  | City of Las Cruces, Mesilla Valley Regional Dispatch Authority, New Mexico State University, NMDPS, Dona Ana County | Sharing incident data and coordinating maintenance and construction resources |
|  | Dona Ana County, Mesilla Valley Regional Dispatch Authority, NMDPS, NMGSD, NMDOT | Sharing incident data and coordinating maintenance and construction resources |
|  | NMDOT, Mesilla Valley Regional Dispatch Authority, NMDPS, City of Las Cruces, Dona Ana County | Sharing incident data and coordinating maintenance and construction resources |
|  | Private Rail Operations, NMDPS, Mesilla Valley Regional Dispatch Authority | Sharing incident data and providing incident response |
|  | Private Rail Operations, City of Las Cruces, Dona Ana County, NMDOT | Obtain HRI advisories |
| Traffic Management | City of Las Cruces, Dona Ana County, NMDOT, New Mexico State University, Private Sector Traveler Information Services | Sharing road network conditions |
|  | NMDOT and the Media | Sharing traveler information |
|  | City of Las Cruces and the Media | Sharing traveler information |
|  | Dona Ana County and the Media | Sharing traveler information |
|  | NMDOT, RoadRUNNER Transit, South Central RTD, Southwest RTD, Local Transit Agencies | Sharing road network conditions |
|  | City of Las Cruces, RoadRUNNER Transit, South Central RTD, Local Transit Agencies | Sharing road network conditions |
|  | NMDOT, Dona Ana County, RoadRUNNER Transit, South Central RTD, Southwest RTD, Local Transit Agencies | Sharing road network conditions |
|  | NMDOT, Mesilla Valley Regional Dispatch Authority, NMDPS, USDOD, Private Ambulance Providers | Sharing road network conditions |
|  | City of Las Cruces, Mesilla Valley Regional Dispatch Authority, NMDPS, New Mexico State University, Private Ambulance Providers | Sharing road network conditions |
|  | Dona Ana County, Mesilla Valley Regional Dispatch Authority, NMDPS, Private Ambulance Providers | Sharing road network conditions |
|  | NMDOT, City of Las Cruces, Dona Ana County, New Mexico State University | Traffic information and control coordination |
|  | NMDOT, City of Las Cruces, Dona Ana County, New Mexico State University | Sharing parking lot information |
|  | NMDOT, City of Las Cruces, New Mexico State University | Sharing parking lot information |
|  | NMDOT and NM State Police | Sharing road network conditions, road closure information emergency traffic control information |
| Transit Management | RoadRUNNER Transit, City of Las Cruces, Dona Ana County, NMDOT | Sharing road network conditions |
|  | South Central RTD, City of Las Cruces, Dona Ana County, NMDOT | Sharing road network conditions |
|  | Southwest RTD, Dona Ana County, NMDOT | Sharing road network conditions |
|  | Local Transit Agencies, City of Las Cruces, Dona Ana County, NMDOT | Sharing road network conditions |
|  | NMDOT, RoadRUNNER Transit, South Central RTD, Southwest RTD, Local Transit Agencies | Sharing roadway maintenance work plans |
|  | City of Las Cruces, RoadRUNNER Transit, South Central RTD, Local Transit Agencies, NMDOT, Las Cruces Public School District | Sharing roadway maintenance work plans |
|  | Dona Ana County, RoadRUNNER Transit, South Central RTD, Southwest RTD, Local Transit Agencies, NMDOT | Sharing roadway maintenance work plans |
|  | NMDOT, Private Sector Traveler Information Services, RoadRUNNER Transit, South Central RTD, Local Transit Agencies | Sharing transit schedule and fare information |
|  | RoadRUNNER Transit, South Central RTD, Local Transit Agencies, Financial Institutions | Completing electronic payment transactions |
|  | MVRDA, RoadRUNNER Transit, South Central RTD, NMDOT, Local Transit Agencies, Independent School Districts | Sharing transit emergency data |
|  | RoadRUNNER Transit, South Central RTD, Southwest RTD, Local Transit Agencies, New Mexico State University, NMDOT | Transit service coordination |
|  | RoadRUNNER Transit, City of Las Cruces | Providing signal priority |
| Traveler Information | NMDOT, City of Las Cruces, Dona Ana County, Media | Sharing traveler information with the media |
|  | City of Las Cruces, RoadRUNNER Transit, NMDOT, South Central RTD, Local Transit Agencies | Providing transit fare and schedule information |
|  | Dona Ana County, RoadRUNNER Transit, NMDOT, South Central RTD, Southwest RTD, Local Transit Agencies | Providing transit fare and schedule information |
|  | NMDOT, RoadRUNNER Transit, South Central RTD, Southwest RTD, Local Transit Agencies | Providing transit fare and schedule information |
|  | City of Las Cruces, Dona Ana County, New Mexico State University, NMDOT | ISP Coordination |
|  | City of Las Cruces, Dona Ana County, NMDOT, Private Sector Traveler Information Services, RoadRUNNER Transit | ISP Coordination |
| Maintenance and Construction Management | City of Las Cruces, Dona Ana County, NMDOT, Mesilla Valley Regional Dispatch Authority | Share roadway maintenance status |
|  | City of Las Cruces, Dona Ana County, NMDOT | Coordinate maintenance and construction work plans |
|  | City of Las Cruces, Mesilla Valley Regional Dispatch Authority, NMDPS, New Mexico State University | Provide maintenance resources |
|  | Dona Ana County, Mesilla Valley Regional Dispatch Authority, NMDPS, NMGSD, New Mexico State University | Provide maintenance resources |
|  | NMDOT, Mesilla Valley Regional Dispatch Authority, NMDPS, New Mexico State University | Provide maintenance resources |
|  | City of Las Cruces, Dona Ana County, NMDOT, Public/Private Utilities, Mesilla Valley Regional Dispatch Authority, Las Cruces Public School District, RoadRUNNER Transit, Local Transit Agencies, South Central RTD | Provide maintenance and construction work plans and current asset restrictions |
|  | City of Las Cruces, Dona Ana County, NMDOT, Public/Private Utilities, Mesilla Valley Regional Dispatch Authority, NMDPS, RoadRUNNER Transit, Local Transit Agencies, South Central RTD, Southwest RTD | Provide maintenance and construction work plans and current asset restrictions |
|  | City of Las Cruces, Dona Ana County, NMDOT, Public/Private Utilities, Mesilla Valley Regional Dispatch Authority, NMDPS, RoadRUNNER Transit, Local Transit Agencies, South Central RTD, Southwest RTD, Texas DPS | Provide maintenance and construction work plans and current asset restrictions |
| Weather Distribution | NOAA, Private Weather Information Providers, City of Las Cruces | Sharing weather information |
|  | NOAA, Private Weather Information Providers, Dona Ana County | Sharing weather information |
|  | NOAA, Private Weather Information Providers, NMDOT | Sharing weather information |
|  | City of Las Cruces, Dona Ana County, NMDOT, Mesilla Valley Regional Dispatch Authority, RoadRUNNER Transit, South Central RTD, Local Transit Agencies, Las Cruces Public School District, Private Sector Traveler Information Services | Sharing road weather information |
|  | Dona Ana County, City of Las Cruces, NMDOT, Mesilla Valley Regional Dispatch Authority, RoadRUNNER Transit, South Central RTD, Local Transit Agencies, Private Sector Traveler Information Services | Sharing road weather information |
|  | NMDOT, City of Las Cruces, Dona Ana County, TxDOT, Mesilla Valley Regional Dispatch Authority, NMDPS, NMGSD, RoadRUNNER Transit, South Central RTD, Southwest RTD, Las Cruces Public School District, Private Sector Traveler Information Services | Sharing road weather information |

# Using the Las Cruces Regional ITS Architecture

The Las Cruces Regional ITS Architecture has been created, in part, to be used as a key reference in the transportation planning process.  This will ensure all proposed ITS projects are consistent with the regional ITS architecture and additional integration opportunities are considered, leading to more efficient implementations.

Planning processes are used to identify projects whose implementation will respond to regional needs.  These projects are placed in programming documents such as a Transportation Improvement Program (TIP) in order to secure funding for the projects.  Once funded, the projects are implemented.  The regional ITS architecture supports all three of these major steps – planning, programming, and implementation.

An important part of developing an ITS Architecture is establishing an approach to using it.  An ITS Architecture provides guidance for planning ITS projects within a region.  It also provides information that can be used in the initial stages of project definition and development.  This section presents the approach for integrating the ITS Architecture that has been updated for Las Cruces into the transportation planning/programming process and leveraging the ITS Architecture for project definition.  The approach facilitates and provides a mechanism for the projects identified in this document or in the ITS Implementation Plan to be planned and deployed in an orderly and integrated fashion.

The primary objective of the ITS Architecture is integration.  The integration of transportation systems to share information and coordinate activities provides significant benefits over the operation of systems in a stovepiped fashion.  The Las Cruces Regional ITS Architecture illustrates the information to be exchanged between transportation systems to meet the transportation needs of the stakeholders in the region (addressed in Section 6.1).

The ITS Architecture links the needs to ITS services and then to the ITS projects that address them.  The ITS Architecture was developed with these objectives in mind through the definition of ITS services or market packages.  By defining the ITS Architecture with services that address the needs, projects can be defined through the planning process using the architecture that addresses these needs through deployment.

## Using the Las Cruces Regional ITS Architecture in the Planning Process

One of the most important outcomes of the Las Cruces Regional ITS Architecture is that it will be used to plan and deploy ITS across the region.  To do this, the ITS Architecture must be integrated into the regional planning process.  As a result of integrating the ITS Architecture into the planning process, the architecture will link the needs of the regions with the ITS deployments in the field.

The goal of the planning process is to make quality, informed decisions pertaining to the investment of public funds for regional transportation systems and services.  Using the regional ITS architecture to support these planning activities is an important step in the mainstreaming of ITS into the traditional decision-making of planners and other transportation professionals.  Once an architecture is complete, it can feed detailed ITS-specific information back into the planning process.

Figure 13 shows some of the key steps in the transportation planning process.  These steps will be elaborated on in following sections.  The process is driven by a regional vision and set of goals.  These drive transportation improvement strategies that are a mix of capital improvements and operational improvements.  The planning organizations evaluate and prioritize the various strategies, and the resulting output is a document called the Long Range Transportation Plan (or sometimes Transportation Plan or Regional Transportation Plan).  This plan is the key output of long range planning.  For Las Cruces, the long range planning document is the current Transportation Plan for the Las Cruces Metropolitan Planning Organization.  This document contains the following:

- LRTP goals, objectives and policies.  The plan is divided into seven elements, containing a goal and a set of objectives/ policies for each element.   The seven elements of the plan are:

  - Intermodalism

  - Aviation/Aerospace

  - Bicycle

  - Pedestrian

  - Public Transportation

  - Rail

  - Thoroughfare

- For each element the plan describes the current situation or issues, evaluations related to the issues, and conclusions/ implementation.

The second document relevant to the planning process is the Las Cruces MOP Transportation Improvement Program (TIP) for 2008-2013.  Once a project is programmed then project development can begin.  All of these steps occur with a variety of critical factors and inputs as shown in the figure.  As shown in the figure, a regional ITS architecture may support each step in this process.

Regional Vision & Goals

Alternate Improvement

Strategies

Capital

Capital

Operational

Evaluation & Prioritization

Of Strategies

Development of Long-Range

Transportation Plan

Development of Transportation

Improvement Program

Project Development

System Operation

Budgets

Economic

Development

Public

Involvement

Environmental

Issues

Feedback

Critical Factors

& Inputs

Long Range Planning

Regional ITS

Architecture

Title IV

Figure 13.  ITS Architecture and the Transportation Planning Process

The Transportation Plan is the expression of Las Cruces’ long-range approach to planning and implementing the multimodal transportation system.  It is the policy forum for balancing transportation investments among modes, geographic areas, and institutions.

How can a Regional ITS Architecture support the transportation planning process?  In the following basic ways that will be expanded upon below:

- The services described in the regional ITS Architecture can provide the basis for operational strategies that can be used to improve the transportation system to meet the region’s vision and goals.

- The regional ITS architecture can be used to support evaluation and prioritization of strategies in two ways.  The first is through the definition in the architecture of archiving and data collection systems that support collecting the data needed for evaluation.  The second is through the detailed definition of ITS projects and their sequencing that can be used to support prioritization efforts.

- The definition of an integrated transportation system described by the regional ITS architecture can support the ITS elements of the Transportation Plan.

- The process of developing and maintaining a regional ITS architecture can help to enhance the linkage between operations and planning through closer involvement of a wider array of stakeholders from both of these areas of transportation.

The discussion below focuses on supporting the MPO transportation planning process, but the architecture can also be used to support the other planning processes used by agencies in the region (e.g.  public safety agencies).  As shown in Figure 14, agencies that do not use federal funds (or operate through the MPO planning process) will still have some form of long range plan and capital plan whose development can be supported by the regional ITS architecture.

Long Range

Transportation Plan (LRTP)

Transportation

Improvement

Program

(TIP)

ITS Architecture

ITS Related

Projects

MPO Planning Process

Funded ITS Related Projects

goals,

objectives,

policies

Strategic / Long

Range Plan

ITS Related

Projects

Other Agency

Planning Process

Capital Plan /

Budget

Figure 14.  Supporting the Transportation Planning Processes

The challenge for achieving integration across planned ITS projects in the regions is to know how they fit together and interact or depend on each other.  The regional ITS architecture can be leveraged to bridge the MPO processes to other agencies planning processes that do not use federal funding.  If all the processes are using the same reference point, the regional ITS architecture, then project integration can start in the planning phase.

### Operational Strategies

The Las Cruces planning process begins with a set of goals which are given in the Transportation Plan as:

- INTERMODALISM- Create a multi-modal transportation network for all transportation users within the MPO area. Preservation of the Physical and Social Environment

- AVIATION/AEROSPACE- To ensure airport facilities and services will maximize aviation/aerospace activities to its optimal potential

- BICYCLE FACILITIES- Advance the use of bicycles as a viable mode of transportation within the Las Cruces MPO area as well as encourage bicycling facilities to be incorporated into all transportation projects

- PEDESTRIAN FACILITIES- Provide pedestrian access to all areas within the Las Cruces MPO.

- PUBLIC TRANSPORTATION- Establish public transportation services that maximize transportation availability within the entire MPO area, as well as to all sectors of the population in a safe, dependable, and comfortable manner and that mitigates the need to use an automobile.

- RAIL- Enhance railroad service within the MPO to address the needs of both passenger and commercial service and provide a safe, viable alternative to traditional transportation modes.

- THOROUGHFARES- Achieve a system of thoroughfare corridors that maximizes traffic movement, system wide and intermodal connectivity, and mitigates vehicular congestion in a cost effective, timely, and environmentally sound manner.

These goals are then further defined by long range objectives.  As shown in Figure 13, the strategies are primarily capital improvements or operational improvements.  The Regional ITS Architecture can provide an array of potential operational improvements through the services that are defined in it.

Strategies that have traditional transportation projects as their primary solution, may add ITS elements or services as a part of the overall strategy solution.  For example, to reduce congestion, a corridor is planned for widening.  This project could also include incorporation of ITS elements and services to better manage the upgraded corridor.  The use of the architecture to support the definition of implementation strategies is assisted by the definition of needs and their mapping to ITS services given in Section 6.1.

### Strategy Evaluation and Prioritization

Transportation planners use a variety of tools to evaluate and prioritize the various strategies for transportation improvement.  Collection of data about the transportation system is a key input to support the evaluation.  Several ITS services, described in the regional ITS architecture can support this data collection, providing the planning organization the ability to measure non-recurring congestion, travel times and travel time reliability, and other aspects of system performance and service reliability across all modes.

The Las Cruces Regional ITS Architecture provides a guide for the archiving of transportation data including the collection of data from various ITS operational systems.  These archiving capabilities revolve around regional examples of the National ITS Architecture entity, Archive Data Management Subsystem (such as the Las Cruces MPO Data Warehouse).  Furthermore the Las Cruces Regional ITS Architecture has examples of ITS services such as ITS Data Mart (collection of historical data from a single source) and ITS Data Warehouse (collection of historical data from multiple aspects of transportation).  These examples of data archiving services describe connections and information that can be useful to planners in performing their evaluation and prioritization efforts.  While the automation of data collection for archiving is usually a future activity (or project), the discussions that occur during the development of the regional ITS architecture often present additional opportunities for data sharing that can occur immediately even before projects for the automation of data sharing are implemented.  This sharing of data between operations and planning, as well as the coordination that occurs in the development of the Regional ITS Architecture are examples of linkages between Planning and Operations.  The concept of linkage between these two disciplines figures prominently in the FHWA Planning for Operations effort ( see http://plan4operations.dot.gov/default.asp).  A complete treatment of these opportunities is included in the FHWA publication: Opportunities for Linking Planning and Operations.  This publication can be found at http://www.ops.fhwa.dot.gov/publications/lpo_ref_guide/index.htm.

FHWA has made available several software tools to support the evaluation and prioritization of ITS related strategies developed as part of the planning process.  These tools include:

- IDAS (ITS Deployment Analysis System) is a software tool developed by the Federal Highway Administration that can be used in planning for Intelligent Transportation System (ITS) deployments.  State, regional, and local planners can use IDAS to estimate the benefits and costs of ITS investments – which are either alternatives to or enhancements of traditional highway and transit infrastructure.  IDAS can currently predict relative costs and benefits for more than 60 types of ITS investments.  For more information see http://idas.camsys.com/.

- DYNASMART-P (Dynamic Network Assignment-Simulation Model for Advanced Roadway Telematics- Planning version) is a software tool developed for traffic operations planning applications under Federal Highway Administration’s Dynamic Traffic Assignment (DTA) research program.  DYNASMART-P combines (1) dynamic network assignment models, used primarily in conjunction with demand forecasting procedures for planning applications, and (2) traffic simulation models, used primarily for traffic operations studies.  For more information see http://www.dynasmart.umd.edu/.

- SCRITS (SCReening for ITS) is a spreadsheet analysis tool for estimating the user benefits of Intelligent Transportation Systems (ITS).  It is intended as a sketch-level or screening-level analysis tool for allowing practitioners to obtain an initial indication of the possible benefits of various ITS applications.  For more information see http://www.fhwa.dot.gov/steam/scrits.htm.

Regional ITS Architecture outputs, specifically the project sequencing output may also be useful to planning staff as an aid to evaluation and prioritization of strategies.  The architecture provides a short-term and long-term, multi-modal view of the ITS systems in the region.  It provides the details of the transportation services and functions that can be provided by the stakeholders via ITS projects.  The Regional ITS Architecture also illustrates the interfaces necessary between transportation systems to meet the transportation needs of the region.  Finally it translates these details to the definition of a set of projects to be implemented.  These projects are grouped by timeframe (e.g.  short term, medium term, long term).  Key to its usefulness for the strategy evaluation and prioritization (as well as for the LRP as discussed below) is that this list of projects goes well beyond the short term projects that get included in the TIP and STIP (see Section 12.2 for a discussion of how the architecture can be used in developing this program element.)  The project sequencing contains information for each project that may be useful to the evaluation or prioritization of the projects including:

  - Stakeholders

  - Cost

  - Mapping to ITS Services

Note that this project sequencing is not a replacement for prioritization, but rather, an input to the process by which projects are prioritized.  In some cases, prioritization may already have occurred and be reflected in the project sequencing outputs.

The Las Cruces Regional ITS Architecture provides a guide for how ITS projects can be deployed to satisfy the goals and objectives defined as part of the planning process.  The architecture provides the details on how ITS can be deployed to meet and satisfy the strategies and transportation services identified for the region.  These details may include the interfaces, the operational concepts and agreements necessary to implement the strategies and transportation services.  With these details, ITS projects can be more clearly defined, funded, and implemented to satisfy the regional goals.

### Long Range Transportation Plan

One of the primary motives for ITS, and the regional ITS architecture, is the need for more effective management of existing transportation infrastructure.  Many regions can no longer rely on adding capacity to keep pace with increasing demand.  Instead, they are relying on more effective, integrated management of the existing infrastructure.  Recognizing this need, many regions are beginning to include a section of the plan on “Management and Operations”, which can be defined as an integrated approach to optimize the performance of existing infrastructure through the implementation of multimodal, intermodal, and often cross-jurisdictional systems, services, and projects.  The recently passed law, “Safe, Accountable, Flexible, Efficient Transportation Equity Act: A Legacy for Users’’ or ‘‘SAFETEA–LU’’, emphasizes the need to include Management and Operations in each region’s Long Range Plan.  According to the legislation (Section 6001 (i)) metropolitan planning areas must include “operational and management strategies to improve the performance of existing transportation facilities to relieve vehicular congestion and maximize the safety and mobility of people and goods.” The Regional ITS Architecture can provide the technical underpinning to this portion of the LRTP.

One of the primary purposes of the Regional ITS Architecture is to define how an integrated transportation system (of ITS elements) might evolve in a region.  To do this, the architecture describes elements (e.g.  various ITS assets) that are interconnected to provide operations and management of the transportation system.  The architecture development and maintenance process provides an accessible way for transportation planners to become more focused on integrated management and operations.  Operations practitioners have a vision for how this integrated transportation system might evolve, and express this via the details of the architecture.

In the development of the Las Cruces Transportation Plan, some of the outputs of the regional ITS architecture can be of use in the plan update.  The outputs and some indication of their use are:

  - ITS Projects from the Regional ITS Architecture can provide an input to the Development of Future Needs and Implementation sections (as well as the TIP).

  - Customized Market Packages from the Las Cruces Regional ITS Architecture offer service-oriented slices of the architecture that facilitate project definition with an understanding of integration necessary to deliver specific services.  The market packages provide planners with insight into the ITS elements to include in a project, which will make the project as comprehensive as possible.  They also provide sample Implementation Strategies that include ITS to address long range objectives.

### Issues/Challenges

One issue to address in using the architecture to support planning is coordination of ITS project planning between the federally funded projects and the non-federally funded projects.  The non-federally funded projects may not be part of the Transportation Plan (and TIP).  The ITS Architecture contains systems and projects that bridge both federally and non-federally funded projects and systems.  Coordinating all of these projects requires an understanding by all stakeholders of the ITS systems and potential of the entire region.  The ITS Architecture provides a common reference point for all stakeholders to gain insight into the integration of the systems in the region.

### Recommendations

It is recommended that Las Cruces MPO, as the Metropolitan Planning Organization responsible for the Transportation Plan and TIP, designate an ITS Planner with responsibility for the application and monitoring of the ITS Architecture in the Transportation Planning Process. (this person would likely be assisted by the ITS Subcommittee).

The roles and responsibilities should include:

- Point of contact for tracking the incremental process of project implementation; projects are typically broken into phases, therefore, someone should be responsible for keeping track of completed and remaining phases and ensure the remaining components are reflected in the planning process for on-going funding support until project completion,

- Point of contact for ITS Architecture questions regarding its application in the planning process,

- Lead the evaluation of ITS (containing) projects for their compliance with the ITS Architecture,

- Outreach to stakeholders about how to use the ITS Architecture in the planning process,

- Provide feedback to the Maintenance Manager of the ITS Architecture (if this is a different person) on any ITS Architecture changes needed as a result of project planning,

- Liaison between MPO, state, and non-MPO planning organizations to share information about the ITS containing projects in the various planning processes and coordinate integration opportunities.

An additional recommendation is that the Regional ITS Architecture be formally adopted by Las Cruces MPO.  This will add “credibility” to architecture for use in planning process and will also encourage rigor in architecture maintenance process (described in Section 13).

## Using the Las Cruces Regional ITS Architecture for Programming

Regional and statewide programming and agency capital planning (a.k.a.  budgeting) involve identifying and prioritizing ITS projects, resulting in funded projects.  Using the regional ITS architecture to define an ITS project links the needs of the region identified in the architecture with the ITS deployed in the field.  In the region, ITS projects are deployed by many organizations including NMDOT, transit agencies and many local agencies and authorities.  If projects of the various organizations are defined from the same reference point, the Las Cruces Regional ITS Architecture, then coordination begins in the initial planning and funding phase.

ITS projects in the region may be funded by a variety of sources.  ITS projects that are funded with federal funds are programmed through the Las Cruces MPO planning process with input from member transportation agencies in the region.  All organizations in a region, whether they use federal funds to deploy ITS or not, perform short term planning via their capital planning (i.e.  budgeting) process.

### Architecture Use in Programming Federal Funds

The Transportation Improvement Program (TIP) is a staged, multiyear, intermodal program of transportation projects that is consistent with the long range transportation plan for the Las Cruces.  The TIP assigns federal funding to a prioritized list of specific projects to be constructed over a several-year period (usually three to six years) after the programs approval.

As discussed in the recommendations of Section 12.1.5, the regional ITS architecture can be adopted as part of or by reference in the Transportation Plan.  If this is the case, ITS projects should flow from the Transportation Plan into the TIP in the same fashion as capital projects.

The architecture can be used to define ITS projects that are submitted for federal funding.  In the current TIP, ITS projects are not defined in much detail.  Sometimes merely a placeholder for ITS projects is included.  A benefit to using a regional architecture to define ITS projects is that the projects can be specified in greater detail thereby allowing more realistic estimates of the ITS services being covered by the project.

When project sponsors submit ITS projects for programming, the planning agencies in some regions require that the sponsors submit architecture-related information about the project.  It is recommended that the Las Cruces MPO TIP project identification forms include specific reference to ITS Subcommittee review to ensure consistency with the ITS Architecture.  Some agencies merely require yes/no questions to be answered regarding the project’s inclusion in the regional architecture while others request more detailed information such as the elements, services, and/or interfaces of the architecture to be deployed in the project.  If an ITS project is submitted which is not included in or is not consistent with the regional architecture, a justification for the project should be required.  If it is justified, the impacted stakeholders support the project, and the project is funded, the regional architecture should be revised to incorporate the project.

In the TIP, projects that contain ITS elements should be designated as ITS projects so that projects sponsors are aware of the associated requirements from the FHWA/FTA Rule/Policy.

The programming process involves prioritizing projects and using federal funds to fund the top priority projects.  Each region has a process for prioritizing projects.  The regional ITS architecture can be useful in this process as it reflects the vision for ITS in the region so a factor in prioritization should be how well a project fits within the regional architecture.  In some regions, projects (of some categories) are allotted additional points if they include elements or interfaces of the architecture.

The Las Cruces Regional ITS Architecture may better support programming if the project sequencing and related architecture elements are updated on a maintenance schedule that supports the TIP cycle.  The architecture should be updated prior to a project submittal request so that it can be used by project sponsors to identify projects.  If thorough updates to the architecture are to correspond to Transportation Plan updates, less major revisions to the architecture can be made to correspond to the TIP cycle.

### Architecture Use in Organization Capital Planning

All agencies including Las Cruces MPO, NMDOT, transit agencies, local municipalities, etc.  use a budgeting process to allocate funds to projects.  The Las Cruces Regional ITS Architecture includes the existing and planned elements of all stakeholders and how they are or will be interfaced with other ITS elements in the region.  Therefore, all organizations can use the architecture to define ITS projects, as defined below, and feed them into their budgeting process.

Many ITS improvements are implemented as part of larger capital improvement projects.  As traditional capital projects are defined and programmed, it is important to identify the associated opportunities for efficient ITS implementation.  The Las Cruces Regional ITS Architecture is a record of the ITS implementation planned by each agency that can be used to identify these opportunities.  Some agencies in other states are considering policies to review each capital project to determine if ITS measures should be included before the project moves forward.  Many agencies do this type of review as good practice, but these opportunities would be identified more consistently and “carry more weight” if this check was formally defined and required by an established policy.

### Architecture Use to Identify and Define Projects

A regional ITS architecture includes a sequence of projects.  Projects from the architecture should feed into the programming and/or capital planning processes.  ITS projects are taken directly from the plan and submitted for funding (with Federal or other funds.)

As the projects defined in a regional ITS architecture are sequenced and have defined characteristics, organizations can use the architecture to define ITS projects to be submitted for funding from any source.

To obtain funding, a project sponsor must define a proposed ITS project.  The information contained in the Las Cruces Regional ITS Architecture can be used to define projects with more detail in order to better scope them and establish project budget requirements.

Some potential ways a regional ITS architecture can be used to define an ITS project include:

- Review the list of stakeholders to identify those that should be involved with the project and those that are or may be impacted.

- Use the stakeholder roles and responsibilities defined in the operational concept to clearly define the roles and responsibilities of the stakeholders involved in the project.

- Review the relevant service(s) (i.e.  market package(s)) to identify elements potentially directly or indirectly associated with the project and recognizing potential partners to share development costs, material and/or labor, facilities, etc.

- Use the defined interfaces between ITS elements (in the customized market packages)  to identify current and future integration opportunities.

- Utilize the sequence of projects to gain insight into the timelines and dependencies of a project with others including identifying opportunities to coordinate with capital projects.

- Apply the project description of the project sequencing including costs, technical feasibility, potential institutional issues and readiness to clearly define the project.

- Gain a thorough understanding of the elements and interfaces included in a project to more accurately estimate project budgets.

### Challenges

One critical issue is the coordination of Federally and non-Federally funded projects in the region.  In many regions, non-Federally funded projects are generally not included in the TIP.  The Regional ITS Architecture contains all ITS elements and projects within the scope of the architecture irrespective of funding source.  As the Las Cruces Regional ITS Architecture enables understanding and coordination of all ITS related projects for all stakeholders in the region, stakeholders can benefit from using an ITS Architecture to plan, program, and deploy all ITS projects not just those that are Federal funded and therefore, must meet the Rule/Policy requirements.

### Recommendations

Agencies that sponsor integration projects that are identified in the regional ITS architecture may face several hurdles.  Involvement by multiple agencies may add perceived risk to the project and the benefits from the project may be more regional in nature and difficult to quantify as hard savings for a particular agency.  The region can offset these hurdles by promoting integration projects to encourage support from individual agency project sponsors.  For example, the Maricopa County Area Governments (MAG) MPO (in Arizona) has a documented process for ITS project prioritization that favors ITS integration projects that are identified in the ITS strategic plan and support multiple agencies.  A prioritization process like this that favors integration projects with regional benefits can help to offset the hurdles faced by individual agency project sponsors.

To ensure ITS is deployed based on the regional vision contained in the Las Cruces Regional ITS Architecture, a process for local and regional planning and funding of projects based on the Las Cruces Regional ITS Architecture should be established.  Policies and procedures to incorporate ITS projects in the programming/budgeting processes could be set or a group of various ITS stakeholders could be assigned the responsibility for monitoring the deployment of ITS in the region.

## Using the Las Cruces Regional ITS Architecture in Project Definition

Projects that emerge from the planning process can benefit from the use of the Las Cruces Regional ITS Architecture in their definition and development.  Project implementation should follow a systems engineering process.  The ITS Architecture is most effective in the early phases of systems engineering processes.  Figure 15 shows a typical project implementation process for deploying ITS projects.

Figure 15.  Project Implementation Process

The project implementation process shown in Figure 15 is a systems engineering process.  It is a process that can be used to systematically deploy ITS while reducing the risks associated with deployments.  The Systems Engineering process is more than just steps in systems design and implementation; it is a life-cycle process.  The process recognizes that many projects are deployed incrementally and expand over time.  US DOT Rule 940 requires that aspects of this systems engineering process be used for ITS projects that are funded with federal funds.

Applying the System Engineering process to ITS project development is a key new requirement that must be addressed by stakeholders using federal funds.  The following are some key references that stakeholders can access to assist in using this process:

General Resources

- FHWA Systems Engineering Website (ops.fhwa.dot.gov/int_its_deployment/sys_eng.htm)

- International Council on Systems Engineering (www.incose.org)

Training

- Introduction to Systems Engineering for Advanced Transportation (NHI Course# 137024)

- Advanced Systems Engineering for Advanced Transportation Projects (CITE - www.citeconsortium.org/courses/syseng.html)

Publications

- Building Quality Intelligent Transportation Systems through Systems Engineering (FHWA-OP-02-046): www.itsdocs.fhwa.dot.gov/jpodocs/repts_te/13620.html  EDL# 13620

- Systems Engineering Guidebook for ITS (FHWA California Division/Caltrans): www.fhwa.dot.gov/cadiv/segb

- System Engineering for Intelligent Transportation Systems, An Introduction for Transportation Professionals: http://ops.fhwa.dot.gov/int_its_deployment/sys_eng.htm

There are similarities between the systems engineering process defined in Figure 15 and the project development process followed by agencies in the region.  A typical agency project development process is as follows:

- Project Selection

- Authorization to Proceed

- Project Definition

  - Purpose and Need

  - Project Scoping

  - Conceptual Design

- Project Design

  - Preliminary Plan Development

  - Semi-Final Plan Development

  - Final Plan Development

- Construction

  - Testing

- Operation and Maintenance

Table 16 shows the relationship this project development process has to the FHWA system engineering process.

Table 16.  Regional Project Development Process Relation to FHWA System Engineering Process

| Project Development Process | Relation | System Engineering Process |
| --- | --- | --- |
| Project Definition Purpose and Need Project Scoping Conceptual Design | => | Concept of Operations |
|  |  | High Level Requirements |
|  |  | Detailed Requirements |
| Project Design Preliminary Plan Development Semi-Final Plan Development Final Plan Development | => => | High Level Design |
|  |  | Detailed Design |
| Construction Testing | => | Implementation |
|  |  | Integration & Test |
|  |  | Subsystem Verification |
|  |  | System Verification |
| Operation and Maintenance | => | Operations & Maintenance |

The Las Cruces Regional ITS Architecture can be used to support development of the concept of operations, requirements and high level design steps in the systems engineering process.

In deployment of an ITS related project, the ITS Architecture should be used as the starting point for developing a project concept of operations (not to be confused with an Architecture’s Operational Concepts that define the roles and responsibilities of the Architecture’s stakeholders).  The concept of operations shows at a high level how the systems involved in a project operate in conjunction with the other systems of the region.  According to the NHI course “Introduction to Systems Engineering for Advanced Transportation”, a Concept of Operations includes the following information:

- Identification of stakeholders,

- Development of a vision for the project,

- Description of where the system(s) will be used,

- Description of organizational procedures or practices appropriate to the system(s), definition of critical performance parameters associated with the systems(s),

- Description of the utilization environment (conditions under which various parts of the system(s) will be used),

- Definition of performance measures used to evaluate the effectiveness of the system(s),

- Considerations of life cycle expectations, and

- Conditions under which the system(s) must operate (e.g.  environmental conditions).

The customized market package diagrams tailored by the regional stakeholders can also assist in definition of requirements for ITS systems involved in a specific project.  The ITS Architecture contains high level functional requirements for all ITS elements in the region.  These high level requirements can be the starting point for developing more detailed requirements.

The ITS Architecture can support high level system design.  The ITS architecture can be used by system designers to identify the ITS standards that are applicable for the interfaces included in the architecture.

While the above discussion relates the architecture to the general system engineering process, Rule 940 does have a specific set of system engineering analysis requirements that apply to all ITS projects that use funds from the Highway Trust Fund.  The required system engineering analysis steps are:

- Identification of portions of the regional ITS architecture being implemented (or if a regional ITS architecture does not exist, the applicable portions of the National ITS Architecture) – in this case the regional ITS architecture;

- Identification of participating agencies’ roles and responsibilities;

- Requirements definitions;

- Analysis of alternative system configurations and technology options to meet requirements;

- Procurement options;

- Identification of applicable ITS standards and testing procedures; and

- Procedures and resources necessary for operations and management of the system.

The Las Cruces Regional ITS Architecture provides inputs to a number of these steps as shown in Table 17.

Table 17.  System Engineering Requirements supported by Las Cruces Regional ITS Architecture

| System Engineering Requirements | Las Cruces Regional ITS Architecture Output |
| --- | --- |
| Identification of portions of the regional ITS architecture being implemented | Mapping project to the elements and interfaces of the regional ITS architecture |
| Identification of participating agencies’ roles and responsibilities (this relates to the Concept of Operations described earlier. | Use Operational Concept as a starting point |
| Requirements definitions | Use Functional Requirements as a starting point. |
| Identification of applicable ITS standards and testing procedures | Use regional architecture standards outputs as a starting point for the standards definition. |

How do you show compliance with the rule requirements?  Document with a System Engineering Review Form (SERF) how outputs meet requirements.  A SERF has been developed for New Mexico by the FHWA Division Office and can be obtained from the FHWA Division representative.

### Issues/Challenges

One of the challenges of using the ITS Architecture and the System’s Engineering process in the implementation of a project is educating stakeholders about the benefits of the process and the process itself.  The systems engineering process is not a new process to many organizations.  It may not be called the systems engineering process, but various stakeholders’ processes probably map to the systems engineering process very well (as shown in Table 16).  Making these types of linkages between processes makes it easier to incorporate the ITS Architecture as a tool in the process.

Another challenge is engaging a broader stakeholder base on a project when the ITS Architecture indicates that possibility.  The entire activity of seeking integration opportunities is more institutional than technical.  There will be instances where getting more stakeholders involved in a project will increase its complexity or cross jurisdictional boundaries that may not have been considered in the initial scope.  It is important to explore these integration opportunities so that, at the very least, they are accounted for and supported in the project design even though they may not be implemented with that specific project.  The ultimate goal is to make ITS deployment as economical as possible.  One way this can be accomplished is by deploying projects across institutional boundaries where different stakeholders get benefit from the ITS deployment.

### Recommendations

It is recommended that Las Cruces MPO, NMDOT, and other agencies involved in deploying ITS projects modify their project development/implementation processes to incorporate the use of the Las Cruces Regional ITS Architecture.  There is no standard definition of how or what process modifications are required – it is merely a function of each agency’s project development processes.  It is recommended that training then be developed and given to representatives of the key agencies to describe the impact of the Las Cruces Regional ITS Architecture on the project development process, and to describe the new requirements that have been placed on ITS project development when federal funds are used.  The process modifications should be distributed to stakeholders so they are aware of the steps to follow and are aware that this process is a necessary part of any project receiving federal funding.

It is also recommended that an individual or group be identified in each organization (Las Cruces MPO, NMDOT, etc.) to review project submittals and evaluate compliance with the Las Cruces Regional ITS Architecture.  It is important to work with the FHWA Division Office Representative in establishing a review process given that they will be involved in approval of projects with federal funding.  The generation of a checklist would make the evaluation more structured and facilitate a consistent approach to each project.

# Maintaining the Architecture

The Las Cruces Regional ITS Architecture is not a static set of outputs.  It must change as plans change, ITS projects are implemented, and the ITS needs and services evolve in the region.  This section describes the maintenance plan for maintaining the Las Cruces Regional ITS Architecture.  The plan covers the following four key areas:

- Who will be involved in the maintenance of the architecture?

- When will the architecture be updated?

- What will be maintained?

- How it will be maintained (i.e.  what configuration control process will be used)?

The Las Cruces Regional ITS Architecture is created as a consensus view of what ITS systems the stakeholders in the region have currently implemented and what systems they plan to implement in the future.  The Las Cruces Regional ITS Architecture will need to be updated to reflect changes resulting from project implementation or resulting from the planning process itself.  Types of changes may include:

- Changes for Project Definition.  When actually defined, a project may add, subtract or modify elements, interfaces, or information flows from the Las Cruces Regional ITS Architecture.  Because the Las Cruces Regional ITS Architecture is meant to describe the current (as well as future) regional implementation of ITS, it must be updated to correctly reflect how the developed projects integrate into the region.  Also, once projects are implemented, interfaces that were shown in the architecture as planned should now be changed to existing.

- Changes for Project Addition/Deletion.  Occasionally a project will be added or deleted through the planning process and some aspects of the Las Cruces Regional ITS Architecture that are associated with the project may be expanded, changed or removed.

- Changes in Project Priority.  Due to funding constraints, or other considerations, the planned project sequencing may change.  Delaying a project may have a ripple effect on other projects that depend on it.  Raising the priority for a project’s implementation may impact the priority of other projects that are dependent upon it.

- Changes in Regional Needs.  Transportation planning is done to address regional needs.  Over time these needs can change and the corresponding aspects of the Las Cruces Regional ITS Architecture that addresses these needs may need to be updated.

- Changes in other Regional/Statewide ITS Architectures.  Changes made in New Mexico Statewide ITS Architecture can affect the Las Cruces Regional ITS Architecture, necessitating changes to maintain consistency between the architectures.

- New Stakeholder: When new stakeholders come to the table, the Las Cruces Regional ITS Architecture will need be updated to reflect their place in the regional view of ITS elements, interfaces, and information flows.

- Changes or Evolution in ITS Standards applicable to ITS Projects in Las Cruces: The architecture maps ITS standards to interfaces (and hence to projects).  Over time, this mapping will need to be updated as standards release new versions, or as new standards are developed and matured.

Finally, the National ITS Architecture may be expanded and updated from time to time to include new user services or better define how existing elements satisfy the user services.  These changes should also be considered as the Las Cruces Regional ITS Architecture is updated.  The National ITS Architecture may have expanded to include a user service that has been discussed in a region, but not been included in the Las Cruces Regional ITS Architecture, or been included in only a very cursory manner.

## Roles and Responsibilities for Maintenance

Responsibility for maintenance of the Las Cruces Regional ITS Architecture lies with the Las Cruces Metropolitan Planning Organization, since they perform the primary regional planning organization functions, and will be one of the primary users of the architecture.  A group of core stakeholders, currently constituted as the ITS Steering Committee, will act as an “institutional framework” to review proposed changes to the architecture.  This group of core stakeholders is important because the Las Cruces Regional ITS Architecture is a consensus framework for integrating ITS systems.  As it was a consensus driven product in its initial creation, so it should remain a consensus driven product as it is maintained.  This section defines the stakeholders and their roles and responsibilities for the maintenance of the LasCruces Regional ITS Architecture.

### Definitions

The following groups or persons have a role in the maintenance of the architecture:

- Stakeholders.  Any government agency or private organization that has a role in providing transportation services in the region.

- Maintenance Working Group.  A group of stakeholder representatives who are responsible for the technical review of updates/changes to the Las Cruces Regional ITS Architecture, and for approving changes to go into the architecture.

- Responsible Agency.  The stakeholder agency with primary responsibility for maintenance of the architecture.

- Maintenance Manager.  The person responsible for overseeing and guiding the maintenance efforts.

### Stakeholders

Stakeholders are any government agency or private organization that is involved with or has an interest in providing transportation services in the region.  Each stakeholder owns, operates, and/or maintains one or more ITS element in the region and, therefore, should be included in architecture.

The success of the change management process outlined in this Maintenance Plan is highly dependent on the participation of the stakeholders identified in the architecture.  Without stakeholders participation in tracking the development of their ITS systems, and properly updating the architecture, the change management process will not succeed and the usefulness of the architecture will diminish over time.

The primary responsibility of the stakeholder agencies is to submit changes to the Las Cruces Regional ITS Architecture that are brought on by new plans or projects that are being planned or deployed for the stakeholder agency.  The stakeholder agency must submit the changes in the Las Cruces Regional ITS Architecture to the Maintenance Working Group.

If stakeholders desire more involvement in the architecture review process, they can get involved through voluntary participation in the Maintenance Working Group.

Each Stakeholder agency that is part of the Maintenance Working Group will designate an Authorized Representative who can sign off on architecture related changes for the agency.

### Maintenance Working Group

The Las Cruces Regional ITS Architecture Maintenance Working Group, or the Maintenance Working Group for short, has the following responsibilities:

- Collecting and compiling proposed changes and updates to the architecture from stakeholder agencies.

- Evaluating each proposed change from a technical standpoint, and reaching a consensus on the proposed change (this may require contacting additional stakeholders if one or more of their systems are affected).

- Approving changes to the architecture.

- Making any institutional or policy related decisions that arise in the maintenance of the architecture

The maintenance working group for the Las Cruces region will be the ITS Steering Committee that is already in existence.  In other words, representatives of stakeholder agencies who are represented in the Las Cruces Regional ITS Architecture are candidates for a voluntary maintenance working group.

The Maintenance Working Group (the ITS Steering Committee or subset of) will elect a Chairperson (and Vice-Chairperson in their absence) to conduct meetings.  The Chairperson is responsible for calling meetings, developing an agenda for meetings, and leading the meetings.  The Chairperson will be elected for a two-year term by a simple majority vote of a Working Group meeting where there is a quorum (2/3 of the member representatives present).  It might be prudent if the Chairperson of the ITS Steering Committee also serves as the Chairperson of the Maintenance Working Group.

### Responsible Agency

The Responsible Agency is the government agency that formally maintains the architecture.  The Responsible Agency assigns resources for making the physical changes to the architecture baseline, and for coordinating the maintenance of the architecture.  The Responsible Agency for the Las Cruces Regional ITS Architecture is Las Cruces MPO, since they are the transportation planning organization for the region, and will be primary users of the architecture.  While they have responsibility for the maintenance of the architecture, they may use other assets (such as NMDOT personnel) to perform the actual updates to the Turbo Architecture database.

### Maintenance Manager

The Las Cruces MPO will appoint a person to serve as Maintenance Manager to coordinate the maintenance activities of the Las Cruces Regional ITS Architecture.  The Maintenance Manager is the coordinator and main point of contact for all maintenance activities, including receiving Change Requests forms, tracking Change Requests, and distributing documentation.  This person will be the Champion defined in Section 4.1.

The Maintenance Manager has the following responsibilities:

- To ensure that ITS planning activities are included in the annual Transportation Plans and Programs, e.g., TIP, Transportation Plan

- Coordinate the activities of the Maintenance Working Group

- Receive Change Request forms and requests for documentation from stakeholders

- Distribute the baseline documents and outputs of the architectures to stakeholders.

- Maintain the “official” records of the Las Cruces Regional ITS Architecture, including the baseline documents, meeting minutes, the Change Request Database, and the list of Points of Contacts for the Stakeholder

- Ensure the status of each Change Request is properly updated in the Change Request Database

- Maintain a complete contact list of all stakeholders within the region.

Some of these responsibilities may be delegated to staff or consultants.

## Timetable for Maintenance

How often will the Las Cruces Regional ITS Architecture be modified or updated?  What events or timetable will be used for making updates or changes to the architecture?  There are two basic approaches that the region will utilize for maintaining the architecture:

- Periodic Maintenance.  Update the architecture based upon one of the recurring activities of the transportation planning process.  For example, it’s natural that the ITS architecture would be updated at the same frequency as the Transportation Plan  is updated (every five  years) or the Transportation Improvement Program is updated (at least every two years).  The update of the architecture will occur several months prior to the transportation planning document update, so that the revised architecture could serve as an input to the planning update.  Publication and versioning costs are minimized for the periodic maintenance approach since there is a new version only once in the maintenance cycle.

- Exception Maintenance.  This approach will be followed if there is an urgent need to make a change, or if a minor change is desired to address some stakeholder need.  In this case the change can be initiated as needed.  Publication and versioning costs are dependent on the frequency of changes made to the regional ITS architecture.

### Periodic Updates

A comprehensive architecture update will occur every two years, concurrent with the formal update of the TIP.  This is a natural result of the Las Cruces Regional ITS Architecture being a component of the regional transportation planning process.  The update is necessary to ensure that the architecture continues to accurately represent the regional view of ITS Systems.  The comprehensive update may include adding new stakeholders, reviewing transportation needs and services for the region, updating the status of projects, and reflecting new goals and strategies, as appropriate.  Operational concepts, system functional requirements, project sequencing, ITS standards, and list of agency agreements may also be updated at this time.

Between major updates of the architecture, the following interim update actions will be performed:

On an annual basis, the Maintenance Manager will actively solicit changes from each key stakeholder a set of needed updates.  The Maintenance Manager will contact the key stakeholders, via e-mail, written correspondence, or by telephone, and inquire if the stakeholder has any changes to the Las Cruces Regional ITS Architecture.  It is the responsibility of the stakeholders to complete and submit the Change Request Forms to the Maintenance Manager for consideration.  Within a defined period, the submitted Change Request Forms will be collected and reviewed by the Maintenance Working Group for consideration in the next minor update of the Las Cruces Regional ITS Architecture.

The Maintenance Plan will also be reviewed at the annual updates for required changes to the Maintenance Plan.  Use of the Las Cruces Regional ITS Architecture and modifications to it may differ from what was anticipated during the initial development of the Maintenance Plan.  Revising the Maintenance Plan will ensure that the change management process defined is effective.

### Event-Driven Updates

There are certain considerations listed above that may call for an event driven update to the architecture.  In this case a stakeholder may submit a Change Request Form to the Maintenance Manager and request that the Maintenance Working Group review and approve the change request prior to the next scheduled update of the Las Cruces Regional ITS Architecture.  This may be necessary if a stakeholder suddenly requires federal funding for a previously unplanned ITS project, and needs the ITS project to be included in the Las Cruces Regional ITS Architecture.

## Architecture Baseline

Establishing an architecture baseline requires clear identification of the architecture products to be maintained, including specific format and version information.  For the Las Cruces Regional ITS Architecture, the following are identified as the architecture baseline:

- Las Cruces Regional ITS Architecture Document (this document)

- Set of Customized Market Packages (Visio file)

- Turbo Architecture Database

- Las Cruces Regional ITS Architecture Web pages

- Change Database

- Stakeholder List

Regarding the Architecture documents, the original source document, in Microsoft Word format, will be held by the maintenance manager, while a PDF version of the documents will be available for general distribution.  In addition, a version number and date will be included inside the cover page.  Each document will use a versioning scheme that identifies the baseline and revision number.  For example, the initial release of the documents at the conclusion of this effort will be version 1.00.  Minor revisions will be 1.01, 1.02, etc.  The next major revision to the document will be version 2.00.  Regarding the set of customized market packages, the Visio file will be maintained by the maintenance manager.

Regarding the Turbo Architecture Database, the maintenance manager will maintain a zipped version of the final delivered Las Cruces Regional ITS Architecture database.  The name, date, and size of the database file inside the zipped file will be entered into an architecture log as version 1.0 of the architecture.

Regarding the web site, a CD-ROM version of the final web site will be maintained by the maintenance manager (this task may be shared/coordinated with the NMDOT).  The version number of the architecture will be clearly visible somewhere on the home page of the web site so that the version being viewed is immediately identifiable.

The Change database will be a Microsoft Access database with the version number in the name of the database.

The Stakeholder list will be a Microsoft Excel file with the version number in the name of the file.

## Change Management Process

This change management process specifies how changes are identified, how often changes will be made, and how the changes will be reviewed, implemented, and released.  The change management process involves five steps:

- Identify Change.  Review what changes are needed and complete and submit a Change Request Form.

- Evaluate and Review Change.  An initial evaluation of the Evaluate the change for completeness and consensus.  The Maintenance Working Group will then review the results of the evaluation and approve the change.

- Update Baseline.  Apply the approved changes to the Las Cruces Regional ITS Architecture documents.

- Notify Stakeholders.  Inform the stakeholders of the updated changes to the Las Cruces Regional ITS Architecture documents, and distribute the documents as necessary.

Stakeholder :

Identify changes and inform Authorized Representative

Authorized

Representative :

Create Change Request, signs off and sends to

Maintenance Manager.

,

Maintenance

Manager:

Gathers Change Requests and distributes to Maintenance Working Group

Maintenance Working  Group :

Evaluates Change Requests

Approve, reject, or request more information  on change

End

Maintenance

Manager :

Updates architecture and informs stakeholders.

More Information

The identification of who performs these steps is shown in Figure 16.

Figure 16.  Change Management Process

### Identify Change

This involves two issues:

- Who can identify a change to the architecture, and

- How will the change request be documented?

The question of who can make change requests is an important one.  If literally anyone can input requests, the region runs the risk of being overrun by requests that will tax scarce resources to review and deliberate the change request.  On the other end of the spectrum, if too much formality or paperwork is added to the process then many valid or needed changes may go unexpressed.

Any Stakeholder identified in the Las Cruces Regional ITS Architecture is allowed to submit Change Requests.  This effectively indicates that all changes have the approval of an existing, defined stakeholder in the ITS Architecture.  If the Change Request is to add a new Stakeholder and that Stakeholder’s ITS Elements and Interfaces, the Responsible Agency for the architecture must submit the Change Request.

A Change Request Form will be used to submit changes for review.  The Maintenance Change Request Form for the Las Cruces Regional ITS Architecture can be found in Appendix D.  The Maintenance Change Request Form includes the following infoation:

- Name of change

- Description of change

- Part of baseline affected (could be check boxes for document, database, web site, and not known)

- Rationale for change

- Originator name or agency

- Date of origination

This information entered on the Change Request Form will be added to a change database, maintained by the Responsible Agency.  The change database will include following additional fields of information:

- Change number (some unique identifier)

- Change disposition (accepted, rejected, deferred)

- Change type (minor or significant)

- Disposition comment

- Disposition date

- The Maintenance Working Group representatives who ere present at the determination of the disposition.

### Evaluate and Review the Change Request

Upon receiving a Change Request by the Maintenance Manager, an initial evaluation of the Change Request will be made for the impact to the overall architecture or the affected document.  The purpose of the evaluation is two-fold:

- Verify that the Change Request form and supporting materials is complete and correct

- Compare with other Change Request forms and determine if there are any conflicts

If the proposal for architecture modification has an impact on other stakeholders, the evaluator(s) will contact the Stakeholders to confirm their agreement with the modification.  All Stakeholders directly affected by the proposed change(s) must approve and sign-off the Change Request before the Maintenance Working Group considers the Change Request.

There are several options as to who performs the initial assessment, including:

- The Maintenance Manager

- Maintenance Working Group

- The person submitting the change

- A consultant, hired to support the maintenance activities of the architecture

Each of the above options has positive and negative implications, but the evaluator must have working knowledge of the architecture to evaluate the proposed changes.  The Maintenance Manager or the Maintenance Working Group will assign the evaluation option to use for each change request evaluation received.

Upon completing the initial assessment, the Change Request form will be reviewed by the Maintenance Working Group (either at a Maintenance Working Group meeting or via some electronic means).  Maintenance Working Group meetings are called by the Maintenance Manager (or their designated representative).

Maintenance Working Group meetings called by the Maintenance Manager will occur at least on an annual basis.  When calling the annual meeting, the Maintenance Manager will send a reminder to all Stakeholders to update their ITS Elements and Interfaces in the architecture, if necessary.  If sufficient Change Request Forms are submitted, the Maintenance Manager may call a Maintenance Working Group meeting at more frequent intervals to review the Change Request forms.  The Maintenance Manager will distribute copies of all Change Request Forms submitted and all supporting materials to all Stakeholders prior to the meeting for their review and assemble an agenda.  Maintenance Working Group meetings can also be requested by one of the stakeholders if there is an urgent need to update the architecture quickly.

The Maintenance Working Group will be provided sufficient time to review the Change Requests before the meeting.  During the meeting, the Maintenance Working Group will review the proposed changes and offer any comments.

After each Change Request is reviewed, if no further comments are offered by the Maintenance Working Group, the Change Request will be considered approved, and the Chairperson will sign off on the Change Request.

If additional comments are made that require action, those comments will be noted on the Change Request form.  Where comments (or changes required) are minor in nature they can be made by the submitter of the Change Request form, or by resources designated by the Maintenance Manager and the change considered approved.  In the case of major comments or changes to the Change Request, the approval of the change will be deferred until the next meeting of the Maintenance Working Group.

If a Change Request is to be withdrawn from consideration, the Chairperson or the Maintenance Manager will be required to sign-off on the Change Request Form to close out the Change Request.

At the end of the meeting, the Maintenance Working Group will agree if all the approved changes to the architecture necessitate an immediate update to the baseline, or whether the update should await either additional changes or the next major revision.  The decision should be based on the number of Change Requests approved and the nature of the approved changes.

Meeting Minutes (or notes) will be kept for all Maintenance Working Group meetings.  Minutes will include, at a minimum:

- an attendance list,

- comments made on each Change Request,

- and the disposition of each Change Request Form (Approved/Withdrawn/Deferred/Request More Information).

Minutes will be distributed to all members of the Maintenance Working Group meeting approximately 5 working days after the meeting.  Comments are due within 10 working days to the Maintenance Manager.  Approved minutes will be signed by the Chairperson and will be distributed to all Stakeholders and posted on the website.  The minutes provide a recording process for the change management process and provide traceability.

The Maintenance Working Group will have the option to handling the review and approval process for minor Change Requests via e-mail exclusively rather than through face to face meetings.

### Update Baseline

Upon approvals of the Change Request Forms, the decision agreed upon by the Maintenance Working Group is implemented.  If the decision is to accept the change and update the baseline then the appropriate portions of the architecture baseline are updated and an updated architecture baseline is defined.  In addition to updating the baseline documents, databases, or other outputs, the configuration status will be updated.  In the discipline of Configuration Management this is known as Configuration Status Accounting.  This accounting is performed by having a document  (the Configuration Status Report) that defines the following information for each separate output of the architecture baseline:

- Output name;

- Output revision number;

- Date of latest revision;

- File Name; and

- Location/Point of Contact.

Periodically, the information in the various outputs of the architecture baseline will be audited to assure that the different representations of the architecture information (e.g.  the database and document) are in sync.  This configuration auditing will be performed by the chair of the ITS Committee at least every two years.

### Update of the Turbo Architecture Database

The updates to the Turbo Architecture database require a knowledge of the tool as well as the National ITS Architecture on which it is based.  This knowledge may reside with the Responsible Agency, either with agency staff or with consultants contracted to support the maintenance effort, but the plan is that the Responsible Agency delegate this update responsibility to the NMDOT (or any private consultant they might hire) to perform the updates.  In order to simplify the overall architecture maintenance process throughout New Mexico, NMDOT will create a single turbo architecture database containing not only the Las Cruces Regional ITS Architecture but also the New Mexico Statewide ITS Architecture and all the other regional ITS architectures in the state (e.g.  Santa Fe, Las Cruces, and Farmington).  The update of the actual Turbo Architecture Database would be done by NMDOT as directed by the Responsible Agency (as a result of the maintenance process described in this document).  The Las Cruces Regional ITS Architecture Turbo Architecture database will be updated by NMDOT in response to approved changes from the Regional ITS Architecture maintenance processes.  The Responsible Agency will receive from NMDOT a separate version controlled Turbo Architecture Database that contains only the Las Cruces Regional ITS Architecture.  This database would be available for use by regional stakeholders in ITS project development efforts.

### Notify Stakeholders

Point of Contacts for each stakeholder will be notified by e-mail from the Maintenance Manager when baseline documents have been updated.  All baseline documents will also be available to stakeholders from a website or other electronic location, such as an ftp site.  It is the responsibility of the Maintenance Manager to ensure the most recent document is available from the website.  The Configuration Status Document will be one of those outputs that are available.

Request for copies or access to the baseline documents will be made to the Maintenance Manager.

After major revisions to the architecture or the baseline documents, the Maintenance Working Group may elect to also provide all baseline documents to members on CD-ROMs, or secured-access files on the Architecture website.

Appendix A

Acronyms

| Acronym | Definition |
| --- | --- |
| AASHTO | American Association of State Highway and Transportation Officials |
| AAMVA | American Association of Motor Vehicle Administrators |
| ACE | Automated Commercial Environment |
| ADA | Americans with Disabilities Act |
| ADMS | Archived Data Management Subsystem |
| ADUS | Archived Data User Service |
| AID | Architecture Interconnect Diagram |
| ANSI | American National Standards Institute |
| APTA | American Public Transportation Association |
| APTS | Advanced Public Transportation System |
| ASTM | American Society for Testing and Materials |
| ATIS | Advanced Traveler Information System |
| ATMS | Advanced Traffic Management System |
| ATS | Automatic Train Stop |
| AVCS | Advanced Vehicle Control System |
| AVI | Automated Vehicle Identification |
| AVL | Automated Vehicle Location |
| BIA | Bureau of Indian Affairs |
| BRT | Bus Rapid Transit |
| C2C | Center to Center |
| C2F | Center to Field |
| CCTV | Closed Circuit TV |
| CFR | Code of Federal Regulations |
| CV | Commercial Vehicle |
| CVAS | Commercial Vehicle Administration Subsystem |
| CVCS | Commercial Vehicle Check Subsystem |
| CVIEW | Commercial Vehicle Information Exchange Window |
| CVISN | Commercial Vehicle Information Systems and Networks |
| CVO | Commercial Vehicle Operations |
| CVS | Commercial Vehicle Subsystem |
| DHS | Department of Homeland Security |
| DMS | Dynamic Message Sign |
| DMV | Department of Motor Vehicles |
| DOT | Department of Transportation |
| DPS | Department of Public Safety |
| DRE | Disaster Response and Evacuation |
| DSRC | Dedicated Short Range Communications |
| E9-1-1 | Enhanced 9-1-1 |
| EM | Emergency Management Subsystem |
| EMC | Emergency Management Center |
| EMMS | Emissions Management Subsystem |
| EMS | Emergency Medical Services |
| EOC | Emergency Operations Center |
| ETA | Expected Time of Arrival |
| EVS | Emergency Vehicle Subsystem |
| FAST | Free and Secure Trade |
| FHWA | Federal Highway Administration |
| FMCSA | Federal Motor Carrier Safety Administration |
| FMS | Fleet and Freight Management Subsystem |
| FTA | Federal Transit Administration |
| FTP | File Transfer Protocol |
| GIS | Geographic Information System |
| GPS | Global Positioning System |
| HAR | Highway Advisory Radio |
| HAZMAT | HAZardous MATerial(s) |
| HMMS | Highway Maintenance Management System |
| HOT | High Occupancy Toll |
| HOV | High Occupancy Vehicle |
| HRI | Highway Rail Intersection |
| HSAS | Homeland Security Advisory System |
| HTML | Hypertext Markup Language |
| HTTP | Hypertext Transfer Protocol |
| IEEE | Institute of Electrical and Electronics Engineers, Inc. |
| IFTA | International Fuel Tax Agreement |
| IRP | International Registration Plan |
| ISAC | Information Sharing and Analysis Center |
| ISO | International Standards Organization |
| ISP | Information Service Provider |
| ITE | Institute of Transportation Engineers |
| ITS | Intelligent Transportation Systems |
| MCMS | Maintenance and Construction Subsystem |
| MCO | Maintenance and Construction Operations |
| MCVS | Maintenance and Construction Vehicle Subsystem |
| MDT | Mobile Data Terminal |
| MPH | Miles per Hour |
| MPO | Metropolitan Planning Organization |
| Las Cruces MPO | Mid Region Council of Governments |
| MS/ETMCC | Message Set for External TMC Communication |
| MTD | Motor Transportation Division |
| NEMA | National Electrical Manufacturers Association |
| NIPC | National Infrastructure Protection Center |
| NTCIP | National Transportation Communications for ITS Protocol |
| OS/OW | Oversize/ Overweight |
| PDA | Personal Digital Assistant |
| PIAS | Personal Information Access Subsystem |
| PIN | Personal Identification Number |
| PIO | Public Information Office |
| PMS | Parking Management Subsystem |
| PRISM | Performance and Registration Information Systems Management |
| PSAP | Public Safety Answering Point |
| RCB | Radio Communications Bureau |
| RECC | Regional Emergency Communications Center |
| RPO | Regional Planning Organization |
| RS | Roadway Subsystem |
| RTD | Regional Transit District |
| RTS | Remote Traveler Support Subsystem |
| SAE | Society of Automotive Engineers |
| SAFER | Safety and Fitness Electronic Records |
| SDO | Standards Development Organization |
| SERPO | Southeast Regional Planning Organization |
| SMS | Security Monitoring Subsystem |
| STIP | State Transportation Improvement Program |
| TAS | Toll Administration Subsystem |
| TCIP | Transit Communications Interface Profiles |
| TCP | Transport Control Protocol |
| TCS | Toll Collection Subsystem |
| TDM | Travel Demand Management |
| TEA-21 | Transportation Efficiency Act for the 21st Century |
| TMC | Traffic Management Center |
| TMDD | Traffic Management Data Dictionary |
| TMS | Traffic Management Subsystem |
| TOC | Traffic Operations Center |
| TRMC | Transit Management Center |
| TRMS | Transit Management Subsystem |
| TRVS | Transit Vehicle Subsystem |
| USCBP | US Customs and Border Protection |
| USDOT | United States Department of Transportation |
| USR | User Service Requirement |
| VMS | Variable Message Sign |
| VS | Vehicle Subsystem |
| WAA | Wide Area Alert |
| WAN | Wide Area Network |
| WIM | Weigh-in Motion |
| WWW | World Wide Web |

Appendix B

Comments and Disposition

| No. | Comment | Source | Date | Completed | Disposition |
| --- | --- | --- | --- | --- | --- |
| 1 | In the “New Mexico State University” information, there are two periods at the end of the sentence. All references to “NMSU Police Dispatch” should be changed to “NMSU Emergency Dispatch Center”, as this 911 facility also dispatches fire, EMS, and public utilities. | Stephen Lopez, Deputy Chief of Police, NMSU Police Department | 06/11/07 | 08/30/07 | Changed NMSU Police Dispatch to NMSU Emergency Dispatch. |
| 2 | NMSU also maintains parking services, transit services, CCTV, speed and traffic monitoring equipment, and dynamic message signs. None of these are listed in the inventory. At the same time, market packages for these are not listed. | Stephen Lopez, Deputy Chief of Police, NMSU Police Department | 06/11/07 | 08/30/07 | Added the following elements: Aggie Shuttle Dispatch, Aggie Shuttle Vehicles, NMSU Equipment Repair Facility, NMSU Fire Vehicles, NMSU ITS Field Equipment, NMSU Maintenance and Construction Vehicles, NMSU Office of Facilities Operation and Utilities, NMSU Parking System, NMSU Traffic Operations Center, and NMSU Website. |
| 3 | In a number of areas, the information is listed as “UNM” rather than “NMSU”. In addition, the items that need to be added to the inventory need to be reflected here, as well. The following is a breakdown of the market packages that should be listed for NMSU: - AD1 (operational); APTS1 (planned for future); APTS2 (operational); APTS5 (operational); APTS6 (operational); APTS7 (planned for future); APTS8 (planned for future); ATIS1 (planned for future); ATIS2 (planned for future); ATMS06 (planned for future); ATMS07 (planned for future); ATMS08 (planned for future); ATMS16 (operational); EM01 (operational); EM02 (planned for future); EM04 (operational); EM05 (operational); EM06 (operational); EM07 (operational); EM08 (operational); EM09 (operational); EM10 (operational); MC01 (planned); MC02 (operational); MC06 (operational); MC07 (operational); MC08 (operational) | Stephen Lopez, Deputy Chief of Police, NMSU Police Department | 06/11/07 | 08/30/07 | The corrections from UNM to NMSU has been made. The following market packages were added for NMSU: AD1. AD2, APTS1, APTS2, APTS5, APTS6, APTS7, APTS8, ATIS1, ATMS01, ATMS06, ATMS07, ATMS08, ATMS16, EM01, EM02, EM06, EM08, EM09, EM10, MC01, MC02, MC06, MC07, MC08, and MC10. |
| 4 | Double-check the names of the NMSU systems and centers. | Patrick Chan, ConSysTec | 08/02/07 | 08/30/07 | Note: NMSU do not actually have a dispatch component for the transit service at this time, as they only operate fixed routes. NMSU has asked the City of Las Cruces into taking over the bus service operations under contract in 2008 or 2009. |
| 5 | Created a generic system, called NMSU ITS Field Equipment, which represent field devices, such as dynamic message signs, CCTV cameras, traffic monitoring equipment, etc. | Patrick Chan, ConSysTec | 08/02/07 | 08/30/07 | Checked with Stakeholder and no objections to the element. |
| 6 | Verify NMSU has its own maintenance group. | Patrick Chan, ConSysTec | 08/02/07 | 08/30/07 | NMSU maintains its own full-time construction crew. However, it is considering contracting maintenance to outside contractors. |
| 7 | Check if there are any transfer points between NMSU’s transit service and other transit agencies. | Patrick Chan, ConSysTec | 08/02/07 | 08/30/07 | There is a "park and ride" service operated by NMDOT that utilizes one of the NMSU parking lots, and theoretically creates a tie-in between their transit service and NMSU’s transit service. |
| 8 | Is there a City of Las Cruces 911/Emergency Communications Center? I'm assuming this is different from the MVRDA. The note says it dispatches MC vehicles after hours. | Patrick Chan, ConSysTec | 08/02/07 | 08/30/07 | No, MVRDA is the only 911/Emergency Communications Center for Las Cruces. |
| 9 | Related to RoadRUNNER, ATMS16-1 - Does City of Las Cruces have a parking management system or parking kiosks? Similarly, are there NMDOT Park and Ride facilities in the region (ATMS16-2)? | Patrick Chan, ConSysTec | 08/02/07 | 08/30/07 | RoadRUNNER Transit would foresee a transit info kiosk and the new intermodal center and any minor transfer centers such as the Mall. |
| 10 | NMDOT District 1: ATMS05-1: - Is HOV something NMDOT D1 is considering? MC05-1 - Is road icing a concern for NMDOT D1? | Patrick Chan, ConSysTec | 08/02/07 | 08/30/07 | HOV was considered on I10 GRIP project during Phase A Corridor study but did not make Phase B. Road icing is a big concern to D1 (snowplowing). |

Appendix C

Functional Requirements

This appendix contains two tables describing the functions assigned to each of the major elements of the Las Cruces Regional ITS Architecture.  Table 18 lists the functions (defined from the Turbo Architecture Database as Equipment Packages) assigned to each Element.  Table 19 provides a definition of each of the functions defined in Table 18.  In addition, these functions are further decomposed into a detailed listing of the functional requirements (defined as a set of “shall” statements) in a separate Excel spreadsheet.

Table 18.  Functions (Equipment Packages) Assigned to Architecture Elements

| Element | Equipment Packages |
| --- | --- |
| Aggie Shuttle Dispatch | Center Secure Area Alarm Support, Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Data Collection, Emergency Early Warning System, Emergency Evacuation Support, Emergency Response Management, Transit Center Fixed-Route Operations, Transit Center Information Services, Transit Center Multi-Modal Coordination, Transit Center Security, Transit Center Vehicle Tracking, Transit Data Collection, Transit Evacuation Support, Transit Garage Maintenance, Transit Vehicle Operator Scheduling |
| Aggie Shuttle Vehicles | On-board Fixed Route Schedule Management, On-board Maintenance, On-board Transit Security, On-board Transit Signal Priority, On-board Transit Trip Monitoring |
| City of Las Cruces Animal Control Vehicles | On-board EV Incident Management Communication |
| City of Las Cruces CCTV | Roadway Basic Surveillance, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Incident Detection, Roadway Infrastructure Monitoring, Roadway Signal Controls, Roadway Work Zone Traffic Control |
| City of Las Cruces DMS | Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Traffic Information Dissemination, Roadway Work Zone Traffic Control |
| City of Las Cruces Field Sensors | Roadway Basic Surveillance, Roadway Data Collection, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring |
| City of Las Cruces Fire and EMS Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| City of Las Cruces ITS Field Equipment | Field Barrier System Control, Field Safeguard System Control, Field Secure Area Sensor Monitoring, Field Secure Area Surveillance, Roadway Data Collection, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Reversible Lanes, Roadway Work Zone Safety |
| City of Las Cruces Maintenance and Construction Vehicles | MCV Barrier System Control, MCV Infrastructure Monitoring, MCV Roadway Maintenance and Construction, MCV Vehicle Location Tracking, MCV Vehicle Safety Monitoring, MCV Vehicle System Monitoring and Diagnostics, MCV Winter Maintenance, MCV Work Zone Support |
| City of Las Cruces Police Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| City of Las Cruces Public Information System | Basic Information Broadcast, Interactive Infrastructure Information, ISP Emergency Traveler Information, ISP Traveler Data Collection, Traveler Telephone Information |
| City of Las Cruces Public Works Dispatch | MCM Environmental Information Collection, MCM Incident Management, MCM Maintenance Decision Support, MCM Roadway Maintenance and Construction, MCM Vehicle and Equipment Maintenance Management, MCM Vehicle Tracking, MCM Winter Maintenance Management, MCM Work Activity Coordination, MCM Work Zone Management, MCM Work Zone Safety Management |
| City of Las Cruces RWIS | Roadway Environmental Monitoring, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Probe Beacons |
| City of Las Cruces Traffic Archive | Government Reporting Systems Support, ITS Data Repository, Traffic and Roadside Data Archival |
| City of Las Cruces Traffic Operations Center | Barrier System Management, Center Secure Area Sensor Management, Center Secure Area Surveillance, Collect Traffic Surveillance, Emergency Data Collection, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Emergency Routing, HRI Traffic Management, Incident Command, MCM Data Collection, MCM Environmental Information Collection, MCM Environmental Information Processing, MCM Incident Management, MCM Maintenance Decision Support, MCM Roadway Maintenance and Construction, MCM Winter Maintenance Management, MCM Work Activity Coordination, MCM Work Zone Management, Safeguard System Management, TMC Environmental Monitoring, TMC Evacuation Support, TMC Freeway Management, TMC Incident Detection, TMC Incident Dispatch Coordination/Communication, TMC Multimodal Coordination, TMC Probe Information Collection, TMC Regional Traffic Control, TMC Reversible Lane Management, TMC Signal Control, TMC Traffic Information Dissemination, TMC Work Zone Traffic Management, Traffic Data Collection, Traffic Maintenance |
| City of Las Cruces Traffic Signals | Roadway Basic Surveillance, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Signal Controls, Roadway Signal Priority, Standard Rail Crossing |
| City of Las Cruces Website | Basic Information Broadcast, Interactive Infrastructure Information, ISP Traveler Data Collection, Traveler Telephone Information |
| Dona Ana County Animal Control Vehicles | On-board EV Incident Management Communication |
| Dona Ana County CCTV | Roadway Basic Surveillance, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Incident Detection, Roadway Infrastructure Monitoring, Roadway Signal Controls |
| Dona Ana County DMS | Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Traffic Information Dissemination |
| Dona Ana County Field Sensors | Roadway Basic Surveillance, Roadway Data Collection, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring |
| Dona Ana County Fire and EMS Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| Dona Ana County Flood Commission Water Level Monitoring Systems | Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Early Warning System, Emergency Environmental Monitoring |
| Dona Ana County ITS Field Equipment | Roadway Data Collection, Roadway Environmental Monitoring, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Probe Beacons, Roadway Reversible Lanes, Roadway Signal Priority, Roadway Work Zone Safety, Roadway Work Zone Traffic Control |
| Dona Ana County Public Information System | Basic Information Broadcast, Interactive Infrastructure Information, ISP Emergency Traveler Information, ISP Traveler Data Collection, Traveler Telephone Information |
| Dona Ana County Public Works Dispatch | MCM Environmental Information Collection, MCM Incident Management, MCM Maintenance Decision Support, MCM Roadway Maintenance and Construction, MCM Vehicle and Equipment Maintenance Management, MCM Vehicle Tracking, MCM Winter Maintenance Management, MCM Work Activity Coordination, MCM Work Zone Management, MCM Work Zone Safety Management |
| Dona Ana County Public Works Vehicles | MCV Barrier System Control, MCV Infrastructure Monitoring, MCV Roadway Maintenance and Construction, MCV Vehicle Location Tracking, MCV Vehicle Safety Monitoring, MCV Vehicle System Monitoring and Diagnostics, MCV Winter Maintenance, MCV Work Zone Support |
| Dona Ana County Sheriff Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| Dona Ana County Traffic Operations Center | Collect Traffic Surveillance, HRI Traffic Management, MCM Data Collection, MCM Environmental Information Processing, MCM Incident Management, MCM Maintenance Decision Support, MCM Roadway Maintenance and Construction, MCM Winter Maintenance Management, MCM Work Activity Coordination, MCM Work Zone Management, TMC Environmental Monitoring, TMC Evacuation Support, TMC Freeway Management, TMC Incident Detection, TMC Incident Dispatch Coordination/Communication, TMC Regional Traffic Control, TMC Reversible Lane Management, TMC Signal Control, TMC Traffic Information Dissemination, TMC Work Zone Traffic Management, Traffic Data Collection, Traffic Maintenance |
| Dona Ana County Traffic Planning System | Government Reporting Systems Support, ITS Data Repository, Traffic and Roadside Data Archival |
| Dona Ana County Traffic Signals | Roadway Basic Surveillance, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Signal Controls, Standard Rail Crossing |
| Dona Ana County Website | Basic Information Broadcast, Interactive Infrastructure Information, ISP Traveler Data Collection, Traveler Telephone Information |
| Dona Ana County/City of Las Cruces Emergency Operations Center (EOC) | Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Call-Taking, Emergency Dispatch, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Incident Command |
| EBID Water Level Monitoring Systems | Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Early Warning System, Emergency Environmental Monitoring |
| Gadsden Ind/Hatch Valley Municipal School District Buses | On-board Fixed Route Schedule Management, On-board Maintenance, On-board Transit Security, On-board Transit Trip Monitoring |
| Gadsden Ind/Hatch Valley Municipal School District Dispatch | Center Secure Area Alarm Support, Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Transit Center Fixed-Route Operations, Transit Center Information Services, Transit Center Security, Transit Center Vehicle Tracking, Transit Evacuation Support, Transit Garage Maintenance, Transit Vehicle Operator Scheduling |
| Las Cruces MPO Data Warehouse | Government Reporting Systems Support, ITS Data Repository, On-Line Analysis and Mining, Traffic and Roadside Data Archival |
| Las Cruces MPO Traffic Monitoring Program | Government Reporting Systems Support, ITS Data Repository, On-Line Analysis and Mining, Traffic and Roadside Data Archival |
| Las Cruces Public School District Buses | On-board Fixed Route Schedule Management, On-board Maintenance, On-board Transit Security, On-board Transit Trip Monitoring |
| Las Cruces Public School District Dispatch | Center Secure Area Alarm Support, Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Transit Center Fixed-Route Operations, Transit Center Information Services, Transit Center Multi-Modal Coordination, Transit Center Security, Transit Center Vehicle Tracking, Transit Environmental Monitoring, Transit Evacuation Support, Transit Garage Maintenance, Transit Vehicle Operator Scheduling |
| Local Transit Dispatch | Center Secure Area Alarm Support, Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Data Collection, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Transit Center Fare and Load Management, Transit Center Fixed-Route Operations, Transit Center Information Services, Transit Center Multi-Modal Coordination, Transit Center Paratransit Operations, Transit Center Security, Transit Center Vehicle Tracking, Transit Data Collection, Transit Environmental Monitoring, Transit Evacuation Support, Transit Garage Maintenance, Transit Vehicle Operator Scheduling |
| Local Transit Vehicles | On-board Fixed Route Schedule Management, On-board Maintenance, On-board Paratransit Operations, On-board Transit Fare and Load Management, On-board Transit Security, On-board Transit Signal Priority, On-board Transit Trip Monitoring |
| Local Transit Website | Basic Information Broadcast, Infrastructure Provided Trip Planning, Interactive Infrastructure Information, ISP Traveler Data Collection |
| Military Installations Operations Office | TMC Traffic Information Dissemination |
| Municipal Emergency Operations Centers (EOC) | Emergency Evacuation Support, Emergency Response Management, Incident Command |
| Municipal Fire and EMS Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| Municipal Police Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| MVRDA Communications Center | Center Secure Area Alarm Support, Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Call-Taking, Emergency Data Collection, Emergency Dispatch, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Emergency Routing, Incident Command, Service Patrol Management |
| MVRDA Reverse 911 System | ISP Emergency Traveler Information, ISP Traveler Data Collection, Traveler Telephone Information |
| New Mexico National Guard Command System | Emergency Early Warning System, Emergency Evacuation Support, Emergency Response Management, Incident Command |
| New Mexico State Radio Communications Bureau (RCB) | Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Incident Command |
| New Mexico Statewide Emergency Operations Center (EOC) | Emergency Call-Taking, Emergency Dispatch, Emergency Evacuation Support, Emergency Response Management, Incident Command |
| NM DPS District 4 Dispatch Center | Center Secure Area Alarm Support, Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Call-Taking, Emergency Data Collection, Emergency Dispatch, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Emergency Routing, Incident Command, Service Patrol Management |
| NM DPS Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| NM Motor Transport Division (MTD) District Offices | Emergency Call-Taking, Emergency Data Collection, Emergency Dispatch, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Emergency Routing |
| NM Motor Transport Division (MTD) Response Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| NMDOT Advanced Traveler Information System | Basic Information Broadcast, Infrastructure Provided Trip Planning, Interactive Infrastructure Information, ISP Traveler Data Collection, Traveler Telephone Information |
| NMDOT Automatic Traffic Recording Devices | Roadway Data Collection, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring |
| NMDOT CCTV | Field Barrier System Control, Roadway Basic Surveillance, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Incident Detection, Roadway Infrastructure Monitoring, Roadway Work Zone Traffic Control |
| NMDOT Consolidated Highway Database (CHDB) | Government Reporting Systems Support, ITS Data Repository, On-Line Analysis and Mining |
| NMDOT District 1 HELP Courtesy Patrol Dispatch | Traffic and Roadside Data Archival, Emergency Call-Taking, Emergency Data Collection, Emergency Dispatch, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Emergency Routing, Service Patrol Management |
| NMDOT District 1 HELP Courtesy Patrol Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| NMDOT District 1 ITS Field Equipment | Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway HOV Control, Roadway Infrastructure Monitoring, Standard Rail Crossing |
| NMDOT District 1 Maintenance and Construction Vehicles | MCV Barrier System Control, MCV Infrastructure Monitoring, MCV Roadway Maintenance and Construction, MCV Vehicle Location Tracking, MCV Vehicle Safety Monitoring, MCV Vehicle System Monitoring and Diagnostics, MCV Winter Maintenance, MCV Work Zone Support |
| NMDOT District 1 Maintenance Office | MCM Automated Treatment System Control, MCM Environmental Information Collection, MCM Incident Management, MCM Maintenance Decision Support, MCM Roadway Maintenance and Construction, MCM Vehicle and Equipment Maintenance Management, MCM Vehicle Tracking, MCM Winter Maintenance Management, MCM Work Activity Coordination, MCM Work Zone Management, MCM Work Zone Safety Management |
| NMDOT District 1 Maintenance Units Dispatch | MCM Automated Treatment System Control, MCM Environmental Information Collection, MCM Environmental Information Processing, MCM Incident Management, MCM Maintenance Decision Support, MCM Roadway Maintenance and Construction, MCM Vehicle and Equipment Maintenance Management, MCM Vehicle Tracking, MCM Winter Maintenance Management, MCM Work Activity Coordination, MCM Work Zone Management, MCM Work Zone Safety Management |
| NMDOT District 1 Public Information System | Basic Information Broadcast, Interactive Infrastructure Information, ISP Emergency Traveler Information, ISP Traveler Data Collection, Traveler Telephone Information |
| NMDOT District 1 Public Transportation Management System (PTMS) | Government Reporting Systems Support, ITS Data Repository, Traffic and Roadside Data Archival |
| NMDOT District 1 Security Monitoring Field Equipment | Field Secure Area Sensor Monitoring, Field Secure Area Surveillance |
| NMDOT District 1 TOC | Barrier System Management, Center Secure Area Sensor Management, Center Secure Area Surveillance, Collect Traffic Surveillance, Emergency Data Collection, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, HRI Traffic Management, Incident Command, MCM Data Collection, MCM Environmental Information Collection, MCM Environmental Information Processing, MCM Incident Management, MCM Maintenance Decision Support, MCM Roadway Maintenance and Construction, MCM Winter Maintenance Management, MCM Work Activity Coordination, MCM Work Zone Management, Rail Operations Coordination, Safeguard System Management, Service Patrol Management, TMC Environmental Monitoring, TMC Evacuation Support, TMC Freeway Management, TMC HOV Lane Management, TMC Incident Detection, TMC Incident Dispatch Coordination/Communication, TMC Probe Information Collection, TMC Regional Traffic Control, TMC Reversible Lane Management, TMC Signal Control, TMC Traffic Information Dissemination, TMC Work Zone Traffic Management, Traffic Data Collection, Traffic Maintenance |
| NMDOT District 1 Website | Basic Information Broadcast, Infrastructure Provided Trip Planning, ISP Traveler Data Collection |
| NMDOT DMS | Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Traffic Information Dissemination, Roadway Work Zone Traffic Control |
| NMDOT Field Sensors | Roadway Basic Surveillance, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring |
| NMDOT Highway Advisory Radio System (HAR) | Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Traffic Information Dissemination, Roadway Work Zone Traffic Control |
| NMDOT Highway Maintenance Management System (HMMS) | Government Reporting Systems Support, ITS Data Repository, On-Line Analysis and Mining, Traffic and Roadside Data Archival |
| NMDOT Historical Traffic Database | Government Reporting Systems Support, ITS Data Repository, Traffic and Roadside Data Archival |
| NMDOT Lane Controls | Roadway Basic Surveillance, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Freeway Control, Roadway Infrastructure Monitoring, Roadway Reversible Lanes, Roadway Traffic Information Dissemination |
| NMDOT Park and Ride Dispatch | Center Secure Area Alarm Support, Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Data Collection, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Transit Center Fixed-Route Operations, Transit Center Information Services, Transit Center Multi-Modal Coordination, Transit Center Security, Transit Center Vehicle Tracking, Transit Data Collection, Transit Environmental Monitoring, Transit Evacuation Support, Transit Garage Maintenance, Transit Vehicle Operator Scheduling |
| NMDOT Park and Ride Facilities | Parking Electronic Payment, Parking Management, Remote Basic Information Reception, Remote Interactive Information Reception, Remote Traveler Security, Traveler Secure Area Sensor Monitoring, Traveler Secure Area Surveillance |
| NMDOT Park and Ride Shuttle Buses | On-board Fixed Route Schedule Management, On-board Maintenance, On-board Transit Security, On-board Transit Trip Monitoring |
| NMDOT Rest Area/Visitor Center/Truck Stop/Service Plaza Kiosks | Remote Basic Information Reception, Remote Interactive Information Reception, Remote Transit Information Services |
| NMDOT Road Closure Gates | Field Barrier System Control, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring |
| NMDOT Roadway Treatment Devices | Roadway Automated Treatment, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Traffic Information Dissemination |
| NMDOT RWIS | Roadway Environmental Monitoring, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Probe Beacons |
| NMDOT Signal Lab | Collect Traffic Surveillance, HRI Traffic Management, TMC Incident Dispatch Coordination/Communication, TMC Signal Control, Traffic Maintenance |
| NMDOT Statewide Crash Information System | Government Reporting Systems Support, ITS Data Repository, Traffic and Roadside Data Archival |
| NMDOT Statewide Pavement Management System | Government Reporting Systems Support, ITS Data Repository, Traffic and Roadside Data Archival |
| NMDOT Statewide TMC | Barrier System Management, Collect Traffic Surveillance, HRI Traffic Management, MCM Data Collection, MCM Environmental Information Collection, MCM Environmental Information Processing, MCM Incident Management, MCM Maintenance Decision Support, MCM Roadway Maintenance and Construction, MCM Winter Maintenance Management, MCM Work Activity Coordination, MCM Work Zone Management, Rail Operations Coordination, TMC Environmental Monitoring, TMC Evacuation Support, TMC Freeway Management, TMC HOV Lane Management, TMC Incident Detection, TMC Incident Dispatch Coordination/Communication, TMC Probe Information Collection, TMC Regional Traffic Control, TMC Reversible Lane Management, TMC Signal Control, TMC Traffic Information Dissemination, TMC Work Zone Traffic Management, Traffic Data Collection, Traffic Maintenance |
| NMDOT Traffic Data Collection System | Roadway Data Collection, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Work Zone Traffic Control |
| NMDOT Traffic Safety Division Data System | Government Reporting Systems Support, ITS Data Repository, Traffic and Roadside Data Archival |
| NMDOT Traffic Signals | Roadway Basic Surveillance, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Signal Controls, Roadway Signal Priority, Standard Rail Crossing |
| NMDOT Work Zone Equipment | Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Work Zone Safety |
| NMSU Emergency Dispatch Center | Center Secure Area Alarm Support, Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Call-Taking, Emergency Data Collection, Emergency Dispatch, Emergency Early Warning System, Emergency Evacuation Support, Emergency Response Management, Emergency Routing, Incident Command |
| NMSU Emergency Operations Center (EOC) | Emergency Early Warning System, Emergency Evacuation Support, Emergency Response Management, Incident Command |
| NMSU Fire Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| NMSU ITS Field Equipment | Roadway Basic Surveillance, Roadway Equipment Coordination, Roadway Field Device Monitoring, Roadway Infrastructure Monitoring, Roadway Traffic Information Dissemination, Roadway Work Zone Traffic Control |
| NMSU Maintenance and Construction Vehicles | MCV Barrier System Control, MCV Infrastructure Monitoring, MCV Roadway Maintenance and Construction, MCV Vehicle Location Tracking, MCV Vehicle System Monitoring and Diagnostics, MCV Winter Maintenance, MCV Work Zone Support |
| NMSU Office of Facilities Operation and Utilities | MCM Incident Management, MCM Maintenance Decision Support, MCM Roadway Maintenance and Construction, MCM Vehicle and Equipment Maintenance Management, MCM Vehicle Tracking, MCM Winter Maintenance Management, MCM Work Zone Management |
| NMSU Parking System | Parking Electronic Payment, Parking Management |
| NMSU Police Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| NMSU Traffic Operations Center | Collect Traffic Surveillance, TMC Evacuation Support, TMC Freeway Management, TMC Incident Detection, TMC Incident Dispatch Coordination/Communication, TMC Regional Traffic Control, TMC Signal Control, TMC Traffic Information Dissemination, Traffic Maintenance |
| NMSU Website | Basic Information Broadcast, Infrastructure Provided Trip Planning, ISP Traveler Data Collection |
| Other NMDOT District Maintenance Dispatch | MCM Incident Management, MCM Maintenance Decision Support, MCM Winter Maintenance Management, MCM Work Activity Coordination, MCM Work Zone Management |
| Private Ambulance Provider Dispatch | Emergency Response Management, Emergency Routing, Incident Command |
| Private Ambulance Provider Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| Private HAZMAT Response Providers | Emergency Response Management, Emergency Routing, Incident Command |
| Private HAZMAT Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| Private Sector Traveler Information Services | Infrastructure Provided Trip Planning |
| Private Tow/Wrecker Dispatch | Emergency Response Management, Emergency Routing, Incident Command, On-board EV En Route Support, On-board EV Incident Management Communication |
| Private Tow/Wrecker Vehicles | On-board EV En Route Support, On-board EV Incident Management Communication |
| Private Travelers Personal Computing Devices | Personal Basic Information Reception, Personal Interactive Information Reception |
| Private Vehicles | Basic Vehicle Reception, Interactive Vehicle Reception, Vehicle Toll/Parking Interface |
| Public/Private Utilities | MCM Incident Management, MCM Roadway Maintenance and Construction, MCM Work Activity Coordination |
| Regional Transit Kiosks | Remote Basic Information Reception, Remote Interactive Information Reception, Remote Transit Fare Management, Remote Transit Information Services, Remote Traveler Security, Traveler Secure Area Sensor Monitoring, Traveler Secure Area Surveillance |
| RoadRUNNER Fixed Route Dispatch System | Center Secure Area Alarm Support, Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Data Collection, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Transit Center Fare and Load Management, Transit Center Fixed-Route Operations, Transit Center Information Services, Transit Center Multi-Modal Coordination, Transit Center Security, Transit Center Vehicle Tracking, Transit Data Collection, Transit Environmental Monitoring, Transit Evacuation Support, Transit Garage Maintenance, Transit Vehicle Operator Scheduling |
| RoadRUNNER Fixed Route Transit Vehicles | On-board Fixed Route Schedule Management, On-board Maintenance, On-board Transit Fare and Load Management, On-board Transit Information Services, On-board Transit Security, On-board Transit Signal Priority, On-board Transit Trip Monitoring |
| RoadRUNNER Paratransit Dispatch Center | Center Secure Area Alarm Support, Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Data Collection, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Transit Center Fare and Load Management, Transit Center Information Services, Transit Center Multi-Modal Coordination, Transit Center Paratransit Operations, Transit Center Security, Transit Center Vehicle Tracking, Transit Data Collection, Transit Environmental Monitoring, Transit Evacuation Support, Transit Garage Maintenance, Transit Vehicle Operator Scheduling |
| RoadRUNNER Paratransit Vehicles | On-board Maintenance, On-board Paratransit Operations, On-board Transit Fare and Load Management, On-board Transit Security, On-board Transit Signal Priority, On-board Transit Trip Monitoring |
| RoadRUNNER Transit Information Center | Basic Information Broadcast, Infrastructure Provided Trip Planning, Interactive Infrastructure Information, ISP Traveler Data Collection, Traveler Telephone Information |
| RoadRUNNER Transit Kiosks | Remote Basic Information Reception, Remote Interactive Information Reception, Remote Transit Fare Management, Remote Transit Information Services, Remote Traveler Security, Traveler Secure Area Sensor Monitoring, Traveler Secure Area Surveillance |
| RoadRUNNER Transit Website | Basic Information Broadcast, Infrastructure Provided Trip Planning, ISP Traveler Data Collection |
| South Central RTD Operations Center | Center Secure Area Alarm Support, Center Secure Area Sensor Management, Center Secure Area Surveillance, Emergency Data Collection, Emergency Early Warning System, Emergency Environmental Monitoring, Emergency Evacuation Support, Emergency Response Management, Transit Center Fare and Load Management, Transit Center Fixed-Route Operations, Transit Center Information Services, Transit Center Multi-Modal Coordination, Transit Center Paratransit Operations, Transit Center Security, Transit Center Vehicle Tracking, Transit Data Collection, Transit Environmental Monitoring, Transit Evacuation Support, Transit Garage Maintenance, Transit Vehicle Operator Scheduling |
| South Central RTD Transit Vehicles | On-board Fixed Route Schedule Management, On-board Maintenance, On-board Paratransit Operations, On-board Transit Fare and Load Management, On-board Transit Security, On-board Transit Signal Priority, On-board Transit Trip Monitoring |
| South Central RTD Transit Website | Infrastructure Provided Trip Planning, ISP Traveler Data Collection |
| Southwest MedEvac | Emergency Call-Taking, Emergency Dispatch, Emergency Response Management, Emergency Routing |
| Southwest RTD Operations Center | Transit Center Information Services, Transit Center Multi-Modal Coordination, Transit Center Security, Transit Data Collection, Transit Environmental Monitoring, Transit Evacuation Support |
| Texas DPS Communications Service | Emergency Evacuation Support, Emergency Response Management |
| TransVista | TMC Environmental Monitoring |
| US Border Patrol Stations | Emergency Call-Taking, Emergency Dispatch |
| White Sands Missile Range Facility | Emergency Call-Taking, Emergency Dispatch |

Table 19 provides a definition of each of the Equipment Packages (functions) from the previous table.

Table 19.  Equipment Package Descriptions

| Equipment Package | Equipment Package Description |
| --- | --- |
| Barrier System Management | This equipment package remotely monitors and controls barrier systems for transportation facilities and infrastructure under control of center personnel. Barrier systems include automatic or remotely controlled gates, barriers and other access control systems. The equipment package also provides an interface to other centers to allow monitoring and control of the barriers from other centers (e.g., public safety or emergency operations centers). |
| Basic Information Broadcast | This equipment package collects, processes, stores, and disseminates traveler information including traffic and road conditions, incident information, maintenance and construction information, event information, transit information, parking information, and weather information. The same information is broadcast to all equipped traveler interface systems and vehicles. |
| Basic Vehicle Reception | This equipment package provides the capability for drivers to receive basic transportation information including traffic and road conditions, incident information, maintenance and construction information, event information, transit information, parking information, weather information, and broadcast alerts. |
| Center Secure Area Alarm Support | This equipment package receives traveler or transit vehicle operator alarm messages, notifies the system operator, and provides acknowledgement of alarm receipt back to the originator of the alarm. The alarms received can be generated by silent or audible alarm systems and may originate from public areas (e.g. transit stops, park and ride lots, transit stations, rest areas) or transit vehicles. The nature of the emergency may be determined based on the information in the alarm message as well as other inputs. |
| Center Secure Area Sensor Management | This equipment package manages sensors that monitor secure areas in the transportation system, processes the collected data, performs threat analysis in which data is correlated with other sensor, surveillance, and advisory inputs, and then disseminates resultant threat information to emergency personnel and other agencies. In response to identified threats, the operator may request activation of barrier and safeguard systems to preclude an incident, control access during and after an incident or mitigate impact of an incident. The sensors may be in secure areas frequented by travelers (i.e., transit stops, transit stations, rest areas, park and ride lots, modal interchange facilities, on-board a transit vehicle, etc.) or around transportation infrastructure such as bridges, tunnels and transit railways or guideways. The types of sensors include acoustic, threat (e.g. chemical agent, toxic industrial chemical, biological, explosives, and radiological sensors), infrastructure condition and integrity, motion and object sensors. |
| Center Secure Area Surveillance | This equipment package monitors surveillance inputs from secure areas in the transportation system. The surveillance may be of secure areas frequented by travelers (i.e., transit stops, transit stations, rest areas, park and ride lots, modal interchange facilities, on-board a transit vehicle, etc.) or around transportation infrastructure such as bridges, tunnels and transit railways or guideways. It provides both video and audio surveillance information to emergency personnel and automatically alerts emergency personnel of potential incidents. |
| Collect Traffic Surveillance | This equipment package remotely monitors and controls traffic sensors and surveillance (e.g., CCTV) equipment, and collects, processes and stores the collected traffic data. The collected information is provided to traffic operations personnel and made available to other centers. |
| Emergency Call-Taking | This equipment package supports the emergency call-taker, collecting available information about the caller and the reported emergency, and forwarding this information to other equipment packages that formulate and manage the emergency response. This equipment package receives 9-1-1, 7-digit local access, and motorist call-box calls and interfaces to other agencies to assist in the verification and assessment of the emergency and to forward the emergency information to the appropriate response agency. |
| Emergency Data Collection | This equipment package collects and stores emergency information that is collected in the course of operations by the Emergency Management Subsystem. This data can be used directly by operations personnel or it can be made available to other data users and archives in the region. |
| Emergency Dispatch | This equipment package tracks the location and status of emergency vehicles and dispatches these vehicles to incidents. Pertinent incident information is gathered from the public and other public safety agencies (see the Emergency Call-Taking equipment package) and relayed to the responding units. Incident status and the status of the responding units is tracked so that additional units can be dispatched and/or unit status can be returned to available when the incident is cleared and closed. |
| Emergency Early Warning System | This equipment package monitors alerting and advisory systems, information collected by ITS surveillance and sensors, and reports from other agencies and uses this information to identify potential, imminent, or in-progress major incidents or disasters. Notification is provided to other equipment packages that provide the emergency response, including public notification using ITS traveler information systems, where appropriate. |
| Emergency Environmental Monitoring | This equipment package collects current and forecast road conditions and surface weather information from a variety of sources, including both weather service providers and vehicle probes. The collected environmental information is monitored and presented to the operator and used to more effectively manage incidents. |
| Emergency Evacuation Support | This equipment package coordinates evacuation plans among allied agencies and manages evacuation and reentry of a population in the vicinity of a disaster or other emergency that poses a risk to public safety. Where appropriate, the affected population is evacuated in shifts, using more than one evacuation route, and including several evacuation destinations to spread demand and thereby expedite the evacuation. All affected jurisdictions (e.g., states and counties) at the evacuation origin, evacuation destination, and along the evacuation route are informed of the plan. The public is provided with real-time evacuation guidance including basic information to assist potential evacuees in determining whether evacuation is necessary. Resource requirements are forecast based on the evacuation plans, and the necessary resources are located, shared between agencies if necessary, and deployed at the right locations at the appropriate times. The evacuation and reentry status are monitored and used to refine the plan and resource allocations during the evacuation and subsequent reentry. |
| Emergency Response Management | This equipment package provides the strategic emergency response capabilities and broad inter-agency interfaces that are implemented for extraordinary incidents and disasters that require response from outside the local community. It provides the functional capabilities and interfaces commonly associated with Emergency Operations Centers. This equipment package develops and stores emergency response plans and manages overall coordinated response to emergencies. It tracks the availability of resources and assists in the appropriate allocation of these resources for a particular emergency response. This equipment package provides coordination between multiple allied agencies before and during emergencies to implement emergency response plans and track progress through the incident. |
| Emergency Routing | This equipment package supports routing of emergency vehicles and enlists support from the Traffic Management Subsystem to facilitate travel along these routes. Routes may be determined by this equipment package based on real-time traffic information and road conditions or routes may be provided by the Traffic Management Subsystem on request. Vehicles are tracked and routes are based on current vehicle location. This equipment package may coordinate with the Traffic Management Subsystem to provide preemption or otherwise adapt the traffic control strategy along the selected route. |
| Field Barrier System Control | This equipment package includes the field equipment that controls barrier systems used to control access to transportation facilities and infrastructure. Barrier systems include automatic or remotely controlled gates, barriers and other access control systems. |
| Field Safeguard System Control | This equipment package includes field equipment that controls safeguard systems for transportation facilities and infrastructure. Safeguard systems include blast shields, exhaust systems and other automatic or remotely controlled systems intended to mitigate the impact of an incident. |
| Field Secure Area Sensor Monitoring | This equipment package includes sensors that monitor conditions of secure areas including facilities (e.g. transit yards) and transportation infrastructure (e.g. bridges, tunnels, interchanges, and transit railways or guideways). A range of acoustic, environmental threat (e.g. chemical agent, toxic industrial chemical, biological, explosives, and radiological sensors), infrastructure condition and integrity and motion and object sensors are included. |
| Field Secure Area Surveillance | This equipment package includes video and audio surveillance equipment that monitors conditions of secure areas including facilities (e.g. transit yards) and transportation infrastructure (e.g. as bridges, tunnels, interchanges, and transit railways or guideways). It provides the surveillance information to the Emergency Management Subsystem for possible threat detection. The equipment package also provides local processing of the video or audio information, providing processed or analyzed results to the Emergency Management Subsystem. This equipment package provides the same functions as the Traveler Secure Area Surveillance equipment package. |
| Government Reporting Systems Support | This equipment package selects and formats data residing in an ITS archive to facilitate local, state, and federal government data reporting requirements. |
| HRI Traffic Management | This equipment package monitors and controls highway-rail intersection (HRI) equipment. Various levels of roadside equipment may be interfaced to this equipment package including standard speed active warning systems and high speed systems which provide additional information on approaching trains and detect and report obstructions in the HRI. This equipment package remotely monitors and reports the status of the HRI equipment and sends control plan updates to the HRI equipment. |
| Incident Command | The equipment package provides tactical decision support, resource coordination, and communications integration for Incident Commands that are established by first responders at or near the incident scene to support local management of an incident. The equipment package supports communications with public safety, emergency management, transportation, and other allied response agency centers, tracks and maintains resource information, action plans, and the incident command organization itself. Information is shared with agency centers including resource deployment status, hazardous material information, traffic, road, and weather conditions, evacuation advice, and other information that enables emergency or maintenance personnel in the field to implement an effective, safe incident response. This equipment package supports the functions and interfaces commonly supported by a mobile command center. |
| Infrastructure Provided Trip Planning | This equipment package provides pre-trip and en-route trip planning services for travelers. It receives origin, destination, constraints, and preferences and returns trip plan(s) that meet the supplied criteria. Trip plans may be based on current traffic and road conditions, transit schedule information, and other real-time traveler information. Candidate trip plans are multimodal and may include vehicle, transit, and alternate mode segments (e.g., rail, ferry, bicycle routes, and walkways) based on traveler preferences. This equipment package also confirms the trip plan for the traveler and supports reservations and advanced payment for portions of the trip. The trip plan includes specific routing information and instructions for each segment of the trip and may also include information and reservations for additional services (e.g., parking) along the route. |
| Interactive Infrastructure Information | This equipment package collects, processes, stores, and disseminates personalized traveler information including traffic and road conditions, transit information, maintenance and construction information, multimodal information, event information, and weather information. Tailored information is provided based on the traveler's request in this interactive equipment package. The interactive service offered by this equipment package is available to the Vehicle, Remote Traveler Support, and Personal Information Access subsystems. |
| Interactive Vehicle Reception | This equipment package provides drivers with personalized traveler information including traffic and road conditions, transit information, maintenance and construction information, multimodal information, event information, and weather information. The provided information is tailored based on driver requests. Both one-time requests for information and on-going information streams based on a submitted traveler profile and preferences are supported. |
| ISP Emergency Traveler Information | This equipment package collects and provides emergency information to the public, including wide-area alerts and evacuation information. It provides emergency alerts, information on evacuation zones and evacuation requirements, evacuation destinations and shelter information, available transportation modes, and traffic and road conditions at the origin, destination, and along the evacuation routes. In addition to general evacuation information, personalized information including tailored evacuation routes, service information, and estimated travel times is also provided based on traveler specified origin, destination, and route parameters. Updated information is provided throughout the evacuation and subsequent reentry as status changes and plans are adapted. |
| ISP Traveler Data Collection | This equipment package collects traveler-related data from other centers, consolidates, verifies, and refines the collected data, and makes this data available in a consistent format to applications that deliver traveler information. A broad range of traveler-related data is collected including traffic and road conditions, transit data, emergency information and advisories, weather data, special event information, traveler services, parking, multimodal data, and toll/pricing data. This equipment package also shares data with other information service providers. |
| ITS Data Repository | This equipment package collects data and data catalogs from one or more data sources and stores the data in a focused repository that is suited to a particular set of ITS data users. This equipment package includes capabilities for performing quality checks on the incoming data, error notification, and archive to archive coordination. This equipment package supports a broad range of implementations, ranging from simple data marts that collect a focused set of data and serve a particular user community to large-scale data warehouses that collect, integrate, and summarize transportation data from multiple sources and serve a broad array of users within a region. |
| MCM Automated Treatment System Control | This equipment package remotely monitors and controls automated road treatment systems that disperse anti-icing chemicals or otherwise treat a road segment. The automated treatment system may be remotely activated by this equipment package or it may include environmental sensors that activate the system automatically based on sensed environmental conditions. This equipment package monitors treatment system operation, sets operating parameters, and directly controls system activation if necessary. |
| MCM Data Collection | This equipment package collects and stores maintenance and construction information that is collected in the course of operations by the Maintenance and Construction Management Subsystem. This data can be used directly by operations personnel or it can be made available to other data users and archives in the region. |
| MCM Environmental Information Collection | This equipment package collects current road and weather conditions using data collected from environmental sensors deployed on and about the roadway. In addition to fixed sensor stations at the roadside, this equipment package also collects environmental information from sensor systems located on Maintenance and Construction Vehicles. It also collects current and forecast environmental conditions information that is made available by other systems. |
| MCM Environmental Information Processing | This equipment package processes current and forecast weather data, road condition information, local environmental data, and uses internal models to develop specialized detailed forecasts of local weather and surface conditions. The processed environmental information products are presented to center personnel and disseminated to other centers. |
| MCM Incident Management | This equipment package supports maintenance and construction participation in coordinated incident response. Incident notifications are shared, incident response resources are managed, and the overall incident situation and incident response status is coordinated among allied response organizations. |
| MCM Maintenance Decision Support | This equipment package recommends maintenance courses of action based on current and forecast environmental and road conditions and additional application specific information. Decisions are supported through understandable presentation of filtered and fused environmental and road condition information for specific time horizons as well as specific maintenance recommendations that are generated by the system based on this integrated information. The recommended courses of action are supported by information on the anticipated consequences of action or inaction, when available. |
| MCM Roadway Maintenance and Construction | This equipment package provides overall management and support for routine maintenance on a roadway system or right-of-way. Services managed include landscape maintenance, hazard removal (roadway debris, dead animals), routine maintenance activities (roadway cleaning, grass cutting), and repair and maintenance of both ITS and non-ITS equipment on the roadway (e.g., signs, traffic controllers, traffic detectors, dynamic message signs, traffic signals, etc.). Environmental conditions information is also received from various weather sources to aid in scheduling routine maintenance activities. |
| MCM Vehicle and Equipment Maintenance Management | This equipment package monitors vehicle and equipment condition, tracks maintenance history, and schedules routine and corrective maintenance based on vehicle utilization and availability schedules. |
| MCM Vehicle Tracking | This equipment package tracks the location of maintenance and construction vehicles and other equipment. Vehicle location and associated information is presented to the operator. |
| MCM Winter Maintenance Management | This equipment package manages winter road maintenance, tracking and controlling snow plow operations, roadway treatment (e.g., salt spraying and other material applications), and other snow and ice control operations. It monitors environmental conditions and weather forecasts and uses the information to schedule winter maintenance activities, determine the appropriate snow and ice control response, and track and manage response operations. |
| MCM Work Activity Coordination | This equipment package disseminates work activity schedules and current asset restrictions to other agencies. Work schedules are coordinated with operating agencies, factoring in the needs and activities of other agencies and adjacent jurisdictions. Work schedules are also distributed to Information Service Providers for dissemination to the traveling public. |
| MCM Work Zone Management | This equipment package remotely monitors and supports work zone activities, controlling traffic through dynamic message signs (DMS), Highway Advisory Radio (HAR), gates and barriers, and informing other groups of activity (e.g., ISP, TM, other maintenance and construction centers) for better coordination management. Work zone speeds, and delays, and closures are provided to the motorist prior to the work zones. This equipment package provides control of field equipment in all maintenance areas, including fixed and portable field equipment supporting both stationary and mobile work zones. |
| MCM Work Zone Safety Management | This equipment package remotely monitors work zone safety systems that detect vehicle intrusions in work zones and warns crew workers and drivers of imminent encroachment. Crew movements are also monitored so that the crew can be warned of movement beyond the designated safe zone. |
| MCV Barrier System Control | This on-board equipment package provides local control of automatic or remotely controlled gates and other barrier systems from a maintenance and construction vehicle. Using this equipment package, maintenance and construction field personnel (e.g., snow plow operators) can open and close gates and other barrier systems without leaving the vehicle, using dedicated short range communications to control the barriers. |
| MCV Infrastructure Monitoring | This on-board equipment package monitors the condition of pavement, bridges, tunnels, associated hardware, and other transportation-related infrastructure (e.g., culverts). It includes vehicle-based sensors that directly monitor the infrastructure, communications that allow roadway-based infrastructure monitoring sensors to be controlled and read, and data communications that allows collected infrastructure condition information to be reported back to a center. |
| MCV Roadway Maintenance and Construction | This equipment package includes the on-board systems that support routine non-winter maintenance on a roadway system or right-of-way. Routine maintenance includes landscape maintenance, hazard removal (roadway debris, dead animals), routine maintenance activities (roadway cleaning, grass cutting), and repair and maintenance of both ITS and non-ITS equipment on the roadway (e.g., signs, traffic controllers, traffic detectors, dynamic message signs, traffic signals, etc.). |
| MCV Vehicle Location Tracking | This on-board equipment package tracks vehicle location and reports the position and timestamp information to a dispatch center. |
| MCV Vehicle Safety Monitoring | This equipment package detects vehicle intrusions in the vicinity of the vehicle and warns crew workers and drivers of imminent encroachment. Crew movements are also monitored so that the crew can be warned of movement beyond the designated safe zone. This equipment package can be used for stationary work zones or in mobile applications where a safe zone is maintained around the moving vehicle. |
| MCV Vehicle System Monitoring and Diagnostics | This equipment package includes on-board sensors capable of monitoring the condition of each of the vehicle systems and diagnostics that can be used to support vehicle maintenance. The status of the vehicle and ancillary equipment and diagnostic information is provided to the vehicle operator, repair facility, and dispatch center. |
| MCV Winter Maintenance | This on-board equipment package supports snow plow operations and other roadway treatments (e.g., salt spraying and other material applications). It supports communications with the center to receive information and instructions that are provided to the vehicle operator and also supports remote control of on-board systems. The equipment package tracks operational status of snow and ice control operations and provides this information back to the center. |
| MCV Work Zone Support | This on-board equipment package provides communications and support for local management of a work zone. It supports communications between field personnel and the managing center to keep the center appraised of current work zone status. It controls vehicle-mounted driver information systems (e.g., dynamic message signs) and uses short range communications to monitor and control other fixed or portable driver information systems in the work zone. |
| On-board EV En Route Support | This on-board equipment package supports dispatch, routing, and tracking of an emergency vehicle. Dispatch and routing information are received and presented to the driver and vehicle location and status are tracked and provided back to the dispatcher. This equipment package supports traffic signal preemption via short range communication directly with signal control equipment. It also supports communications with care facilities, sharing patient status and care facility status between the en route emergency vehicle and the care facility. |
| On-board EV Incident Management Communication | This on-board equipment package provides communications support to first responders. Information about the incident, information on dispatched resources, and ancillary information such as road and weather conditions are provided to emergency personnel. Emergency personnel transmit information about the incident such as identification of vehicles and people involved, the extent of injuries, hazardous material, resources on site, site management strategies in effect, and current clearance status. |
| On-board Fixed Route Schedule Management | This on-board equipment package monitors schedule performance and identifies corrective actions when a deviation is detected. It provides two-way communication between the transit vehicle and center, enabling the center to communicate with the vehicle operator and monitor on-board systems. |
| On-board Maintenance | This on-board equipment package collects and processes transit vehicle maintenance data including mileage and vehicle operating conditions. This maintenance information is provided to the management center and used to schedule future vehicle maintenance and repair. |
| On-board Paratransit Operations | This on-board equipment package forwards paratransit and flexible-route dispatch requests to the operator and forwards acknowledgements to the center. It coordinates with, and assists the operator in managing multi-stop runs associated with demand responsive transit services including paratransit. |
| On-board Transit Fare and Load Management | This on-board equipment package supports fare collection using a standard fare card or other non-monetary fare medium, detects payment violations, and counts passengers as they embark and disembark from the vehicle. It collects data required to determine accurate ridership levels and implement variable and flexible fare structures. Collected passenger and fare data are made available to the center. |
| On-board Transit Information Services | This equipment package furnishes en-route transit users with real-time travel-related information on-board a transit vehicle. Current information that can be provided to transit users includes transit routes, schedules, transfer options, fares, real-time schedule adherence, current incidents, weather conditions, non-motorized transportation services, and special events are provided. In addition to tailored information for individual transit users, this equipment package also supports general annunciation and/or display of general schedule information, imminent arrival information, and other information of general interest to transit users. |
| On-board Transit Security | This equipment package provides security and safety functions on-board the transit vehicle. It includes surveillance and sensor systems that monitor the on-board environment, silent alarms that can be activated by transit user or vehicle operator, operator authentication, and a remote vehicle disable function. The surveillance equipment includes video (e.g. CCTV cameras), audio systems and/or event recorder systems. The sensor equipment includes threat sensors (e.g. chemical agent, toxic industrial chemical, biological, explosives, and radiological sensors) and object detection sensors (e.g. metal detectors). |
| On-board Transit Signal Priority | This on-board equipment package provides the capability for transit vehicles to request signal priority at signalized intersections, ramps, and interchanges through short range communication directly with traffic control equipment at the roadside. |
| On-board Transit Trip Monitoring | This on-board equipment package tracks vehicle location, monitors fuel usage, collects operational status (doors opened/closed, running times, etc.) and sends the collected, timestamped data to the Transit Management Subsystem. |
| On-Line Analysis and Mining | This equipment package provides advanced data analysis, summarization, and mining features that facilitate discovery of information, patterns, and correlations in large data sets. Multidimensional analysis, selective summarization and expansion of data details, and many other advanced analysis services may be offered by various implementations of this equipment package. |
| Parking Electronic Payment | This equipment package supports electronic payment of parking fees using in-vehicle equipment (tags) or contact or proximity cards. It includes the field elements that provide the interface to the in-vehicle or card payment device and the back-office functionality that performs the transaction. |
| Parking Management | This equipment package detects and classifies vehicles at parking facility entrances, exits, and other designated locations within the facility. Current parking availability is monitored and used to inform drivers through dynamic message signs/displays so that vehicles are efficiently routed to available spaces. Parking facility information, including current parking rates and directions to entrances and available exits, is also provided to drivers. Coordination with traffic management supports local traffic control coordination in and around the parking facility. |
| Personal Basic Information Reception | This equipment package receives formatted traffic advisories, road conditions, transit information, broadcast alerts, and other general traveler information broadcasts and presents the information to the traveler. The traveler information broadcasts are received by personal devices including personal computers and personal portable devices such as personal digital assistants (PDAs) and pagers. |
| Personal Interactive Information Reception | This equipment package provides traffic information, road conditions, transit information, yellow pages (traveler services) information, special event information, and other traveler information that is specifically tailored based on the traveler's request and/or previously submitted traveler profile information. The interactive traveler information capability is provided by personal devices including personal computers and personal portable devices such as personal digital assistants (PDAs). |
| Rail Operations Coordination | This equipment package provides coordination between rail operations and traffic management centers. It receives train schedules, maintenance schedules, incidents, priority messages, and any other forecast events that will impact highway-rail intersection (HRI) closures from Rail Operations. The provided information is used to develop forecast HRI closure times and durations which may be applied in advanced traffic control strategies or delivered as enhanced traveler information. This equipment package includes the processing and algorithms necessary to derive HRI closure times and the communications capabilities necessary to communicate with rail operations and interface to the traffic control and information distribution capabilities included in other Traffic Management Subsystem equipment packages. |
| Remote Basic Information Reception | This equipment package receives formatted traffic advisories, road conditions, transit information, broadcast alerts, and other general traveler information broadcasts and presents the information to the traveler with a public traveler interface. This equipment package includes the receiver and public display device such as a kiosk, large-scale display monitor or other public display. |
| Remote Interactive Information Reception | This equipment package provides traffic information, road conditions, transit information, yellow pages (traveler services) information, special event information, and other traveler information that is specifically tailored based on the traveler's request and/or previously submitted traveler profile information. The interactive traveler information capability is provided by a public traveler interface, such as a kiosk. |
| Remote Transit Fare Management | This equipment package provides the capability for the traveler to use a common fare medium for transit fares, tolls, and/or parking lot charges. It accepts a service request and means of payment, verifies eligibility, calculates the amount due, collects payment, and identifies payment problems. This equipment package may be implemented using a traveler card reader in a kiosk that includes a communications interface to the financial infrastructure to support payment collection and reconciliation. |
| Remote Transit Information Services | This equipment package furnishes transit users with real-time travel-related information at transit stops, multi-modal transfer points, and other public transportation areas. It provides transit users with information on transit routes, schedules, transfer options, available services, fares, and real-time schedule adherence. In addition to tailored information for individual transit users, this equipment package supports general annunciation and/or display of imminent arrival information and other information of general interest to transit users. |
| Remote Traveler Security | This equipment package provides the capability to report an emergency or summon assistance from secure areas such as transit stops, transit stations, modal transfer facilities, rest stops and picnic areas, park-and-ride areas, tourism and travel information areas, and emergency pull off areas. This package includes interfaces that support initiation of an alarm and presentation of the returned alarm acknowledgement as well as a broadcast message to advise or warn the traveler. |
| Roadway Automated Treatment | This equipment package automatically treats a roadway section based on environmental or atmospheric conditions or under center control. Treatments include fog dispersion, anti-icing chemicals, etc |
| Roadway Basic Surveillance | This equipment package monitors traffic conditions using fixed equipment such as loop detectors and CCTV cameras. |
| Roadway Data Collection | This equipment package collects traffic, road, and environmental conditions information for use in transportation planning, research, and other off-line applications where data quality and completeness take precedence over real-time performance. This equipment package includes the sensors, supporting roadside infrastructure, and communications equipment that collects and transfers information to a center for archival. |
| Roadway Environmental Monitoring | This equipment package measures environmental conditions and communicates the collected information back to a center where it can be monitored and analyzed. A broad array of general weather and road surface information may be collected. Weather conditions that may be measured include temperature, wind, humidity, precipitation, and visibility. Surface and sub-surface sensors can measure road surface temperature, moisture, icing, salinity, and other measures. |
| Roadway Equipment Coordination | This equipment package supports direct communications between field equipment. It includes field elements that control and send data to other field elements. This includes coordination between remote sensors and field devices (e.g., Dynamic Message Signs) and coordination between the field devices themselves (e.g., direct coordination between traffic controllers that are controlling adjacent intersections.). |
| Roadway Field Device Monitoring | This equipment package monitors the operational status of field devices and detects and reports fault conditions. Consolidated operational status (device status, configuration, and fault information) are reported to the Maintenance and Construction Management Subsystem for resolution and repair. A local interface is provided to field personnel for local monitoring and diagnostics, supporting field maintenance, repair, and replacement of field devices. |
| Roadway Freeway Control | This equipment package includes the field equipment used to control traffic on freeways including ramp meters, interchange connector meters, mainline meters, and lane control signals. |
| Roadway HOV Control | This equipment package monitors and controls high occupancy vehicle (HOV) and high occupancy toll (HOT) lanes. It includes traffic sensors that monitor HOV lane usage and display equipment such as lane control signals that provide lane status to drivers. |
| Roadway Incident Detection | This equipment package provides incident detection using traffic detectors and surveillance equipment. It monitors for unusual traffic conditions that may indicate an incident or processes surveillance images, watching for potential incidents. This equipment package provides potential incident information as well as traffic flow and images to the center for processing and presentation to traffic operations personnel. |
| Roadway Infrastructure Monitoring | This equipment package monitors the condition of pavement, bridges, tunnels, associated hardware, and other transportation-related infrastructure (e.g., culverts). It includes sensors that monitor the infrastructure and the communications necessary to report this data to a center or vehicle-based maintenance system. |
| Roadway Probe Beacons | This equipment package monitors traffic and road conditions by collecting information from passing vehicles that are equipped with a transponder or other short range communications device. The probe data collected by this equipment package may include link travel times, average speeds, road conditions, and any other data that can be measured and communicated by passing vehicles. This equipment package consists of field equipment that communicates with passing vehicles using short range communications, collects the information provided by the vehicles, and forwards this information back to the Traffic Management Subsystem. |
| Roadway Reversible Lanes | This equipment package includes field elements that monitor and control reversible lane facilities. It includes the traffic sensors, surveillance equipment, lane control signals, physical lane access controls, and other field elements that manage traffic on these facilities. It provides current reversible lane facility status information and accepts requests and control commands from the controlling center. |
| Roadway Signal Controls | This equipment package includes the field elements that monitor and control signalized intersections. It includes the traffic signal controllers, signal heads, detectors, and other ancillary equipment that supports traffic signal control. It also includes field masters, and equipment that supports communications with a central monitoring and/or control system, as applicable. The communications link supports upload and download of signal timings and other parameters and reporting of current intersection status. This equipment package represents the field equipment used in all levels of traffic signal control from basic actuated systems that operate on fixed timing plans through adaptive systems. It also supports all signalized intersection configurations, including those that accommodate pedestrians. |
| Roadway Signal Priority | This equipment package includes the field elements that receive signal priority and/or signal preemption requests from vehicles approaching a signalized intersection and controls traffic signals accordingly. Depending on the type of request and implementation, this equipment package may override (preempt) current signal timing or delay phase transition. In signal priority systems, the request for priority may or may not be granted, based on the overall traffic situation at the intersection. |
| Roadway Traffic Information Dissemination | This equipment package includes field elements that provides information to drivers, including dynamic message signs and highway advisory radio. |
| Roadway Work Zone Safety | This equipment package includes field elements that detect vehicle intrusions in work zones and warns crew workers and drivers of imminent encroachment. Crew movements are also monitored so that the crew can be warned of movement beyond the designated safe zone. |
| Roadway Work Zone Traffic Control | This equipment package controls traffic in areas of the roadway where maintenance and construction activities are underway, monitoring and controlling traffic using field equipment such as CCTV cameras, dynamic messages signs, and gates/barriers. Work zone speeds and delays are provided to the motorist prior to the work zones. |
| Safeguard System Management | This equipment package remotely monitors and controls safeguard systems for transportation facilities and infrastructure. Safeguard systems include blast shielding, exhaust systems and other automatic or remotely controlled systems intended to mitigate the impact of an incident. When access to a transportation facility is impacted by the activation of a safeguard system, travelers and appropriate subsystems are notified. |
| Service Patrol Management | This equipment package supports dispatch and communication with roadway service patrol vehicles that monitor roads to aid motorists, offering rapid response to minor incidents. |
| Standard Rail Crossing | This equipment package manages highway traffic at highway-rail intersections (HRIs) where operational requirements do not dictate advanced features (e.g., where rail operational speeds are less than 80 miles per hour). Either passive (e.g., the crossbuck sign) or active warning systems (e.g., flashing lights and gates) are supported depending on the specific requirements for each intersection. These traditional HRI warning systems may also be augmented with other standard traffic management devices. The warning systems are activated on notification of an approaching train by interfaced wayside equipment. The equipment at the HRI may also be interconnected with adjacent signalized intersections so that local control can be adapted to highway-rail intersection activities. Health monitoring of the HRI equipment and interfaces is performed; detected abnormalities are reported through interfaces to the wayside interface equipment and the traffic management subsystem. |
| TMC Environmental Monitoring | This equipment package assimilates current and forecast road conditions and surface weather information using a combination of weather service provider information, information collected by other centers such as the Maintenance and Construction Management Subsystem, and data collected from environmental sensors deployed on and about the roadway. The collected environmental information is monitored and presented to the operator. This information can be used to issue general traveler advisories and support location specific warnings to drivers. Other equipment packages process the collected information and provide decision support. |
| TMC Evacuation Support | This equipment package supports development, coordination, and execution of special traffic management strategies during evacuation and subsequent reentry of a population in the vicinity of a disaster or major emergency. A traffic management strategy is developed based on anticipated demand, the capacity of the road network including access to and from the evacuation routes, and existing and forecast conditions. The strategy supports efficient evacuation and also protects and optimizes movement of response vehicles and other resources that are responding to the emergency. This equipment package coordinates the evacuation with the Traffic Management Subsystem (representing centers in other affected jurisdictions) and the Emergency Management Subsystem. |
| TMC Freeway Management | This equipment package provides center monitoring and control of freeway traffic control systems including ramp control, interchange control, and mainline lane control systems. Approaches covered include ramp metering, interchange connector metering, overhead lane control signals, freeway mainline metering, and variable speed control systems. |
| TMC HOV Lane Management | This equipment package provides center monitoring and control of HOV lanes. It coordinates freeway ramp meters and connector signals with HOV lane usage signals to provide preferential treatment to HOV lanes. In advanced implementations, it automatically detects HOV violators. |
| TMC Incident Detection | This equipment package identifies and reports incidents to Traffic Operations Personnel. It remotely monitors and controls traffic sensor and surveillance systems that support incident detection and verification. It analyzes and reduces the collected sensor and surveillance data, external alerting and advisory and incident reporting systems, anticipated demand information from intermodal freight depots, special event information, and identifies and reports incidents and hazardous conditions |
| TMC Incident Dispatch Coordination/ Communication | This equipment package formulates and manages an incident response that takes into account the incident potential, incident impacts, and/or resources required for incident management including proposing and facilitating the dispatch of emergency response and service vehicles as well as coordinating response with all appropriate cooperating agencies. |
| TMC Multimodal Coordination | This equipment package supports center-to-center coordination between the Traffic Management and Transit Management Subsystems. It monitors transit operations and provides traffic signal priority for transit vehicles on request from the Transit Management Subsystem. |
| TMC Probe Information Collection | This equipment package collects, assimilates, and disseminates vehicle probe data collected from roadside beacons and centers controlling transit vehicles, emergency vehicles, toll collection points, and route-guided vehicles. It estimates traffic and road conditions based on the aggregated probe data and disseminates this information to other centers. |
| TMC Regional Traffic Control | This equipment package supports coordination between traffic management centers in order to share traffic information between centers as well as control of traffic management field equipment. This coordination supports wide area optimization and regional coordination that spans jurisdictional boundaries; for example, coordinated signal control in a metropolitan area or coordination between freeway operations and arterial signal control within a corridor. |
| TMC Reversible Lane Management | This equipment package remotely monitors and controls reversible lanes. It provides an interface to reversible lane field equipment (traffic sensors, surveillance equipment, lane control signals, physical lane access controls, etc.) and to traffic operations personnel to support central monitoring and control of these facilities. |
| TMC Signal Control | This equipment package provides the capability for traffic managers to monitor and manage the traffic flow at signalized intersections. This capability includes analyzing and reducing the collected data from traffic surveillance equipment and developing and implementing control plans for signalized intersections. Control plans may be developed and implemented that coordinate signals at many intersections under the domain of a single traffic management subsystem and are responsive to traffic conditions and adapt to support incidents, preemption and priority requests, pedestrian crossing calls, etc. |
| TMC Traffic Information Dissemination | This equipment package disseminates traffic and road conditions, closure and detour information, incident information, driver advisories, and other traffic-related data to other centers, the media, and driver information systems. It monitors and controls driver information system field equipment including dynamic message signs and highway advisory radio, managing dissemination of driver information through these systems. |
| TMC Work Zone Traffic Management | This equipment package coordinates work plans with maintenance systems so that work zones are established that have minimum traffic impact. Traffic control strategies are implemented to further mitigate traffic impacts associated with work zones that are established, providing work zone information on driver information systems such as dynamic message signs. |
| Traffic and Roadside Data Archival | This equipment package collects and archives traffic, roadway, and environmental information for use in off-line planning, research, and analysis. The equipment package controls and collects information directly from equipment at the roadside, reflecting the deployment of traffic detectors that are used primarily for traffic monitoring and planning purposes rather than for traffic management. |
| Traffic Data Collection | This equipment package collects and stores traffic information that is collected in the course of traffic operations performed by the Traffic Management Subsystem. This data can be used directly by operations personnel or it can be made available to other data users and archives in the region. |
| Traffic Maintenance | This equipment package monitors the operational status of field equipment and detects failures. It presents field equipment status to Traffic Operations Personnel and reports failures to the Maintenance and Construction Management Subsystem. The equipment package tracks the repair or replacement of the failed equipment. The entire range of ITS field equipment may be monitored by this equipment package including sensors (traffic, infrastructure, environmental, security, speed, etc.) and devices (highway advisory radio, dynamic message signs, automated roadway treatment systems, barrier and safeguard systems, cameras, traffic signals and override equipment, ramp meters, beacons, security surveillance equipment, etc.). |
| Transit Center Fare and Load Management | This equipment package manages fare collection and passenger load management at the transit center. It provides the back office functions that support transit fare collection, supporting payment reconciliation with links to financial institutions and enforcement agencies for fare violations. It collects data required to determine accurate ridership levels, establish fares, and distribute fare information. This equipment package supports two-way data communication between the transit vehicle and center to support collection of fare and passenger loading information. |
| Transit Center Fixed-Route Operations | This equipment package manages fixed route transit operations. It supports planning and scheduling of fixed and flexible route transit services. The package allows fixed-route and flexible-route transit services to develop and disseminate schedules and automatically updates customer service operator systems with the most current schedule information. This equipment package also supports automated dispatch of transit vehicles. Current vehicle schedule adherence and optimum scenarios for schedule adjustment are also provided. |
| Transit Center Information Services | This equipment package collects the latest available information for a transit service and makes it available to transit customers and to Information Service Providers for further distribution. Customers are provided information at transit stops and other public transportation areas before they embark and on-board the transit vehicle once they are enroute. Information provided can include the latest available information on transit routes, schedules, transfer options, fares, real-time schedule adherence, current incidents, weather conditions, yellow pages, and special events. In addition to general service information, tailored information (e.g., itineraries) are provided to individual transit users. |
| Transit Center Multi-Modal Coordination | This equipment package determines the need for transit priority on routes and at certain intersections and requests transit vehicle priority at these locations. The equipment package also supports schedule coordination between transit properties and coordinates with other surface and air transportation modes. As part of schedule coordination, this equipment package shares transit transfer cluster (a collection of stops, stations, or terminals where transfers can be made conveniently) and transfer point information between Multimodal Transportation Service Providers, Transit Agencies, and ISPs. An interface to Traffic Management also supports demand management strategies. |
| Transit Center Paratransit Operations | This equipment package manages demand responsive transit services, including paratransit services. It supports planning and scheduling of these services, allowing paratransit and other demand response transit services to plan efficient routes and better estimate arrival times. This equipment package also supports automated dispatch of paratransit vehicles. Customer service operator systems are updated with the most current schedule information. |
| Transit Center Security | This equipment package monitors transit vehicle operator or traveler activated alarms received from on-board a transit vehicle. It supports transit vehicle operator authentication and provides the capability to remotely disable a transit vehicle. This equipment package also includes the capability to alert operators and police to potential incidents identified by these security features. |
| Transit Center Vehicle Tracking | This equipment package monitors transit vehicle location. The location information is collected via a data communication link between the transit vehicles and the transit center. The location information is presented to the transit operator on a digitized map of the transit service area. The location data may be used to determine real time schedule adherence and update the transit system’s schedule in real-time. The real-time schedule information is provided to Information Service Providers and the Transit Center Information Services equipment package, which furnish the information to travelers. |
| Transit Data Collection | This equipment package collects and stores transit information that is collected in the course of transit operations performed by the Transit Management Subsystem. This data can be used directly by operations personnel or it can be made available to other data users and archives in the region. |
| Transit Environmental Monitoring | This equipment package assimilates current and forecast road conditions and surface weather information from a variety of sources, including both weather service providers and vehicle probes. The collected environmental information is monitored and forwarded to other agencies to more effectively manage transit operations. |
| Transit Evacuation Support | This equipment package manages transit resources to support evacuation and subsequent reentry of a population in the vicinity of a disaster or other emergency. It supports coordination of regional evacuation plans, identifying the transit role in a regional evacuation and identifying transit resources that would be used. During an evacuation, this equipment package coordinates the use of transit and school bus fleets, supporting evacuation of those with special needs and the general population. Transit service and fare schedules are adjusted and updated service and fare information is made available through traveler information systems. This equipment package coordinates the functions in other Transit equipment packages to support these requirements. |
| Transit Garage Maintenance | This equipment package provides advanced maintenance functions for the transit property. It collects operational and maintenance data from transit vehicles, manages vehicle service histories, and monitors operators and vehicles. It collects vehicle mileage data and uses it to automatically generate preventative maintenance schedules for each vehicle by utilizing vehicle tracking data from a prerequisite vehicle tracking equipment package. In addition, it provides information to proper service personnel to support maintenance activities and records and verifies that maintenance work was performed. |
| Transit Vehicle Operator Scheduling | This equipment package automates and supports the assignment of transit vehicles and operators to enhance the daily operation of a transit service. It provides the capability to assign operators to routes or service areas in a fair manner while minimizing labor and overtime services, considering operator preferences and qualifications, and automatically tracking and validating the number of work hours performed by each individual operator. This operator scheduling function is often performed at a Transit Garage facility. |
| Traveler Secure Area Sensor Monitoring | This equipment package includes sensors that monitor conditions of secure areas that are frequented by travelers (i.e., transit stops, transit stations, rest areas, park and ride lots, modal interchange facilities, etc). The equipment package monitors areas for environmental threats (e.g., chemical agent, toxic industrial chemical, biological, explosives, and radiological sensors), intrusion and motion, and object detection. |
| Traveler Secure Area Surveillance | This equipment package manages surveillance equipment that monitors secure areas in the transportation system that are frequented by travelers (i.e., transit stops, transit stations, rest areas, park and ride lots, modal interchange facilities, etc). This package collects the images and audio inputs at the secure area and provides the surveillance information to the Emergency Management Subsystem. The equipment package also provides local processing of the video or audio information, providing processed or analyzed results to the Emergency Management Subsystem. This equipment package provides the same functions as the Field Secure Area Surveillance equipment package. |
| Traveler Telephone Information | This equipment package services voice-based traveler requests for information that supports traveler telephone information systems like 511. The equipment package takes requests for traveler information, which could be voice-formatted traveler requests, dual-tone multifrequency (DTMF)-based requests, or a simple traveler information request, and returns the requested traveler information in the proper format. In addition to servicing requests for traveler information, this equipment package also collects and forwards alerts and advisories to traveler telephone information systems. |
| Vehicle Toll/Parking Interface | This equipment package includes the on-board systems that pay for tolls and parking electronically. It includes in-vehicle equipment that communicates with the toll/parking plaza (e.g., a tag) and an optional interface to a traveler card to allow use of a common payment medium for all transportation services. |

Appendix D

Maintenance Request Form

Las Cruces Regional ITS Architecture

Maintenance Change Request (MCR) Form

| To Be Completed By Stakeholder(s) Requesting Changes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Originator Name: |  |  |  | Date Submitted |  |
| Originator Telephone: |  |  | Originator Fax: | Originator E-Mail: |  |
| Originator Agency: |  |  |  | Functional Area: |  |
| Agency Authorized Signature: |  |  |  | Signature Date: |  |
| Description of Proposed Change: |  |  |  |  |  |
| Rationale for Proposed Change: |  |  |  |  |  |
| Affected Agency: |  | Authorized Signature: |  | Signature Date: |  |
| Affected Agency: |  | Authorized Signature: |  | Signature Date: |  |
| List Attachments: |  |  |  |  |  |
| Baseline Documents Affected: _____ Website ______Turbo Architecture ______Customized MPs ______Arch Document ______Other (describe) |  |  |  |  |  |
| To Be Completed By Maintenance Manager |  |  |  |  |  |
| Change Request Number: | Date CR Received: |  |  |  | Date CR Logged: |
| Date Initially Discussed: | Disposition: Accepted Rejected More Info |  |  |  | Disposition Comments |
| Date Discussed: | Disposition: Accepted Rejected More Info |  |  |  | Disposition Comments |
| Date Discussed: | Disposition: Accepted Rejected More Info |  |  |  | Disposition Comments |
| Date of Maintenance Working Group Approval (If Applicable): |  |  |  |  |  |
| Baseline Documents Affected/Version implemented Turbo Architecture Date: ______ Version: _____ Website Date: _______ Version: ____ Customized MPs Date: _______ Version: _____ Date: _______ Version: ____ Architecture Doc Date: _______ Version: _____ ____________ Date: _______ Version: ____ |  |  |  |  |  |